----------1000-------------------------------

INT 10 - VIDEO - SET VIDEO MODE

AH = 00h

AL = mode (see below)

Return: AL = video mode flag (Phoenix BIOS)

20h mode > 7

30h modes <= 7 except mode 6

3Fh mode 6

AL = CRT controller mode byte (Phoenix 386 BIOS v1.10)

Notes:       IBM standard modes do not clear the screen if the high bit of AL is set

(EGA or higher only)

SeeAlso: AX=0070h,AX=007Eh,AX=10F0h,AX=6F05h,AH=FFh"GO32",INT 5F/AH=00h

Values for video mode:

text/ text   pixel pixel    colors  disply  scrn system

grph  resol  box   resoltn          pages   addr

00h = T   40x25  8x14           16gray     8    B800 EGA

= T   40x25  8x16             16       8    B800 MCGA

= T   40x25  9x16             16       8    B800 VGA

01h = T   40x25  8x14             16       8    B800 EGA

= T   40x25  8x16             16       8    B800 MCGA

= T   40x25  9x16             16       8    B800 VGA

02h = T   80x25  8x14           16gray     4    B800 EGA

= T   80x25  8x16             16       4    B800 MCGA

= T   80x25  9x16             16       4    B800 VGA

03h = T   80x25  8x14             16       4    B800 EGA

= T   80x25  8x16             16       4    B800 MCGA

= T   80x25  9x16             16       4    B800 VGA

04h = G   40x25  8x8     320x200           4    B800 CGA,PCjr,EGA,MCGA,VGA

05h = G   40x25  8x8     320x200 4gray          B800 CGA,PCjr,EGA

= G   40x25  8x8     320x200   4            B800 MCGA,VGA

06h = G   80x25  8x8     640x200   2            B800 CGA,PCjr,EGA,MCGA,VGA

07h = T   80x25  9x14            mono     var   B000 MDA,Hercules,EGA

= T   80x25  9x16            mono           B000 VGA

0Bh =                 reserved (used internally by EGA BIOS)

0Ch =                 reserved (used internally by EGA BIOS)

0Dh = G   40x25  8x8     320x200  16       8    A000 EGA,VGA

0Eh = G   80x25  8x8     640x200  16       4    A000 EGA,VGA

0Fh = G   80x25  8x14    640x350 mono      2    A000 EGA,VGA

10h = G   80x25  8x14    640x350   4       2    A000 64k EGA

= G                  640x350  16            A000 256k EGA,VGA

11h = G   80x30  8x16    640x480 mono           A000 VGA,MCGA,ATI EGA,ATI VIP

12h = G   80x30  8x16    640x480 16/256k        A000 VGA,ATI VIP

= G   80x30  8x16    640x480 16/64          A000 ATI EGA Wonder

13h = G   40x25  8x8     320x200 256/256k       A000 VGA,MCGA,ATI VIP

----------1001-------------------------------

INT 10 - VIDEO - SET TEXT-MODE CURSOR SHAPE

AH = 01h

CH = bit 7    should be zero

bits 6,5 cursor blink

(00=normal, 01=invisible, 10=erratic, 11=slow)

(00=normal, other=invisible on EGA/VGA)

bits 4-0 top scan line containing cursor

CL = bottom scan line containing cursor (bits 0-4)

Notes: buggy on EGA systems--BIOS remaps cursor shape in 43 line modes, but

returns unmapped cursor shape

UltraVision scales size to the current font height by assuming 14-line

monochrome and 8-line color fonts; this call is not valid if cursor

emulation has been disabled

applications which wish to change the cursor by programming the

hardware directly on EGA or abovve should call INT 10/AX=1130h or

read 0040h:0085h first to determine the current font height

BUG:   AMI 386 BIOS and AST Premier 386 BIOS will lock up the system if AL

is not equal to the current video mode

SeeAlso: AH=03h,AX=CD05h

----------1002-------------------------------

INT 10 - VIDEO - SET CURSOR POSITION

AH = 02h

BH = page number

0-3 in modes 2&3

0-7 in modes 0&1

0 in graphics modes

DH = row (00h is top)

DL = column (00h is left)

SeeAlso: AH=03h,AH=05h

----------1003-------------------------------

INT 10 - VIDEO - GET CURSOR POSITION AND SIZE

AH = 03h

BH = page number

0-3 in modes 2&3

0-7 in modes 0&1

0 in graphics modes

Return: AX = 0000h (Phoenix BIOS)

CH = start scan line

CL = end scan line

DH = row (00h is top)

DL = column (00h is left)

Notes: 	a separate cursor is maintained for each of up to 8 display pages

many ROM BIOSes incorrectly return the default size for a color display

(start 06h, end 07h) when a monochrome display is attached

SeeAlso: AH=01h,AH=02h

----------1004-------------------------------

INT 10 - VIDEO - READ LIGHT PEN POSITION (EGA Only)

AH = 04h

Return: AH = light pen trigger flag

00h not down/triggered

01h down/triggered

DH,DL = row,column of character light pen is on

CH = pixel row (graphics modes 04h-06h)

CX = pixel row (graphics modes with >200 rows)

BX = pixel column

Notes: 	on a CGA, returned column numbers are always multiples of 2 (320-

column modes) or 4 (640-column modes)

returned row numbers are only accurate to two lines

----------1005-------------------------------

INT 10 - VIDEO -  SELECT ACTIVE DISPLAY PAGE

AH = 05h

AL = new page number (00h to number of pages - 1) (see AH=00h)

SeeAlso: AH=0Fh

----------1006-------------------------------

INT 10 - VIDEO - SCROLL UP WINDOW

AH = 06h

AL = number of lines by which to scroll up (00h = clear entire window)

BH = attribute used to write blank lines at bottom of window

CH,CL = row,column of window's upper left corner

DH,DL = row,column of window's lower right corner

Note:  affects only the currently active page (see AH=05h)

Warning: some implementations have a bug which destroys BP

SeeAlso: AH=07h,AH=72h,AH=73h

----------1007-------------------------------

INT 10 - VIDEO - SCROLL DOWN WINDOW

AH = 07h

AL = number of lines by which to scroll down (00h=clear entire window)

BH = attribute used to write blank lines at top of window

CH,CL = row,column of window's upper left corner

DH,DL = row,column of window's lower right corner

Note:  affects only the currently active page (see AH=05h)

Warning: some implementations have a bug which destroys BP

SeeAlso: AH=06h,AH=72h,AH=73h

----------1008-------------------------------

INT 10 - VIDEO - READ CHARACTER AND ATTRIBUTE AT CURSOR POSITION

AH = 08h

BH = page number (00h to number of pages - 1) (see AH=00h)

Return: AH = attribute

bit    7: blink

bits 6-4: background color

000 black

001 blue

010 green

011 cyan

100 red

101 magenta

110 brown

111 white

bits 3-0: foreground color

0000 black       1000 dark gray

0001 blue        1001 light blue

0010 green       1010 light green

0011 cyan        1011 light cyan

0100 red         1100 light red

0101 magenta     1101 light magenta

0110 brown       1110 yellow

0111 light gray  1111 white

AL = character

Notes:	 for monochrome displays, a foreground of 1 with background 0 is underlined

the blink bit may be reprogrammed to enable intense background colors

using AX=1003h or by programming the CRT controller

SeeAlso: AH=09h,AX=1003h

----------1009-------------------------------

INT 10 - VIDEO - WRITE CHARACTER AND ATTRIBUTE AT CURSOR POSITION

AH = 09h

AL = character to display

BH = page number (00h to number of pages - 1) (see AH=00h)

BL = attribute (text mode) or color (graphics mode)

if bit 7 set in graphics mode, character is xor'ed onto screen

CX = number of times to write character

Notes: all characters are displayed, including CR, LF, and BS

replication count in CX may produce an unpredictable result in graphics

modes if it is greater than the number of positions remaining in the

current row

SeeAlso: AH=08h,AH=0Ah,AH=4Bh,INT 17/AH=60h,INT 1F,INT 43,INT 44

----------100A-------------------------------

INT 10 - VIDEO - WRITE CHARACTER ONLY AT CURSOR POSITION

AH = 0Ah

AL = character to display

BH = page number (00h to number of pages - 1) (see AH=00h)

BL = attribute (PCjr only) or color (graphics mode)

if bit 7 set in graphics mode, character is xor'ed onto screen

CX = number of times to write character

Notes: all characters are displayed, including CR, LF, and BS

replication count in CX may produce an unpredictable result in graphics

modes if it is greater than the number of positions remaining in the

current row

SeeAlso: AH=08h,AH=09h,AH=4Bh,INT 17/AH=60h,INT 1F,INT 43,INT 44

----------100B--BH00-------------------------

INT 10 - VIDEO - SET BACKGROUND/BORDER COLOR

AH = 0Bh

BH = 00h

BL = background/border color (border only in text modes)

SeeAlso: AH=0Bh/BH=01h

----------100B--BH01-------------------------

INT 10 - VIDEO - SET PALETTE

AH = 0BH

BH = 01h

BL = palette ID

00h background, green, red, and brown/yellow

01h background, cyan, magenta, and white

SeeAlso: AH=0Bh/BH=00h

----------100C-------------------------------

INT 10 - VIDEO - WRITE GRAPHICS PIXEL

AH = 0Ch

BH = page number

AL = pixel color (if bit 7 set, value is xor'ed onto screen)

CX = column

DX = row

Notes: valid only in graphics modes

BH is ignored if the current video mode supports only one page

SeeAlso: AH=0Dh,AH=46h

----------100D-------------------------------

INT 10 - VIDEO - READ GRAPHICS PIXEL

AH = 0Dh

BH = page number

CX = column

DX = row

Return: AL = pixel color

Notes: valid only in graphics modes

BH is ignored if the current video mode supports only one page

SeeAlso: AH=0Ch,AH=47h

----------100E-------------------------------

INT 10 - VIDEO - TELETYPE OUTPUT

AH = 0Eh

AL = character to write

BH = page number

BL = foreground color (graphics modes only)

Notes: characters 07h (BEL), 08h (BS), 0Ah (LF), and 0Dh (CR) are interpreted

and do the expected things

IBM PC ROMs dated 4/24/81 and 10/19/81 require that BH be the same as

the current active page

SeeAlso: AH=02h,AH=0Ah

----------100F-------------------------------

INT 10 - VIDEO - GET CURRENT VIDEO MODE

AH = 0Fh

Return: AH = number of character columns

AL = display mode (see AH=00h)

BH = active page (see AH=05h)

Notes: if mode was set with bit 7 set ("no blanking"), the returned mode will

also have bit 7 set

EGA, VGA, and UltraVision return either AL=03h (color) or AL=07h

(monochrome) in all extended-row text modes

SeeAlso: AH=00h,AH=05h,AX=1130h,AX=CD04h

----------101000----------------------------

INT 10 - VIDEO - SET SINGLE PALETTE REGISTER (PCjr,EGA,MCGA,VGA)

AX = 1000h

BL = palette register number (00h-0Fh)

= attribute register number (undocumented)

10h attribute mode control register (should let BIOS control this)

11h overscan color register (see also AX=1001h)

12h color plane enable register (bits 3-0 enable corresponding

text attribute bit)

13h horizontal PEL panning register

14h color select register

BH = color or attribute register value

Notes: on MCGA, only BX = 0712h is supported

under UltraVision, the palette locking status (see AX=CD01h)

determines the outcome

SeeAlso: AX=1002h,AX=1007h,AX=CD01h

----------101001-----------------------------

INT 10 - VIDEO - SET BORDER (OVERSCAN) COLOR (PCjr,EGA,VGA)

AX = 1001h

BH = border color (00h-3Fh)

BUG: the original IBM VGA BIOS incorrectly updates the parameter save area

and places the border color at offset 11h of the palette table

rather than offset 10h

Note: under UltraVision, the palette locking status (see AX=CD01h)

determines the outcome

SeeAlso: AX=1002h,AX=1008h,AX=CD01h

----------101002-----------------------------

INT 10 - VIDEO - SET ALL PALETTE REGISTERS (PCjr,EGA,VGA)

AX = 1002h

ES:DX -> palette register list

Note: under UltraVision, the palette locking status (see AX=CD01h)

determines the outcome

SeeAlso: AX=1000h,AX=1001h,AX=1009h,AX=CD01h

Format of palette register list:

Offset  Size	      Description

00h   16 BYTEs  colors for palette registers 00h through 0Fh

10h      BYTE	   border color

----------101003-----------------------------

INT 10 - VIDEO - TOGGLE INTENSITY/BLINKING BIT (Jr, PS, TANDY 1000, EGA, VGA)

AX = 1003h

BL = new state

00h background intensity enabled

01h blink enabled

Note: although there is no function to get the current status, bit 5 of

0040h:0065h indicates the state

SeeAlso: AH=08h

----------101007-----------------------------

INT 10 - VIDEO - GET INDIVIDUAL PALETTE REGISTER (VGA,UltraVision v2+)

AX = 1007h

BL = palette or attribute (undoc) register number (see AX=1000h)

Return: BH = palette or attribute register value

Notes: UltraVision v2+ supports this function even on color EGA systems in

video modes 00h-03h, 10h, and 12h; direct programming of the palette

registers will cause incorrect results because the EGA registers are

write-only.  To guard against older versions or unsupported video

modes, programs which expect to use this function on EGA systems

should set BH to FFh on entry.

SeeAlso: AX=1000h,AX=1009h

----------101008-----------------------------

INT 10 - VIDEO - READ OVERSCAN (BORDER COLOR) REGISTER (VGA,UltraVision v2+)

AX = 1008h

Return: BH = border color (00h-3Fh)

Notes: UltraVision v2+ supports this function even on color EGA systems in

video modes 00h-03h, 10h, and 12h; direct programming of the palette

registers will cause incorrect results because the EGA registers are

write-only.  To guard against older versions or unsupported video

modes, programs which expect to use this function on EGA systems

should set BH to FFh on entry.

SeeAlso: AX=1001h

----------101009-----------------------------

INT 10 - VIDEO - READ ALL PALETTE REGISTERS AND OVERSCAN REGISTER (VGA)

AX = 1009h

ES:DX -> 17-byte buffer (see AX=1002h)

Notes: UltraVision v2+ supports this function even on color EGA systems in

video modes 00h-03h, 10h, and 12h; direct programming of the palette

registers will cause incorrect results because the EGA registers are

write-only.  To guard against older versions or unsupported video

modes, programs which expect to use this function on EGA systems

should set the ES:DX buffer to FFh before calling.

SeeAlso: AX=1002h,AX=1007h,AX=CD02h

----------101010-----------------------------

INT 10 - VIDEO - SET INDIVIDUAL DAC REGISTER (VGA/MCGA)

AX = 1010h

BX = register number

CH = new value for green (0-63)

CL = new value for blue (0-63)

DH = new value for red (0-63)

SeeAlso: AX=1012h,AX=1015h

----------101012-----------------------------

INT 10 - VIDEO - SET BLOCK OF DAC REGISTERS (VGA/MCGA)

AX = 1012h

BX = starting color register

CX = number of registers to set

ES:DX -> table of 3*CX bytes where each 3 byte group represents one

byte each of red, green and blue (0-63)

SeeAlso: AX=1010h,AX=1017h

----------101013-----------------------------

INT 10 - VIDEO - SELECT VIDEO DAC COLOR PAGE (VGA)

AX = 1013h

BL = subfunction

00h select paging mode

BH = 00h select 4 blocks of 64

BH = 01h select 16 blocks of 16

01h select page

BH = page number (00h to 03h) or (00h to 0Fh)

Note: not valid in mode 13h

SeeAlso: AX=101Ah

----------101015-----------------------------

INT 10 - VIDEO - READ INDIVIDUAL DAC REGISTER (VGA/MCGA)

AX = 1015h

BL = palette register number

Return: DH = red value

CH = green value

CL = blue value

SeeAlso: AX=1010h,AX=1017h

----------101017-----------------------------

INT 10 - VIDEO - READ BLOCK OF DAC REGISTERS (VGA/MCGA)

AX = 1017h

BX = starting palette register

CX = number of palette registers to read

ES:DX -> buffer (3 * CX bytes in size) (see also AX=1012h)

Return: buffer filled with CX red, green and blue triples

SeeAlso: AX=1012h,AX=1015h

----------101018-----------------------------

INT 10 - VIDEO - undocumented - SET PEL MASK (VGA/MCGA)

AX = 1018h

BL = new PEL value

SeeAlso: AX=1019h

----------101019-----------------------------

INT 10 - VIDEO - undocumented - READ PEL MASK (VGA/MCGA)

AX = 1019h

Return: BL = value read

SeeAlso: AX=1018h

----------10101A-----------------------------

INT 10 - VIDEO - GET VIDEO DAC COLOR-PAGE STATE (VGA)

AX = 101Ah

Return: BL = paging mode

00h four pages of 64

01h sixteen pages of 16

BH = current page

SeeAlso: AX=1013h

----------10101B-----------------------------

INT 10 - VIDEO - PERFORM GRAY-SCALE SUMMING (VGA/MCGA)

AX = 101Bh

BX = starting palette register

CX = number of registers to convert

SeeAlso: AH=12h/BL=33h

----------1011-------------------------------

INT 10 - VIDEO - TEXT-MODE CHARACTER GENERATOR FUNCTIONS (PS, EGA, VGA)

AH = 11h

The following functions will cause a mode set, completely resetting

the video environment, but without clearing the video buffer

AL = 00h, 10h: load user-specified patterns

ES:BP -> user table

CX	  = count of patterns to store

DX	  = character offset into map 2 block

BL	  = block to load in map 2

BH	  = number of bytes per character pattern

AL = 01h, 11h: load ROM monochrome patterns (8 by 14)

BL	  = block to load

AL = 02h, 12h: load ROM 8 by 8 double-dot patterns

BL	  = block to load

AL = 03h: set block specifier

BL	  = block specifier

(EGA/MCGA) bits 0,1 = block selected by chars with attribute bit 3 = 0

bits 2,3 = block selected by chars with attribute bit 3 = 1

(VGA) bits 0,1,4 = block selected by attribute bit 3 = 0

bits 2,3,5 = block selected by attribute bit 3 = 1

AL = 04h, 14h: load ROM 8x16 character set (VGA)

BL	= block to load

The routines called with AL=1xh are designed to be called only

immediately after a mode set and are similar to the routines called

with AL=0xh, except that:

Page 0 must be active.

Bytes/character is recalculated.

Max character rows is recalculated.

CRT buffer length is recalculated.

CRTC registers are reprogrammed as follows:

R09 = bytes/char-1 ; max scan line (mode 7 only)

R0A = bytes/char-2 ; cursor start

R0B = 0		; cursor end

R12 = ((rows+1)*(bytes/char))-1 ; vertical display end

R14 = bytes/char	; underline loc

(*** BUG: should be 1 less ***)

SeeAlso: AX=CD10h

----------1011-------------------------------

INT 10 - VIDEO - GRAPHICS-MODE CHARACTER GENERATOR FUNCTIONS (PS, EGA, VGA)

AH = 11h

AL = 20h: set user 8 by 8 graphics characters (INT 1F)

ES:BP -> user table

AL = 21h: set user graphics characters

ES:BP -> user table

CX	  = bytes per character

BL	  = row specifier

00h user set

DL = number of rows

01h 14 rows

02h 25 rows

03h 43 rows

AL = 22h: ROM 8 by 14 set

BL = row specifier (see above)

AL = 23h: ROM 8 by 8 double dot

BL = row specifier (see above)

AL = 24h: load 8x16 graphics characters (VGA/MCGA)

BL = row specifier (see above)

AL = 29h: load 8x16 graphics characters (Compaq Systempro)

BL = row specifier (see above)

Notes: these functions are meant to be called only after a mode set

UltraVision v2+ sets INT 43 to the appropriate font for AL=22h,23h,24h,

and 29h

SeeAlso: INT 1F, INT 43

----------101130-----------------------------

INT 10 - VIDEO - GET FONT INFORMATION (EGA, MCGA, VGA)

AX = 1130h

BH = pointer specifier

00h INT 1Fh pointer

01h INT 43h pointer

02h ROM 8x14 character font pointer

03h ROM 8x8 double dot font pointer

04h ROM 8x8 double dot font (high 128 characters)

05h ROM alpha alternate (9 by 14) pointer (EGA,VGA)

06h ROM 8x16 font (MCGA, VGA)

07h ROM alternate 9x16 font (VGA only)

11h (UltraVision v2+) 8x20 font (VGA) or 8x19 font (autosync EGA)

12h (UltraVision v2+) 8x10 font (VGA) or 8x11 font (autosync EGA)

Return: ES:BP = specified pointer

CX    = bytes/character

DL    = character rows on screen - 1

Note: for UltraVision v2+, the 9xN alternate fonts follow the corresponding

8xN font at ES:BP+256N

SeeAlso: AX=1100h,AX=1120h,INT 1F,INT 43

----------1012--BL10-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (PS, EGA, VGA, MCGA) - GET EGA INFO

AH = 12h

BL = 10h

Return: BH = 00h color mode in effect (I/O port 3Dxh)

01h mono mode in effect (I/O port 3Bxh)

BL = 00h  64k bytes memory installed

01h 128k bytes memory installed

02h 192k bytes memory installed

03h 256k bytes memory installed

CH = feature bits

CL = switch settings

----------1012--BL20-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (PS,EGA,VGA,MCGA) - ALTERNATE PRTSC

AH = 12h

BL = 20h  select alternate print screen routine

Notes: installs a PrtSc routine from the video card's BIOS to replace the

default PrtSc handler from the ROM BIOS, which usually does not

understand screen heights other than 25 lines

some adapters disable print-screen instead of enhancing it

SeeAlso: INT 05

----------1012--BL30-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (VGA) - SELECT VERTICAL RESOLUTION

AH = 12h

BL = 30h

AL = vertical resolution

00h 200 scan lines

01h 350 scan lines

02h 400 scan lines

Return: AL = 12h if function supported

----------1012--BL31-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (VGA, MCGA) - PALETTE LOADING

AH = 12h

BL = 31h

AL = 00h enable default palette loading

01h disable default palette loading

Return: AL = 12h if function supported

----------1012--BL32-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (VGA, MCGA) - VIDEO ADDRESSING

AH = 12h

BL = 32h

AL = 00h enable video addressing

01h disable video addressing

Return: AL = 12h if function supported

----------1012--BL33-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (VGA, MCGA) - GRAY-SCALE SUMMING

AH = 12h

BL = 33h

AL = 00h enable gray scale summing

01h disable gray scale summing

Return: AL = 12h if function supported

SeeAlso: AX=101Bh,AX=BF06h

----------1012--BL34-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (VGA) - CURSOR EMULATION

AH = 12h

BL = 34h

AL = 00h enable alphanumeric cursor emulation

01h disable alphanumeric cursor emulation

Return: AL = 12h if function supported

----------1012--BL35-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (PS) - DISPLAY-SWITCH INTERFACE

AH = 12h

BL = 35h

AL = 00h initial adapter video off

01h initial planar video on

02h switch active video off

03h switch inactive video on

80h *UNDOCUMENTED* set system board video active flag

ES:DX -> buffer (128 byte save area if AL = 0, 2 or 3)

Return: AL = 12h if function supported

----------1012--BL36-------------------------

INT 10 - VIDEO - ALTERNATE FUNCTION SELECT (PS, VGA) - VIDEO REFRESH CONTROL

AH = 12h

BL = 36h

AL = 00h enable refresh

01h disable refresh

Return: AL = 12h if function supported

----------1013-------------------------------

INT 10 - VIDEO - WRITE STRING (AT and later,EGA)

AH = 13h

AL = write mode

bit 0: update cursor after writing

1: string contains alternating characters and attributes

BH = page number

BL = attribute if string contains only characters

CX = number of characters in string

DH,DL = row,column at which to start writing

ES:BP -> string to write

Notes: recognizes CR, LF, BS, and bell

also available PC or XT with EGA or higher

HP 95LX only supports write mode 00h

BUG: on the IBM VGA Adapter, any scrolling which may occur is performed on

the active page rather than the requested page

SeeAlso: AH=09h,AH=0Ah

----------101A-------------------------------

INT 10 - VIDEO - DISPLAY COMBINATION (PS,VGA/MCGA)

AH = 1Ah

AL = 00h read display combination code

Return:          BL = active display code (see below)

BH = alternate display code

01h set display combination code

BL = active display code (see below)

BH = alternate display code

Return: AL = 1Ah if function was supported

Values for display combination code:

00h no display

01h monochrome adapter w/ monochrome display

02h CGA w/ color display

03h reserved

04h EGA w/ color display

05h EGA w/ monochrome display

06h PGA w/ color display

07h VGA w/ monochrome analog display

08h VGA w/ color analog display

09h reserved

0Ah MCGA w/ digital color display

0Bh MCGA w/ monochrome analog display

0Ch MCGA w/ color analog display

FFh unknown display type

----------101B-------------------------------

INT 10 - VIDEO - FUNCTIONALITY/STATE INFORMATION (PS,VGA/MCGA)

AH = 1Bh

BX = implementation type

0000h return funtionality/state information

ES:DI -> 64 byte buffer for state information (see below)

Return: AL = 1Bh if function supported

ES:DI buffer filled with state information

SeeAlso: AH=15h

Format of state information:

Offset Size   Description

00h   DWORD  address of static funtionality table (see below)

04h   BYTE   video mode in effect

05h   WORD   number of columns

07h   WORD   length of regen buffer in bytes

09h   WORD   starting address of regen buffer

0Bh   WORD   cursor position for page 0

0Dh   WORD   cursor position for page 1

0Fh   WORD   cursor position for page 2

11h   WORD   cursor position for page 3

13h   WORD   cursor position for page 4

15h   WORD   cursor position for page 5

17h   WORD   cursor position for page 6

19h   WORD   cursor position for page 7

1Bh   WORD   cursor type

1Dh   BYTE   active display page

1Eh   WORD   CRTC port address

20h   BYTE   current setting of register (3?8)

21h   BYTE   current setting of register (3?9)

22h   BYTE   number of rows

23h   WORD   bytes/character

25h   BYTE   display combination code of active display

26h   BYTE   DCC of alternate display

27h   WORD   number of colors supported in current mode

29h   BYTE   number of pages supported in current mode

2Ah   BYTE   number of scan lines active

(0,1,2,3) = (200,350,400,480)

2Bh   BYTE   primary character block

2Ch   BYTE   secondary character block

2Dh   BYTE   miscellaneous flags

bit 0 all modes on all displays on

1 gray summing on

2 monochrome display attached

3 default palette loading disabled

4 cursor emulation enabled

5 0 = intensity; 1 = blinking

6 PS/2 P70 plasma display (without 9-dot wide font) active

7 reserved

2Eh  3 BYTEs reserved (00h)

31h   BYTE   video memory available

00h = 64K, 01h = 128K, 02h = 192K, 03h = 256K

32h   BYTE   save pointer state flags

bit 0 512 character set active

1 dynamic save area present

2 alpha font override active

3 graphics font override active

4 palette override active

5 DCC override active

6 reserved

7 reserved

33h 13 BYTEs	reserved (00h)

Format of Static Functionality Table:

Offset Size    Description

00h   BYTE    modes supported #1

bit 0 to bit 7 = 1 modes 0,1,2,3,4,5,6 supported

01h   BYTE    modes supported #2

bit 0 to bit 7 = 1 modes 8,9,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh supported

02h   BYTE    modes supported #3

bit 0 to bit 3 = 1 modes 10h,11h,12h,13h supported

bit 4 to bit 7 reserved

03h  4 BYTEs  	reserved

07h   BYTE    scan lines supported

bit 0 to bit 2 = 1 if scan lines 200,350,400 supported

08h   BYTE	    total number of character blocks available in text modes

09h   BYTE	    maximum number of active character blocks in text modes

0Ah   BYTE    miscellaneous function flags #1

bit 0 all modes on all displays function supported

1 gray summing function supported

2 character font loading function supported

3 default palette loading enable/disable supported

4 cursor emulation function supported

5 EGA palette present

6 color palette present

7 color paging function supported

0Bh   BYTE    miscellaneous function flags #2

bit 0 light pen supported

1 save/restore state function 1Ch supported

2 intensity blinking function supported

3 Display Combination Code supported

4-7 reserved

0Ch   WORD    reserved

0Eh   BYTE    save pointer function flags

bit 0 512 character set supported

1 dynamic save area supported

2 alpha font override supported

3 graphics font override supported

4 palette override supported

5 DCC extension supported

6 reserved

7 reserved

0Fh   BYTE    reserved

----------101C-------------------------------

INT 10 - VIDEO - SAVE/RESTORE VIDEO STATE (PS50+,VGA)

AH = 1Ch

AL = 00h return state buffer size

Return: BX = number of 64-byte blocks needed

01h save video state

ES:BX -> buffer

02h restore video state

ES:BX -> buffer containing previously saved state

CX = requested states

bit 0 video hardware

1 BIOS data areas

2 color registers and DAC state

3-15 reserved

Return: AL = 1Ch if function supported