Unless otherwise stated, all segment registers are assumed to point to

their normal values (ie: use int86() instead of int86x()).

0x0100	Allocate DOS Memory Block

0x0100 -> ax

bytes/16 -> bx

carry clear:

ax -> segment

dx -> selector - DON'T USE - use dosmemX() instead.  Just

return this value as the selector for 0x0101.

carry set:

ax -> error

bx -> size of largest block

0x0101	Free DOS Memory Block

0x0101 -> ax

selector -> dx

0x0102	Resize DOS Memory Block

0x0102 -> ax

bytes/16 -> bx

selector -> dx

carry set:

(success)

carry clear:

ax -> error

bx -> max bytes/16

0x0200	Get Real Mode Interrupt Vector

0x0200 -> ax

vector -> bl

cx -> segment

dx -> offset

0x0201	Set Real Mode Interrupt Vector

0x0201 -> ax

vector -> bl

segment -> cx

offset -> dx

0x0204	Get Protected Mode Interrupt Vector

0x0204 -> ax

vector -> bl

cx -> selector

edx -> offset

Note: To chain to the previous vector use these opcodes:

.byte	0x2e

ljmp	__old_vector

iret

The extra iret is required because some vectors are tasks, and will

return to just after the ljmp.  The 0x2e causes the ljmp to use CS

instead of DS.

0x0205	Set Protected Mode Interrupt Vector

0x0205 -> ax

vector -> bl

selector -> cx

offset -> edx

Note: For hardware interrupts, the only register you can

assume is set properly is CS.  All other segment registers

must be preserved and reset if needed.  Do not use the

transfer buffer or issue any I/O or other system calls from

within an interrupt handler.  For software handlers, you may

assume all segment registers are correct and that you can do

any I/O or system call required.

Interrupts that happen while the processor is in real mode

will not call this handler.

Warning: longjmp'ing out of an interrupt handler may not work

correctly in all cases - especially FP exception handling.

0x0300	Simulate Real Mode Interrupt

0x0300 -> ax

vector -> bx

0 -> cx

_go32_dpmi_regs -> edi

edi -> _go32_dpmi_regs (filled in)

Note: Values other than zero in CX are not supported.

0x0301	Call Real Mode Procedure with Far Return Frame

0x0301 -> ax

0 -> bx

0 -> cx

_go32_dpmi_regs -> edi

edi -> _go32_dpmi_regs (filled in)

Note: Values other than zero in CX are not supported.

0x0302	Call Real Mode Procedure with IRET Frame

0x0302 -> ax

0 -> bx

0 -> cx

_go32_dpmi_regs -> edi

edi -> _go32_dpmi_regs (filled in)

Note: Values other than zero in CX are not supported.

0x0303	Allocate Real Mode Callback

0x0303 -> ax

cb_offset -> esi

code_selector -> ds

_go32_dpmi_regs -> edi

data_selector -> es

carry clear:

cx -> segment

dx -> offset

carry set:

ax -> error

0x0304	Free Real Mode Callback Address

0x0304 -> ax

segment -> cx

offset -> dx

carry clear:

success

carry set:

ax -> error

0x0500	Get Free Memory Information

0x0500 -> ax

int[12] -> di

di -> int[12]

Contents of buffer int[12]:

[0] = available memory (coreleft)

[1] = available pages

[2] = available lockable pages

[3] = linear space

[4] = unlocked pages

[5] = available physical pages

[6] = total physical pages

[7] = free linear space

[8] = max size of paging file

Note: only [0] is guaranteed to not be -1.  Try in this order:

Physical left - 5,0.  Virtual left - 0.