AT720 - BIOS PATCH FOR 720K DISKETTES IN 1.2M DRIVES

Chuck Guzis

Sydex

153 North Murphy Ave.

Sunnyvale, CA  94086

(408) 739-4866 (voice)

(408) 738-2860  (BBS)

We've had several users of our Con>Format and FORMATQM utilities call in

inquiring about the 720K diskette format on 1.2 Mbyte high-density 5.25"

drives.

Briefly, this format is identical to the 3.5" 720K format, placed on a

5.25" diskette.  This mode does not require high-density diskettes, nor

does it sacrifice reliability for a doubling of the available diskette

space.	Normally, when a 360K diskette is formatted in a high-density

drive, every other track of the diskette is formatted, with the alternate

tracks left blank.  This is because the high-density drive crams twice

as many tracks (80) into the same space as the 40 tracks of the double-

density 360K drive.  The 720K format merely uses every track available.

Unfortunately, some BIOS implementations for the AT clones make the

assumption that if a 5.25" diskette isn't high-density, then the diskette

must be 360K.  This results in "Sector not found" errors.  Most Phoenix

and IBM BIOS implementations do not have this problem, but some of the

less-common versions, such as the AMI BIOS do.

AT720 is a VERY small resident which fixes this problem in those un-

cooperative BIOS sets.	Just put a command of the form "AT720" in your

autoexec.bat file, and the problem will be taken care of.

We don't ask anything for this program--in order to make use of the 720K

format, you'll have to register one of our formatters--and we'll get your

money there.