An abridged spice3 manual, it might even be useful.  This manual

covers basic spice syntax and commands for using Spice TR3e2 in both

interactive and batch modes.  It may not be complete, but a more

complete version is being worked on.

SPICE (simulation program with integrated circuit emphasis) was

developed in the Electronics Research Laboratory at the University of

California at Berkeley.  Spice3 is based on spice2.  When used in batch

mode, most of the syntax and commands are the same, but there are

changes.  For example, the output can be similar, using the -o option,

or the may be written in a "RAW" file format (the raw file format must

be used if it is to be plotted with Spice TR3e2's graphics capability).

Polynomial non-linear sources are replaced by a General non-linear

source (NOTE: most opamp libraries are made for spice2g6 and use

polynomial non-linear sources-> exception:  most of Linear Technology

Corporation's models will work with either spice2 or spice3).

Polynomial nonlinear capacitors are not supported in Spice3.

*************** input format ****************************

A circuit to be analyzed by Spice is described by a netlist which

defines the circuit topology, element (or part) values, and may

include the type of analysis to be run.

Spice uses ASCII input files.  The input format for Spice is of the

free format type.  Fields on a line are separated by one or more

blanks, a comma, an equal (=) sign, or a left or right parenthesis;

extra spaces are ignored.  A line may be continued by entering a "+" in

column one of the following line.  Comments can be included in the

listing-just place a "*" in column one of the line (a space will also

work for Spice3).

The first line in a Spice input file is assumed to be the circuit

title.  The last line must be the ".END" line.  Between are the parts,

type of analysis, and any subcircuits or models.  The order of these

statements is arbitrary (except [of course], that continuation lines

must immediately  follow  the  line  being continued, and element lines

belonging to a subcircuit must be between the  ".SUBCKT" and ".ENDS"

lines for that subcircuit).  Other files may be added by using the

".include file_name" command.

Spice is not case-sensitive most of the time (the ".include" command

must be lower case).

In Spice2, nodes must be labeled as number, in Spice3 they may be may

be arbitrary character strings, except for the ground node which must

be named "0".  All nodes must have a dc path to ground. The circuit

cannot contain a loop of voltage sources and/or inductors and cannot

contain a series connection of current sources and/or capacitors.

Every node must have at least two connections, except for transmission

line nodes (to permit unterminated transmission lines) and MOSFET

substrate nodes (which have two internal connections anyway).

Numbers may be interred directly, followed by a integer exponent, or a

scale factor.  6000, 6e3, and 6k all have the same value. The scale

used are:  T=e12  G=e9   Meg=e6   K=e3    mil=25.4e-6

m=e-3  u=e-6  n=e-9    p=e-12  f=e-15

Letters immediately following a number that are not scale factors are

ignored, and letters immediately following a scale factor are ignored.

Hence, 10, 10V, 10VOLTS, and 10hz all represent the same number, and M,

MA, MSEC, and MMHOS all represent the same scale factor.

Each element in the circuit is specified by an element line that

contains the element name, the circuit nodes to which the element is

connected, and the values of the parameters that determine the

electrical characteristics of the element.  The first letter of the

element name specifies the element type.  The  format  for the SPICE

element types is given in what follows.  The strings  XXXXX, YYYYY, and

ZZZZZ denote arbitrary alphanumeric strings.  For example, a resistor

name must begin with the letter `R' and can  contain one or more

characters.  Hence, R, R1, RSE, ROUT, and R3AC2ZY are valid resistor

names.

******* Command line Usage ************

spice [ -n ] [ -b ] [ -o outfile ] [ -r rawfile ] [ input_file ]

-n (or -N)

Don't try to source the file "spice.rc" upon startup.

Normally Spice tries to find this file in the current

directory.

-b (or -B)

Run in batch mode. Spice will read the specified input

file and do the simulation.

-r rawfile (or -R rawfile)

Use rawfile as the default file into which the results of

the simulation are saved.  If the -r option is used, the

.print and .plot commands will be ignored.

-o outfile (or -O outfile)

Print analysis time and memory usage to a file, and if the

-r option is not used, use the .print and .plot commands to

make a Spice2 type output file.

Further arguments are taken to be Spice input files, which are read and

saved. (If batch mode is requested then they are run immediately.)

Spice files specified on the command line are read in before the

"spice.rc" file is read.  Thus if you define aliases there, any that

are used in a Spice source file mentioned on the command line won't be

recognized.

**************** Parts ******************************

Data fields that are enclosed in less than and greater than signs ("<"

or ">") are optional.

resistor

Format: Rname Node1 Node2 value

Examples: R1 32 45 1k

rload 8 0 200

The name must start with the letter R.  The value may be

positive or negative, but not zero.

capacitor

Format: Cname Node1 Node2 value

Examples: C1 32 45 1uf

cload 8 0 200pf

The name must start with the letter C.

inductor

Format: Lname +Node -Node value

Examples: L1 32 45 1uh

lload 8 0 200uh

The name must start with the letter L.

mutual inductor

Format: Kname Lname1 Lname2 value

Example: k1 L5 L9 .987

The name must start with the letter K.  Two inductors are

referenced.  The standard dot convention determines polarity-

with the positive node on the inductors having the dot.  Value

is the coefficient of coupling, which must be between 0 and 1.

Lossless Transmission Lines

Format: Tname N1 N2 N3 N4 Z0=value TD=value

Tname N1 N2 N3 N4 Z0=value F=freq NL=nrmlen

Example: t1 1 0 2 0 z0=75 td=15ns

The name must start with the letter T. N1 and N2 are the nodes

at port one; N3 and N4 are the nodes at port two.  Z0 is the

characteristic impedance.  The length of the line must be

expressed in either one of two forms : the delay may be specified

directly (TD=value) or a frequency (F=freq) must be given.  If

the frequency is given, the normalized electrical length

(NL=value) may be given (the default is NL=.25).

NOTE1: This models only one propagating mode.  If all four nodes

are distinct in the actual circuit, then two modes may be

excited.  To simulate this, two transmission line elements

are required.

NOTE2: The lossy transmission line with zero loss may be more

accurate due to implementation details.

sources

independent source

Format: _name +node -node [dc value] [ac value] [tran_value]

Examples: Vcc 10 0   15

Icc 2 5   1ma

vin 1 0 dc 0 ac 1 pulse(-0.5 0.5 20us 20us 20us 200us 400us)

If the name starts with a V it is a voltage source and if it

starts with an I it is a current source.  The transient value

may have the following values:

PULSE(v1 v2 pulse_delay [rise_time fall_time pulse_width period])

SIN(offset_voltage amplitude frequency [start_delay damping_coef])

PWL(time_point1 volt_or_amp1 [t2 v_or_a2 ...])

linear dependent sources

Format: _name +nodeOut -nodeOUT +nodeIN -nodeIN value

Examples: G1 10 0 5 0  15mmho

E1  2 0 5 0  1e8

If the name starts with a G, it is a voltage controlled

current source.  If the name starts with a E, it is a voltage

controlled voltage source.

Format: _name +nodeOut -nodeOUT vname value

Examples: F1 1 2 vz 26

H3 5 7 vy 345

Vname is the name of a voltage source through which the

controlling current flows.  If the name starts with a F, it is

a current controlled current source.  If the name starts with

a H, it is a current controlled voltage source.

non-linear dependent sources (these sources are not spice2 compatible)

Format: Bname +nodeOut -nodeOUT <I=expr> <V=expr>

Examples: b1 1 0 i=cos(v(1))+sin(v(2))

b2 5 3 v=log(v1))^2

The name must start with the letter B.  The values of the V

and I parameters determine the voltages and currents across

and through the device, respectively.  Only one of these

parameters must be given.  The expressions given for V and I

may be any function of voltages and currents through voltage

sources in the system.  The following functions of real

variables are defined:  abs     asinh     cosh    sin

acos    atan      exp     sinh

acosh   atanh     ln      sqrt

asin    cos       log     tan

The following operations are defined:

+   -   *   /   ^   unary

If the argument of log, ln or sqrt becomes less than zero, the

absolute value of the argument is used.  If a divisor becomes

zero, an error will result.  Other problems may arise where

the partial derivative of a function is undefined.

Parts that need MODELS:

Some devices (or parts) that spice simulates require many parameter

values.  Since these devices may be used more than once in a circuit

(with the same set of parameters), they have a set of "model"

parameters that is defined on a separate model line with the format:

.MODEL model_name type(parameters)

Examples:

.model mod1 npn (BF=50 IS=1e-13 VBF=50)

.model dm1 d (is=2e-17)

Among the model types used in Spice TR3e2bl are npn, pnp, d, njf, pjf,

ltra and sw.

NOTE: Although the model line is required for these parts, spice will

use the default values for the unspecified parameter values.

Please see some "real" spice documentation for the semiconductor

default values.

diodes (D)

Format: Dname NodeA NodeC modname

Example: d1 32 45 d1n4841

The name must start with the letter D.  NodeA is the anode and

NodeC is the cathode.

bipolar junction transistors (NPN, PNP)

Format: Qname collector base emitter modname

Examples: Q1 32 45  40 Q2N2222A

q27 1 2 3 q2n2222a

The name must start with the letter Q.

junction FETs (NJF, PJF)

Format: Jname drain gate source modname

Example: J1 32 45  40 JN2

The name must start with the letter J.

Lossy Transmission Lines (LTRA)

Format: Oname N1 N2 N3 N4 modname

Example: o1 1 0 2 0  tran1

The name must start with the letter O. N1 and N2 are the nodes

at port one; N3 and N4 are the nodes at port two.  Note: a

lossy transmission line with zero loss may be more accurate

than a lossless transmission line due to implementation details.

MODEL    parameters

R             resistance/length (default 0.0)

L             inductance/length (default 0.0)

G             conductance/length (default 0.0)

C             capacitance/length (default 0.0)

LEN           length of line required -> no default

REL           breakpoint control (default 1)

ABS           breakpoint control (default 1)

NOSTEPLIMIT   do not limit timestep to less than time delay (flag)

NOCONTROL     don't do complex timestep control (flag)

LININTERP     use linear interpolation (flag)

MIXEDINTERP   use linear when quadratic seems bad (flag)

COMPACTREL    special reltol for history compaction

COMPACTABS    special abstol for history compaction

TRUNCNR       use Newton-Raphson method for timestep control (flag)

TRUNCDONTCUT  don't limit timestep to keep impulse-response errors

low (flag)

Only the following types of lines have been implemented so far:

RLC -  uniform transmission line with series loss only

RC  -  uniform RC line

LC  -  lossless transmission line

RG  -  distributed series resistance and parallel conductance

Any other combination will yield erroneous results and should not be tried.

Switches (SW)

Format: Sname N+ N- NC+ NC- modname <on><off>

Example: s1 2 5 3 0 sw1

The name must start with the letter S (for a voltage

controlled switch).  Switches must always have a finite

positive value.  It is wise to set switch impedances only high

and low enough to be negligible with respect to other circuit

elements.  Switches will not change state in AC analyses.

MODEL    parameters

VT     threshold voltage (default 0.0v)

VH     hysteresis (default 0.0v)

RON    on resistance (default 1 ohm)

ROFF   off resistance (default 1/GMIN ohm)

******************** Subcircuits **********************

A subcircuit consists of spice elements and can be referenced in a

fashion similar to device models.  Spice will automatically insert the

group of elements wherever the subcircuit is referenced.

Format for subcircuit reference: xname node1 node2 <node...> subname

The name must start with the letter X.

Format for subcircuit: .subckt subname sub_node1 <sub_node...>

parts sub_nodes...

.ends

The subcircuit must end with the line: ".ends <subname>"

Ground (the 0 node) may be used in a subcircuit but can not

be listed as an external node.  Node references in a subcircuit

are independent of (not the same node as) any node which may have

the same name in either the main circuit or another

subcircuit.

**  example of a subcircuit **

.subckt pienet 2 3

c1 2 0 100p

l1 2 3 10u

c2 3 0 100p

.ends pienet

** end of example **

*************** Spice file commands *********************

These commands are placed in the input files for use in the batch mode,

or with the interactive "run" command.

dc operating point

Format: .op

This command directs spice3 to include the dc operating

information in the RAW output file.

DC analysis

Format: .dc source_name vstart vstop vincrement

AC analysis

Format: .ac dec nd fstart fstop

dec is for the decade variation, oct or lin can be used instead

for the octave or linear variation.  nd is the number of points

per decade, octave or absolute, depending on option chosen.

At least one independent source must have been specified with

an ac value for the analysis to be meaningful.

NOTE: AC analysis is a linear small-signal analysis.  No clipping

of signals will take place.

Transient analysis

Format: .tran tstep tstop <tstart <tmax>>

Transient analysis always start at time zero, if tstart is

non-zero, the data between zero and tstart is not saved.

***************** other file commands **********************

These commands are placed in the input files, all are used in the batch

mode, some will not do anything in the interactive mode.  Try it.  If

it works, it works.

.NODESET  V(node_number)=value V(node_number)=value ...

This line may help spice find the DC or initial transient solution

by making a preliminary pass with the specified nodes held to the

given voltages.  The restriction is then released and the iteration

continues to the true solution.  The .NODESET line may be necessary

for convergence on bistable or astable circuits, but it should not

otherwise be needed.

.IC V(node_number)=value V(node_number)=value ...

This line is for setting transient initial conditions.  It has two

different interpretations, depending on whether the UIC parameter

is specified on the .TRAN line.  The .IC line should not be confused

with the .NODESET line -> the .NODESET line does not affect final

bias solution (except for multi-stable circuits).

If the UIC parameter is not specified on the .TRAN control line,

the DC bias (initial transient) solution is computed before the

transient analysis.  In this case, the node voltages specified on

the .IC control line is forced to the desired initial values during

the bias solution.  During transient analysis, the constraint on

these node voltages is removed.  This is the preferred method since

it allows spice to compute a consistent DC solution.

If the UIC parameter is specified on the .TRAN control line, the

node voltages specified on the .IC control line are used to compute

the capacitor, diode, BJT JFET, and MOSFET initial conditions.  This

is equivalent to specifying the IC=... parameter on each device

line, but is more convenient.  The IC=... parameter can still be

specified and takes precedence over the .IC values.  Since no DC

bias (initial transient) solution is computed before the transient

analysis, one should take care to specify all dc source voltages on

the .IC control line if they are to be used to compute device

initial conditions.

.OPTIONS  option1=value option2=value ...

This line allows the user to reset program control and user options

for specific simulation purposes.  Any combination of the following

options may be included, in any order.  Below, 'x' represents some

positive number.

option        effect

GMIN=x        the minimum conductance allowed (default 1.0e-12)

RELTOL=x      the relative error tolerance (default .001 -> 0.1

percent)

ABSTOL=x      the absolute current error tolerance (default 1

picoamp)

VNTOL=x       the absolute voltage error tolerance

(default 1 microvolt)

CHGTOL=x      the charge tolerance (default 1.0e-14)

TRTOL=x       the transient error tolerance (default 7)  This

parameter is an estimate of the factor by which

spice overestimates the actual truncation error.

PIVTOL=x      the absolute minimum value for a matrix entry to be

accepted as a pivot (default 1.0e-13)

PIVREL=x      the relative ratio between the largest column entry

and an acceptable pivot value (default 1.0e-3)

TNOM=x        the nominal temperature at which device parameters

are measured (default 27 deg C)

TEMP=x        the operating temperature of the circuit (default

27 deg C)

ITL1=x        the DC iteration limit (default 100)

ITL2=x        the DC transfer curve iteration limit (default 50)

ITL3 and ITL5 are not implemented in spice3

ITL4=x        the transient analysis timepoint iteration limit

(default 10)

METHOD=name   sets the numerical integration method used by spice.

possible names "Gear" or "trapezoidal" (or

"trap") (default trap)

.save [ node1 ] [ node2 ... ]

Save a set of outputs, discarding the rest.  Useful only in batch

mode when a raw file format is used. If a node has been mentioned in

a save command, it will appear in the raw file.  If there are no

save commands given, all outputs are saved.  The save command must

be given before an analysis command to have an affect.

.print analysis_type output_var1 <output_var2 ... output_var8>

.plot analysis_type output_var1 <output_var2 ... output_var8>

These commands are ignored if a raw file is produced.

Examples:

.print ac vdb(1) db(out)

.print tran v(2) i(vin)

.plot tran out

.four fundamental_frequency [ value ... ]

This command is ignored if a raw file is produced.  Does a fourier

analysis of each of the given values, using the first 10 multiples

of the fundamental frequency.

Example:

.four 10k out

.include file_name

Examples:

.include d1n4148.mod

.include ltc\lm301a

***************** interactive commands ******************

This chapter describes the most important interactive commands

available for use with Spice TR3e2.  It must be remembered that Spice

3e2 was written for the Unix operating system.  Not all of the

"Berkeley" commands will work under DR(MS)-DOS.  Commands that did not

work, or were considered not useful, are not listed.  Those who may

disagree on my definition of "what is useful" are free to call Berkeley

for the "real" documentation(??).

Analysis Commands:

Note: While commands, in general, are not case sensitive in input

files, they ARE case sensitive in the interactive mode -> use

lower case.

op   Do an operating point analysis.

tran [ .tran card args ]

Do a transient analysis.

ac [ .ac card args ]

Do an ac analysis.

dc [ .dc card args ]

Do a dc transfer curve analysis.

Other Commands:

alias [ word ] [ text ... ]

Causes word to be aliased to text.  History substitutions

may be used, as in C-shell aliases.

cd [ directory ]

Change the current working directory to directory, or to

the user's home directory if none is given.

destroy [ plot_name ] [ all ]

Throws away the data in the named plot.  This can be

necessary if a lot of large simulations are being done.

Spice should warn the user if the size of the memory usage

is approaching the maximum allowable size (within about 90%).

It is advisable to run the rusage command occasionally if

running out of space is a possibility, since Spice will crash

if it does runs out of memory. If the argument to destroy is

all, all plots except the constant plot will be thrown away

(it is not possible to destroy the constant plot).  If no

argument is given the current plot is destroyed.

display [ vector_name ... ]

Prints a summary of currently defined vectors, or of the

names specified.  The vectors are sorted by name unless the

variable nosort is set.  The information given is the name of

the vector, the length, the type of the vector, and whether

it is real or complex data.  Additionally, one vector will be

labeled [scale]. When a command such as plot is given without

a vs argument, this scale is used for the X-axis. It is

always the first vector in a rawfile, or the first vector

defined in a new plot. If you undefine the scale (i.e, let

TIME = []), a random remaining vector will become the scale.

edit [ file ]

If a filename is given, then edit that file and (when done)

load it (making the circuit the current one)- else edit the

current SPICE3 deck.  The default editor is "VI".  The "set

editor=XXX" command (in either spice or DOS) can be used to

select a different editor.

fourier fundamental_frequency [ value ... ]

Does a fourier analysis of each of the given values, using

the first 10 multiples of the fundamental frequency (or the

first nfreqs, if that variable is set). The output is like

that of the .four  card. The values may be any valid

expression.  The values are interpolated onto a fixed-space

grid with the number of points given by the fourgridsize

variable, or 200 if it is not set.  The interpolation will be

of degree polydegree if that variable is set, or 1.  If

polydegree is 0, then no interpolation will be done.  This is

likely to give erroneous results if the time scale is not

monotonic, though.

hardcopy file plotargs

Just like plot, except creates a file containing the plot.

The default file format is postscript.  Other available

formants are not useful under DOS.

history [ number ]

Print out the history, or the last number commands typed at

the keyboard.  Historical substitution is possible, !!<CR>

will repeat the last command, !1<CR> will repeat the first

command given.

linearize [ vec ... ]

Create a new plot with all of the vectors in the current

plot, or only those mentioned if arguments are given.  The

new vectors will be interpolated onto a linear time scale,

which is determined by the values of tstep, tstart, and tstop

in the currently active transient analysis.  The currently

loaded deck must include a transient analysis (a tran command

may be run interactively before the last reset, alternately),

and the current plot must be from this transient analysis.

This command is needed because SPICE3 doesn't output the

results from a transient analysis in the same manner that

SPICE2 did.

listing [ logical ] [ physical ] [ deck ] [ expand ]

Print a listing of the current circuit. If the "logical"

argument is given, the listing is with all continuation lines

collapsed into one line, and if the "physical" argument is

given the lines are printed out as they were found in the

file. The default is logical. A deck listing is just like the

physical listing, except without the line numbers it

recreates the input file verbatim (except that it does not

preserve case).  If the word "expand" is present, the circuit

will be printed with all subcircuits expanded.

load [ filename ] ...

Loads the raw data from the files named. The default

filename is rawspice, or the argument to the -r flag

if there was one.

plot exprs [ samep ] [ ylimit ylo yhi ] [ xlimit xlo xhi ]

[ xindices xilo xihi ] [ xcompress comp ] [ xdelta xdel ]

[ ydelta ydel ] [ xlog ] [ ylog ] [ vs xname ] [ xlabel word ]

[ ylabel word ]

Plot the given exprs on the screen.  If samep is given, the

values of the other parameters (other than xname) from the

previous plot, or hardcopy command will be used unless

re-defined on the command line.  The xlimit and ylimit

arguments determine the high and low x- and y-limits of the

axes, respectively. The xindices arguments determine what

range of points are to be plotted - everything between the

xilo'th point and the xihi'th point is plotted.  The

xcompress argument specifies that only one out of every comp

points should be plotted. If an xdelta or a ydelta parameter

is present, it specifies the spacing between grid lines on

the X- and Y-axis.  These parameter names may be abbreviated

to xl, yl, xind, xcomp, xdel, and ydel respectively.  The

xname argument is an expression to use as the scale on the

x-axis. If xlog or ylog are present, the X or Y scale

respectively will be logarithmic.  The xlabel and ylabel

arguments cause the specified labels to be used for the X and

Y axes, respectively.

print [ col ] [ line ] expr ...

Prints the vector described by the expression expr. If the

col argument is present, print the vectors named side by

side. If line is given, the vectors are printed horizontally.

col is the default, unless all the vectors named have a

length of one, in which case line is the default.  The

options width, length, and nobreak are effective for this

command. If the expression is all, all of the vectors

available are printed.  The scale vector (time, frequency)

will always be in the first column unless the variable

noprintscale is true.

quit

Quit, return to DOS.

rusage [ resource ... ]

Print resource usage statistics.  If any resources are given,

just print the usage of that resource.  Currently valid

resources are:

elapsed

The amount of time elapsed since the last rusage

elapsed call.

space

Data space used.

time

CPU time used so far.

everything

all

All of the above, and more.

resume

Resume a simulation after a stop (a control C will stop a

run).

run [ rawfile ]

Run the simulation as specified in the input file. If a

rawfile is given, the data is written, it is also

available interactively.

save [ node1 ] [ node2 ... ]

Save a set of outputs, discarding the rest. If a node has

been mentioned in a save command, it will appear in the

working plot after a run has completed.  If there are no save

commands given, all outputs are saved.  The save command must

be given before an analysis command to have an affect.  To

save the branch current through voltage source, add

"source_name#branch" to the list of nodes.

set [ variable ] [ variable = value ]

Set the variable to value, if it is present.  If no value is

given then the value is the boolean 'true'.  You can set a

variable to be any value (numeric or string).  If a variable

is set to a list of values that are enclosed in parentheses

(which must be separated from their values by white space),

the value of the variable is that list.

Variables for Spice TR3e2:

abstol  The absolute tolerance used by the diff command.

appendwrite

Append to the file when a write command is issued,

if one already exists.

combplot

Plot vectors by drawing a vertical line from each

point to the X-axis, as opposed to joining the

points.  Note that this option is subsumed in the

plottype option, below.

editor  The editor to use for the edit command.

fourgridsize

How many points to use for interpolating into when

doing fourier analysis.

gridsize

If this variable is set to an integer, this number

will be used as the number of equally spaced

points to use for the Y-axis when plotting.

Otherwise the current scale will be used (which

may not have equally spaced points).  If the

current scale isn't strictly monotonic, then this

option will have no effect.

height  The length of the page for print col.

history

The number of events to save in the history list.

nfreqs  The number of frequencies to compute in the

fourier command. (Defaults to 10.)

noaskquit

Do not check to make sure that there are no

circuits suspended and no plots unsaved.  Normally

SPICE3 will warn the user when he tries to quit if

this is the case.

nobjthack

Assume that BJT's have 4 nodes.

nobreak

Don't have print col break between pages.

noclobber

Don't overwrite existing files when doing IO

redirection.

nogrid  Don't plot a grid when graphing curves (but

do label the axes).

nomoremode

If nomoremode is not set, whenever a large amount

of data is being printed to the screen (e.g, the

print or display commands), the output will be

stopped every screenful and will continue when a

carriage return is typed. If nomoremode is set

then data will scroll off the screen without

hesitation.

noprintscale

Don't print the scale in the leftmost column when

a print col command is given.

nosort  Don't have display sort the variable names.

numdgt

The number of digits to print when printing tables

of data (fourier, print col).  The default

precision is 6 digits.  Approximately 16 decimal

digits are available, so numdgt should not be more

than 16.  If the number is negative, one fewer

digit is printed to ensure constant widths in

tables.

polydegree

The degree of the polynomial that the plot command

should fit to the data. If polydegree is N, then

nutmeg will fit a degree N polynomial to every set

of N points and draw 10 intermediate points in

between each end point.  If the points aren't

monotonic, then it will try rotating the curve and

reducing the degree until a fit is achieved.

polysteps

The number of points to interpolate between every

pair of points available when doing curve fitting.

The default is 10. (This should really be done

automatically.)

prompt  The prompt, with the character `!' replaced by

the current event number.

rawfile

The default name for rawfiles created.

rawfileprec

The number of digits to use in the ascii rawfile

format.  The default is 15.

reltol  The relative tolerance used by the diff command.

sourcepath

A list of the directories to search when a source

command is given.  The default is the current

directory and the SPICE_PATH\scripts.

units   If this is degrees, then all the trig functions

will use degrees instead of radians.

unixcom

When this variable is set, Spice will try to

"shell" any non-spice command.

vntol   The absolute voltage tolerance used by the

diff command.

width   The width of the page for print col.

setcirc [ circuit name ]

Change the current circuit. The current circuit is the one

that is used for the simulation commands below.  When a

circuit is loaded with the "source" command (see below) it

becomes the current circuit.

setplot [ plotname ]

Set the current plot to the plot with the given name, or if

no name is given, prompt the user with a menu.  (Note that

the plots are named as they are loaded, with names like tran1

or op2. These names are shown by the setplot and display

commands and are used by diff, below.) If the "New plot" item

is selected, the current plot will become one with no vectors

defined.  Note that here the word "plot" refers to a group of

vectors that are the result of one SPICE run.  When more than

one file is loaded in, or more than one plot is present in

one file, they are keep separate and only the vectors in the

current plot can be plotted.

shell [ args ... ]

Fork a shell, or execute the arguments as a command to DOS.

If the "set unixcom" command is used (as it is in the default

spice.rc file), Spice will try to "shell" any non-spice

command that it can not "source".

show device

Show a device's parameters.

source file

Read the Spice input file file.  Interactive Spice commands

may be included in the file, but they must be enclosed

between the lines ".control" and ".endc"  (the file

"spice.rc" which is read on startup does not need these two

lines).  These commands are executed immediately after the

circuit is loaded.  Spice3 command scripts, other than

spice.rc, must begin with a blank or title line.  Also, any

line beginning with the characters "*#" will be executed as a

command.

The use of this command is some what unnecessary, since Spice

will try to "source" any non-spice command.

unalias [ word ... ]

Removes any aliases present for the words.

unset [ word ] ...

Unset the variables word.

version [ version_id ]

displays the version of Spice that is running.  If there are

arguments, it checks to make sure that the arguments match

the current version of Spice.  This is mainly used as a

command line in rawfiles.

where

Gives the name of the last node or device to cause

non-convergence (for transient and operating point analysis).

Note: only one node or device is printed- there may be more

than one problem.

write [ file ] [ exprs ... ]

Writes the expr's to file. First vectors are grouped together

by plots, and written out as such.  Additionally, if the

scale for a vector isn't present, it is automatically written

out as well.  The default filename is rawspice.raw, or the

argument to the -r flag on the command line, if there was

one, and the default expression list is all.