INTRODUCTION

AMQM - THE ALL MODE QSO MAKER

=============================

AMQM is not just  another terminal program. This is especially made for

making  QSOs with  DX stations  on  HF digital  modes  such as  PACTOR,

AMTOR, and RTTY.  It works with CW too.  Sorry it can not make  QSOs of

its own -  as far as I  know unattended HF  is not yet allowed  in many

countries. But  AMQM will  reduce your typing  almost by 90%.  You only

have to type the Call sign, Name,  RST rx/tx and QTH of the DX station.

These things you can do while  the QSO is progressing. AMQM will do the

rest. It will even keep a log  and keep a recorded file of all what you

did. This program is written for lazy hams  - like me (of course I  had

to write  the program, what  to do? But  it helps me  to reduce  lot of

typing  after  that -  hi!).  Any  way  it  also keeps  your  expensive

transceiver more than 50%  less time transmitting  for cool long  life.

AMQM program  can be  even use  for Packet Radio  or any  other digital

mode  as a  simple terminal  program, but  AMQM is  written for Digital

DXing and CW with TNC.  Look the new idea Packet Help with AMQM!!!

VERY IMPORTANT

==============

I HAVE NO GUARANTEE THAT AMQM WILL WORK WITH  YOUR COMPUTER AND/OR TNC.

YOU ARE USING THIS SOFTWARE ON YOUR OWN  RISK. I AM NOT RESPONSIBLE FOR

ANY  TYPE OF DAMAGE,  DATA LOSS TO  YOUR COMPUTER, TNC OR  TO THE RADIO

EQUIPMENT  BY USING THIS PROGRAM.  YOU CAN USE THE  PROGRAM ONLY IF YOU

AGREE WITH ABOVE CONDITIONS. AMQM CARRY NO WARRANTY OF ANY KIND.

LICENSE

=======

AMQM is  not  a "public  domain"  program.  AMQM is  a  shareware.  The

unregistered version  may be freely copied and distributed. The program

and/or manual  may not be modified in  any way. But you  are allowed to

add  or  change QSO  files, you  may  also rewrite  the manual  in your

language. But  should not change  the idea given  in the original  AMQM

English manual  or  should not  add any  new  items to  it.  It is  not

allowed to charge more than US$10 for distribution.

You  are hereby  granted a  limited license  to use  this software  for

evaluation  purposes for a period of 30 days. If you intend to continue

using this  software after the 30 day  evaluation period, you must make

a  registration payment  of US$30.  Please refer  to  the registeration

form at the end of this document.

Copyright (c) 1997 by Nadisha Ranmuthu.

All rights reserved.

FEATURES OF AMQM

================

*    Can be configure for virtually any TNC/DATA CONTROLLER.

*    Works with CW, RTTY, AMTOR, PACTOR and any other digital mode.

*    User  definable  text  to send.  AMQM  randomly  select  text  and

process  with information of the DX stations  call, name and other

info such  as report  etc. So, it  does not  send identical  overs

again  and again, and there is always  a variation as you actually

typing  text. Facility to insert  additional information (when you

start  the over)  to be sent,  such as  request to  repeat name or

report or your QSL info or repeating of your name.

*    Automatic log keeping. And as you  enter a DX callsign, AMQM  will

read  its own  log  book  to display  any  log entries  with  same

callsign.

*    Automatic recording  (capture) of the full AMQM session. A utility

program is supplied  to up date this file (It deletes all activity

of  3 days  or  older). This  utility works  only  with registered

users. Type QMACT to start the program.

*    AMQM call CQ automatically... (Once called by F8)

*    File Up-loading. Suppose  if you want  to up load  a file, a  msg,

ascii text  file, or 7+  file in  to a Amtor/Pactor  MBX (or  to a

Packet  Radio BBS),  you can  do that  with this  function. If you

wish you  can have additional text files for  you to send, in case

some one  asks for your  address - keep  one file handy  with your

address in it - why bother to type these things - hi hi!

*    AMQM does  not send  commands to  the TNC,  unless you have  given

commands in the AMQM.INI file  or in the QSO files. In a  way this

is  good,  you  can  control  your  TNC  as  you  like.  You   can

easily/anytime change parameters of your TNC, who cares!

REQUIREMENTS

============

IBM PC xt or compatible       (286 or higher recommended)

With one RS232 COM

port free to connect

your TNC.

MS/PC DOS 3.3 or higher

128K Free RAM

360K Floppy                   (HDD recommended)

Tnc/Radio, Connecting cables  (AEA-PK232 Ready)

KNOWN PROBLEMS

==============

*    The  unregistered version  works  only for  15 mints.  When  it is

about to  finish  (before 3  mints) it  will  give you  a  warning

signal (a beep  every 5 seconds).  Exit form AMQM and  restart the

program to work another 15 mints.

*    Registered version works only with registered call signs.

UNPACKING LIST

==============

AMQM.EXE       x    The program                                       1

AMQM.INI       x    Initializing & default information                2

AMQM.LOG       c    The log file (Empty - only headers)               3

AMQM.DOC            This document file you are reading                4

AQACT.EXE           Automatic Capture file maintenance                5

FILE_ID.DIZ         All about AMQM in few words                       6

MACRO.CMD      x    Function information file                         7

OUTPUT.TXT     c    Blank capture File                                8

ARQQSO.INF     o    Amtor mode QSO information file                   9

CWQSO.INF      o    C.W. mode QSO information file                   10

RTTYQSO.INF    o    R.T.T.Y. mode QSO information file               11

PKTQSO.INF     o    VHF Packet mode HELP information file            12

x =  You must have this file to run AMQM

o =  You must have  at least one of those files   to use

that  mode. You also can create any    new *QSO.INF

files for new modes.

c =  AMQM will create this file

INSTALL AMQM

============

How to set-up  and configure AMQM.  This is not that  difficult. Please

unzip the AMQM1.ZIP in the directory  where you want to keep AMQM  (may

be in C:\HAM\AMQM like  in my one). Don't expect mega full  of files to

fill your hard disk. You even can keep all  in a small 360K floppy disk

and run AMQM from the floppy (but It will be slow).

Now first  thing you  must do after  you decompress, configure  AMQM to

your PC  and TNC. This  too is  not very difficult,  using an editor  -

edit  the file name  AMQM.INI (may  be EDIT would  do if  you type EDIT

AMQM.INI). In this file you have to  enter the COM port number of  your

PC  where you  have connected  your TNC,  the baud-rate  (communication

speed),  your callsign, selcall, name, qth and  some default values. It

also takes TNC initializing  commands, if you wish to have  any. On the

INI  file  supplied  with  AMQM  is  configured  for  AEA-PK232. Please

consult your TNC manual for any other info.

AMQM.INI            Info of the line

Line_No

--------

COM1:1200,N,8,1     Port: baud_rate, parity,

data_bits, stop_bits                             -1

4S7NR               My call - your call sign                         -2

SSNR                My selcall - your four letter selcall.           -3

NADISHA             Handle - your name                               -4

COLOMBO             QTH - Your Location                              -5

SRI LANKA           Country                                          -6

20m                 Default band (can be changed)                    -7

RTTY                Default mode (can be changed)                    -8

Un-Registered       Registration ID. (If any)                        -9

599-22              Contest Report                                  -10

AES ON              }

CBE ON              } any number of TNC command lines.

MCO OFF             } As soon as you start AMQM, these lines will

RESTART             } be issued to the TNC.

Once AMQM is setup properly, you should be  able to type AMQM and start

the program.

But  before you do so, you will have  to change some of the info that I

have entered  in my  QSO files.  You may  find CWQSO.INF,  RTTYQSO.INF,

ARQQSO.INF (may be some new  QSO) files with your AMQM. Each file store

information of  what to send during a CW,  RTTY or AMTOR qso. Edit each

of  these files and change  the station setup info  that I have entered

to yours. The  other info also can be changed as you like. You may also

want  to change any TNC commands, if you are not using a AEA-PK232 TNC.

If  you want to  know more info  a bout these  QSO files, look  for the

topic *Info on QSO files* at the end of this manual.

DXING WITH AMQM

===============

When you enter in to AMQM, you will see a simple greetings screen

giving  some info on AMQM. At this point  press any key to go in to the

DXing  screen (Well AMQM  does not have  100 screens, this  is the only

screen you  will see all  the time. In  this screen (Sorry  you can not

change colours), you may  see two text areas. The bigger  upper area is

for what  you receive from  TNC, and the  lower small area is  for what

you  are typing. At the  very bottom, I have  used to display Functions

of Function  keys. On the top  line you may see  the copyright msg, and

on  the  middle  line  (where  the  two  upper  and  lower  screens are

separated)  you will see info like callsign,  name, qth, reports, band,

mode, greeting code, etc.  Now you may have seen hundreds  of this type

of terminal  programs, there  nothing new  about it.  Meaning you  have

nothing  much new to learn  about, and only few  things that you should

know is how to operate AMQM.

Once  you are in the AMQM, it will ask few questions. First it will ask

the greeting information. You  can enter anything but, better you enter

M for morning, A for afternoon or E for  evening. This will be used for

automatic greeting.  Then it will ask the  band of operation. Just type

20m or 15m  or what ever you like (if you type anything it will accept)

but write something meaningful, it will be used mainly for logging.

Next question  will be  the mode  you are  going to  operate, a  tricky

question... You  can work on  any mode you  like, but depending  on the

mode  you select, your  QSO.INF file will  be selected. If  there is no

QSO.INF  file  for the  mode  you select  (let  say if  you  type GTOR)

program will ask the  question again, till you enter a  name where AMQM

finds a QSO.INF file.

In case  you want to work  on a mode which is  not defined as a QSO.INF

file -  may be  GTOR. Just  type a mode  available (like  RTTY) to  get

access to AMQM  terminal screen  then manually change  the TNC to  GTOR

and work as on  a normal terminal. But If you request to  send any text

from F10 menu, AMQM will send RTTY text. Best thing is to have  a small

GTORQSO.INF  file created with proper information. If  you want to know

more  info about these QSO.INF  files, look for the  topic "Info on QSO

files" at the end.

F1 to F5

--------

All the function keys from 1 to  5 are assigned to data inputs you need

to use for QSOs.

F1 = Callsign of the DX station.   (Default = "DX")

F2 = Name of the DX station.

F3 = Qth of the DX station.

(As  this field is not  used in any of  the default QSO text,

you may  keep this field blank.  This filed is  only used for

in the logbook, but u can use this in qso file if you wish).

F4 = Report (Received).

F5 = Report Transmitted.           (Default 579)

All  data fields are limited character, auto  clearing type entries. On

these simple inputs no curser keys  working other than [Enter] key  and

[BACK SPACE] - Delete key will be working.

Although all inputs are  similar data entries, F1 is slightly different

from others.  When  you enter  F1  (Callsign of  the dx  station)  AMQM

checks  the log file for  previous QSOs made with  the same station, if

found, the AMQM will show them on the middle of the screen, in  a small

(green colour) window.

F6 & F7

-------

F6 [**]  is nothing,  you can  either programmed  something, or may  be

next  release   of  AMQM  will   have  something   depending  on   your

suggestions. F7  [OTHR] is exactly  the same 3  inputs (Greeting  Code,

Band and Mode) that  you entered at the beginning. As  these inputs are

not often used  and we have not  much function keys left, they  are all

assign to one F7 function. Sorry - ha!

Going On Air

------------

Now you have entered all the info needed and ready to work (lets

assume a RTTY qso). Connect your  TNC to the radio, and tune to a  RTTY

frequency  of the band you  selected. Let say you  heard my good friend

VK5RY (which  you have  never worked  him before)  calling CQ.  Quickly

press F1 and enter his callsign, press F10 and "0" enter.

Now you will see AMQM  answers. When he answer your call,  press F5 and

enter  the report  for him.  When  he finish,  just press  F10  and "1"

enter. Woooo.. thats AMQM. On his turn while he  is transmitting, press

F4 and enter the report  he gave you, enter  his name RON (as he  print

on  your screen) in  to F2, also  his QTH if you  like in to  F3. As he

finish his over you are  ready to answer and  you have nothing else  to

enter  now... if you  wish to give  your station info  and continue the

qso  for  tree overs,  press F10  and "2"  enter, otherwise  to finish,

press F10 and  "3" enter. (F10-"3" also LOGs your QSO). To give a extra

(small) final, press F10 and "E".

If this  is  bit  confusing,  and  if you  want  to  see  how  AMQM  is

responding, just switch off your radio while the TNC  is on, and assume

you are working  with some one  and work on AMQM.  You will see how  it

works rather than going on air and make errors at the first time.

WHAT YOU SEE ON F10 SCREEN

--------------------------

SEND COMMANDS.

""""""""""""""

(1) Intro           DX OVERS, First introduction (rst,Name,Qth),

(2) Setup           Second about working condition,

(3) Sign off        Third signing off.

(R)peat/Request     Short  over,  must  use  with  ADDitions  to   give

repeates or to request information from DX.

Final=(E)/add(Q)rz? Short Grand Final Over,  Add Q to send a QRZ? after

final.  On ARQ modes, (Q)rz will not work, but Link

down will be issued. Without any addition.

(0)Answer           Answer to a CQ  call. (on ARQ modes, Answer will be

sent on FEC mode)

(Q)rz?              Genaral QRZ?

CqCq=(G)            CQ, optianal directional can be entered in  F1. The

same text will be used for F8.

Connect(K)          Connect   Request.  Callsign  of   the  station  to

connect  will  be  requested  by  an  input  befour

connect.

(Works only on AMTOR and PACTOR).

DO NOT SEND RST

"""""""""""""""

(X)dn't snd RST     When Add  X on the first  DX over, RST will  not be

sent. You can add RST in the next over.

ADD TO SEND

"""""""""""    Adds  work only with  (2)(3) and (R).  But requests work

with DX over (1) too, in addition to above.

(N)ame         Add repetetion NAME

(R)st          Add repetetion of RST

(Q)th          Add repetetion of QTH

(I)nfo Qsl     Add QSL information

(C)all         Correction of your callsign. Also works on (1).

(M)Name?       Request DX to repeat/give his Name.

(T)Rst?        Request DX to repeat/give his RST.

(W)Qth?        Request DX to repeat/give his QTH.

(A)sk Qsl?     Request DX to give his QSL information.

F8  Calling CQ

--------------

But what  if no one  is calling CQ? Ok  for you to call  CQ, just press

F8...  now   wait  till  some   one  answers  your   call.  AMQM   will

automatically  call CQ one  after the other with  a 15 seconds interval

for some  one to answer, if some  one is answering (mean  if the TNC is

sending some  letters to the terminal) AMQM will wait... either for you

to respond to  him or for  him to fed-up  and go (so  AMQM can call  CQ

again).    To stop  calling auto  CQ  just press  enter. Please  not by

entering data in to F1-F5  inputs will not stop  auto CQ. But any  data

entry above F5 will also stop calling auto CQ.

Auto Loging

-----------

Now that  is not the end of dxing with AMQM,  when you send the F10-"3"

last text over, AMQM  also logs automatically your QSO  to the AMQM.LOG

file, and Green colour  line will appear with logged information on the

middle of the screen to  indicate the logging. The info of DX callsign,

Name, QTH, reports  you entered in  to F1-F5 will  become Defaults.  If

there  is no callsign is  entered auto log function  will not work with

F10-"3". If  you need, AMQM.LOG  can be  read from a  dos text  viewer,

like LIST.COM. When you  run AQACT (Capture file processing program, it

will show  you number of log entries made on  each day - only available

for registered users).

File Up Load

------------

What is  F9 FILE function? Well if you want to  send a file, usually 7+

or  ASCII.TXT you can  press F9 and  enter the name  of the file  to be

sent.  Don't worry  no matter the  length of  your file,  AMQM will not

overflow  the buffer of your  TNC which leads to  errors, but AMQM will

handle it  intelligently as  Fast/Slow as your  link.   When sending  a

file,  user should not type  anything on the keyboard  -- in which case

file up-loading will be terminated immediately.

Manual Typing

-------------

If you really enjoy typing text (then why use AMQM?) you can simply  do

that as  on  the AMQM,  just like  typing text  on  any other  terminal

program.

But On  AMQM all text lines will word truncate, (move  to a new line on

a space)  after  65 characters.  This is  because it  is  easy for  the

person who reads  your text at  the other end  without moving his  head

too much -  hi! The word truncate will  work even when AMQM  is sending

F10  text, but when  you send a  file using F9,  word truncate function

will be disabled.  So you have to  take care about that or  in the case

7+, it will know what to do.

Exit AMQM

---------

There are two functions that you can not see  any hint on the screen of

AMQM, that is EXIT and CLEAR SCREEN.

Quit AMQM      =     press  CTRL+Q      (^Q)

CLear SCreen   =     press  CTRL+S      (^S)

Function Key Summery of AMQM

----------------------------

F1      Enter DX call sign.

F2      Enter Name of the DX station.

F3      Enter QTH of the DX station.

F4      Enter RST rcved.

F5      Enter RST to send.

F6      -- not used.

F7      Enter Greeting, Band and Mode.

F8      Call CQ-CQ and continue automatically.

F9      Send File (Up load).

F10     Process and send QSO overs.

^S      Clear screen.   (Ctrl+S)

^Q      Quit from AMQM. (Ctrl+Q)

INFO LINE

---------

The  line between the  lower screen  (input are)  and the  upper screen

(output area) is called the info line. On this you  may see some useful

information for  you to work  on AMQM.  Although not indicated,  as you

enter some data into F1  (DX call) that will appear on the left side of

the info line.  If no call is entered,  it shows DX. Which is  used for

when calling  CQ DX. Second  place will be  the name of  DX, Third QTH,

Then  the reports (Rcved/Sent) within brackets. At the end it show what

band and  mode of operation. It  also shows greeting  code, G= (M/A/E).

Then shows the  QSO number. Indicating  number of  QSOs logged for  the

day.

Now what more  to type about AMQM, the  rest you have to learn  by your

self. I told  you I am  lazy type, to  type text -  that is why  I made

AMQM. As  you see, AMQM  is simple and  straight forward, there  are no

hidden menus and other nonsense. But it simply does  what it should do,

nice  and user  - friendly  manner. Isn't  it good?  Please write about

your views to me:

����������������������������������������Ŀ

�  Packet Radio : 4S7NR @HS0AC           �

�        E-mail : nadisha @ceylon.net    �

����������������������������������������Ĵ

�    Snail Mail : 58/3, Suhada Place,    �

�                 Thalapathpitiya Road,  �

�                 Nugegoda, SRI LANKA.   �

�    Tele Phone : +94-1-853 643          �

������������������������������������������

73 de Nadisha (4S7NR)

���������������������

����������������������������������Ŀ

� HOW TO EDIT AND CHANGE QSO FILES �

������������������������������������

INFO IN A QSO FILE

------------------

first few lines                         <---- (1)

---                                     <------ (2)

F10 menu display text                   }

will be on these                        }-------- (3)

three lines.                            }

*A*                                     <---------- (4)

details of the commands...              }

any number of lines.                    }------ (5)

%11 callsign is must                    }

somewhere in this text.                 }

+++                                     <---------------(6)

*B*                                     <---------- (4)

details of the commands...              }

any number of lines.                    }------ (5)

%11 callsign is must                    }

somewhere in this text.                 }

+++                                     <---------------(6)

*C*                                     <---------- (4)

any number of such                      }

commands as above                       }------ (5)

may continue one                        }

after the other....                     }

+++                                     <---------------(6)

(1)  First  few lines are the commands for the TNC, when this mode (qso

file) is selected, AMQM  will issue the  commands listed here,  to

set the TNC to proper mode.

(2)  --- is to indicate the end of TNC commands.

(3)  Three  lines of  input message,  when F10  is pressed  to send any

text, this 3 line menu/help will be displayed.

(4)  Command name, between two  star signs. *C* means, if u  enter C at

F10, the text bellow  *C* will be sent. (single character commands

only).

(5)  Any number of text lines with text control codes (see bellow  text

control codes).  You may  have to  type some  TNC commands  and/or

control  characters for the TNC to start  the transmitter ptt when

you send text,  and any finishing codes, depending on  the TNC you

are  using.  (TNC control  commands/characters are  different from

AMQM text control characters,  for TNC control codes refer to your

TNC manual - example PK232 to press PTT of txver is X <Enter>).

(6)  +++ is to indicate, end of text stream.

TEXT CONTROL CODES FOR QSO FILES

--------------------------------

When a text block is  started with *x* command. AMQM reads two lines at

a  time unless specified. It randomly decide what line to send, and add

the  line in to TNC buffer.  There are other commands  that you can put

in a text stream to control what you are sending.

-    Reads single line and add this to text.

+    Reads  three lines (using for RST) after  this line, select either

first  or second  randomly and  send, if  X was  issued  with text

selection input send the third line, instead of first or second.

&    Make a input. Line prior to  this line should have the input  text

followed by  length (two  digits), variable  number (two  digits).

Any line after & command is normal.

=    Print text (max 79 characters).

?    Conditional addition, ?N text....  if character N was  issued with

text selection  input, this  line will  be added  to text  stream.

(Above codes must be issued at a beginning of a line).

%00  Variables,  two digit  variable  to be  sent, such  as  name, qth,

report, greeting, ect. List  of variables are given below. (Can be

used anywhere in the text).

VARIABLES USED IN QSO FILES

---------------------------

%01     DX Call sign (F1)               %13     Your name

%02     DX Name (F2)                    %14     Your QTH

%03     DX Qth (F3)                     %15     Your Country

%04     RST Rcvd (F4)                   %16     Band/Frequency

%05     RST Sent (F5)                   %17     Mode of operation

%06     PK232 Pactor Over = ^Z          %18     PSWD - System

%07     Greeting Code (F7)              %19     Contest Report

%08     CW Greeting "GM"                %20     Line feed

%09     Long Greeting "Good Morning"    %21     TSTS - System

%10     5NN type RST sent               %22     UPFN - System

%11     Your Call sign                  %23     Free

%12     4 letter Selcall                %24     Free

You should not  try to change  any text in the  macro command file.  It

can be changed, but information on that can not be given (too

complicated)   as  some  of  the   parameters  involved  with  internal

programming in AMQM. If  you can understand the logic of  it you may be

able to  program F6 function key for your own  input.  But remember not

to program or change any other keys, this might lead to problems.

����������������������

� AMQM REGISTER FORM �

����������������������

TYPE YOUR NAME : ......................................

TYPE YOUR ADDRESS : ......................................

......................................

......................................

......................................

PACKET RADIO MAIL : ............................. (if any)

ELECTRONIC MAIL : ............................. (if any)

��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ     Any /suffix can

MY CALL SIGN : �  ��  ��  ��  ��  ��  �     be entered in the

������������������������     QSO file.

1   2   3   4   5   6   7   8   9   10  11  12

��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ��Ŀ

MY RADIO NAME : �  ��  ��  ��  ��  ��  ��  ��  ��  ��  ��  ��  �

(HANDLE)       ������������������������������������������������

US$ 30 INCLUDED : ( ) By check     ( ) By cash.

SIGNATURE : ................    DATE : ............

::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

When you send the registration  form, please make sure you have entered

your callsign,  name, handle and  address. If any  of these  fields are

not *** valid ***, the word NOCALL or NONAME  will be entered there. If

there is no address I can not reply you back with the registration.

Your AMQM  can be used only  with your registered call.   I don't think

you will like to go on air with NOCALL .... ?

When  you send money,  try to send  it by  a check (easy  and safe way)

address to  my name "Nadisha Ranmuthu".  If  you send cash, please send

it in a registered cover.

If you wish to pay  your registration in *NON US CURRENCY* please write

to  me and check  the amount.  It will  be slightly  more than  US$ 30,

including added  bank charges. If you write  on snail mail, please send

a addressed envelop with a  IRC. Sorry, I will not answer this question

on packet radio - please no queries to address HS0AC my HOME BBS.

Thank you for selecting this program for your digital DXing.

73 de Nadisha.

4S7NR