AUTOFAX version 0.2, (c) Copyright 1988, Rich Bono - NM1D

This version of AUTOFAX requires a 100% PC compatible with DOS 2.x

An EGA or CGA graphics card and monitor.  A RS-232  COM1: port.

Kantronix KAM.

This is user supported software.  If you would like to help further

the development of  AUTOFAX  and similar software  please register

with the author.  If you send $ 20.00  or more you will  be placed

on the update list.   You are free to give a copy of this  program

to others.  You may NOT,  charge for such copies,  or in any other

way make  money from it or any modified version  of it.   The only

exception to this is granted  to recognized user  groups,  who may

charge a small  (not more that $5.00)  fee for distribution costs.

Any  comments or suggestions sent  with your registration  will be

gladly accepted.

Send your registration to:

Richard Bono

7 Redfield Circle

Derry, New Hampshire  03038

Installation notes for the current version of AUTOFAX:

This version of AUTOFAX communicates with Kantronix KAM (and KPC-4)

TNCs on the COM1 RS-232 serial port at 9600 baud.  If you have another

terminal emulator, you may wish to set the tnc's ABAUD to 9600 and PERM it.

If you do not have another terminal emulator, AUTOFAX will act as a simple

terminal emulator to allow you to control the TNC.  If the TNC is not

currently set for 9600 baud then see your manual on how to use the auto-baud

rate detection feature of the TNC.

AUTOFAX tries to identify your video card, If you have an EGA card

(100% compatible) then AUTOFAX will use 640x350 mode.  If you don't have an

EGA card, then AUTOFAX will use CGA mode (640x200).  If you do have an EGA,

but AUTOFAX does not recognize it (maybe it is not 100% compatible), you can

start AUTOFAX with the /e switch to force EGA mode.  Also, if autofax seems to

find an EGA card, and you DO NOT have one installed (!), then start AUTOFAX

with the /c switch to force CGA mode.

Example:

AUTOFAX /c		This forces CGA mode

AUTOFAX /e		This forces EGA mode

When you start the program, after the initial screen, you will see

the following messages:

COM1: set for 9600 Baud . . .

Press F1 to toggle from CMD: mode to WEFAX mode

When in WEFAX mode:

Press '1' for 1 scan  per line

Press '2' for 2 scans per line

Press '3' for 3 scans per line

Press '4' for 4 scans per line

Press '5' for 5 scans per line

Press '6' for 6 scans per line

When in CMD: mode:

Press ESC to exit to DOS

These are the commands to AUTOFAX, they are simply one keystroke commands.

When the CMD: prompt appears, you may communicate with the TNC as you normally

do as AUTOFAX includes a SIMPLE terminal emulator.  To begin reception of

WEFAX pictures, connect the proper cable to your radio and tune in the WEFAX

signal. For best results try to find a signal that is not too weak.  When you

have found a WEFAX signal, press the F1 key.  You will start to see the

picture appear at the top of the screen.  Be patient, it can take a few

minutes to recieve a full screen picture.  After you recieve your first

picture, you will probable notice that it is not aligned correctly on your

screen.  This is because that WEFAX pictures send no sync pulses once the

picture has started.  To allow you to sync your pictures, you may use the

left and right arrow keys.  Watch for the start of a picture, most start with

a band of mostly one color with a small gap of the contrasting color (ie: all

white with a small gap of black), this is to allow you to sync your screen

with the sending FAX station.  Use the arrow keys to move the small gap to the

left edge of the picture (if it is closer to the right, use the right arrow

key, as the picture will 'wrap around' the screen).  Once the sync bar is

aligned properly, you may press the HOME key to erase the screen and start

the image at the top of the screen.  With a little practice you will be able

to do this procedure with your eyes (almost) closed.

If you have a graphics printer that is supported by your system, you may

be able to print the image on your screen to your printer.  To do this, before

running the AUTOFAX program, install your graphic screen dump software (called

GRAPHICS on some systems).  Then when you have a complete image on your screen

press your 'PRINT SCREEN' key.

Future plans for AUTOFAX include:

Higher resolution images.

Saving images to disk files.

Sending continuous images directly to the printer.

More complete terminal emulation with file capture and transmision.

Plus . . .

If you have an ICOM radio, send for information on AUTOLOG, a computer

controlled interface to control your rig and automate logging of contacts!

Note: KAM, KPC-4, and KPC-2 are trademarks of Kantronix Inc. Lawrence Kansas

gladly accepted.

#	 Send your registration to:

#	            Richard Bono

#	            7 Redfield Circle

#	            Derry, New Hampshire  03038

#EGA found

#EGA not found, using CGA

#Using