MEMSPD

v1 12/27/96

Copyright (c) 1995-96 by Ray Van Tassle.

This is NOT freeware.  This is postcard-ware.  Send me a nice

picture post card as the registration fee.

But if you want to, or if this has been VERY valuable to you, a

money contribution will be gratefully accepted.  You decide if

you should send money, and how much.  How could I be any more fair??

Ray Van Tassle

1020 Fox Run Lane

Algonquin, Il. 60102 USA

(847)658-4941

MEMSPD performs memory access timing tests, to check the access speed.

SAMPLE COMMAND LINES:

To log information to a file, for posting or printing---

>del mycpu.log

>memspd My 486, OPTI chipset >>mycpu.log

>memspd -w My 486, memory write >>mycpu.log

>memspd -f My 486, with cache disabled >>mycpu.log

This program is a complement to CACHECHK.  One comment commonly made

about cachechk is that it's measurements are dependent on CPU speed, and

that it does not drive the memory bus hard enough to saturate the RAM

bandwidth.

MEMCHK attempts to address that problem.  Cachechk makes it measurements

by moving data from memory to a register, and timing how long that

takes. So there are two components: the time it takes to actually

execute the instructions, and the time it takes to actually move the

data from memory.

Memchk does it's job by directly executing code (in 32-bit memory), and

timing how long that takes.  Depending on the switches you give, the body

of the timing loop is either:

a) NOP, or

b) JMP $+8, or

c) JMP $+16, or

d) JMP $+32.

The body of the loop is executed enough times to take about 500 msec per

data point.

Conceptually, the way the Pentium and 486 memory is organized is by

"cache lines".  On a 486, a line is 16 bytes.  On a Pentium, a line

is 32 bytes.  When the CPU attempts to read any byte from memory, that

entire line is read from the memory (or the L2 cache). This is called a

"line fill".

When the 1st NOP is executed, the entire line (16 or 32 bytes) gets

loaded from memory, and the NOP is executed.  When the 2nd NOP is

executed, it is already in the L1 cache line, so no extra RAM access is

necessary.  A NOP is executed in 1 clock cycle.

The JMP option is an attempt to partially eliminate the instruction

execution time from the measurements. It is a two-byte instruction, and

takes (on a 486) 3 clock cycles.  More advanced CPUs, like the

PentiumPro, the Cyrix 6X86, and AMD K5, have multiple instruction

pipelines, so the actual effective instruction clock time is hard to

determine.  A NOP is an alias mnemonic for "XCHG AX,AX", so with multiple

pipelines, register renaming, etc, it's effective clock time may also

hard to determine.

On a machine with a 32-byte line (Pentium-class CPU), 32 bytes will be

read from the memory, but only 2 bytes will be read by the CPU (the two bytes

of the instruction).

--- Instruction bytes used per cache line ---

Instr		486 (16 byte line)	P5 (32 byte line)

------		------------		-------------

NOP		16 of 16		32 of 32

JMP $+8		 4 of 16		 8 of 32

JMP $+16	 2 of 16		 4 of 32

JMP $+32	 2 of 16,& skip 1 line	 2 of 32

L1 CACHE CHARACTERISTICS

------------------------

The Intel Pentium has two L1 caches: 8KB data and 8KB code.

The Cyrix 6x86 has one L1 cache: 16KB unified (both code & data). It

also has a 256 byte fully-associative instruction cache.  MEMSPD does

not seem to show the effects of this 256 byte cache.

The AMD K5 has two L1 caches: 8KB data and 16KB code.  Internally, the

K5 has a RISC core;  the 8x86 instructions are converted to RISC

instructions.  I believe that it is these RISC instructions (rather than

the 8x86 instructions) that are placed in the code cache. Interestingly

enough, CACHECHK shows the 8KB data cache, and MEMSPD shows the 16KB

code cache.

Since the L2 is outside of the CPU, it caches any type of memory data,

and does not differentiate between code bytes and data bytes.

ACCURACY

--------

MEMSPD accesses different sizes of memory blocks, from 16 bytes to 8MB,

whereas CACHECHK goes from 1KB to 4MB.  At the smaller sizes, the loop

overhead is probably large enough so that the measurements loose

accuracy, although I have tried to factor this out.  Also, the numbers

are so small, and the byte-count per datapoint so large, that I run into

problems maintaining arithemetic accuracy.

BREAKPOINTS

-----------

A 10% jump in memory access speed determines a "cache breakpoint", unless

the CPU is 150MHZ or more, then it's a 5% jump.  With faster and faster

CPUs, this gets harder and harder to determine, because the access speeds

are so damned fast.

For example, my Cyrix 6x86 shows 1.9 nanoseconds per byte for L1, and

4.2 ns/byte for L2.  The bus is 4 bytes wide, so this is 7.6 ns/word and

16.8 ns/word, respectively.

In one nanosecond, light travels about one foot; electrons about 0.5 ft.

So, for each 32-bit word accessed, an electron has only been able to move

about 8 feet (~2.5 meters).  This is incredibly fast!!!!

WHAT DOES ALL THIS MEAN?

------------------------

You'll have to interpret these numbers yourself.  I tried to come up

with a tool that would take drive the memory subsystem as hard

as it could, and to take measurements.  Whether the numbers are

meaningful is something you'll have to decide for yourself.