/=============================================================================\

Palette utilities utilizing Binary Space Partitioning Tree algorithms

Written by:   Mark Betz

Released:     February 5, 1993

Disposition:  Released to the PUBLIC DOMAIN. These utilities and the

related source code may be used for any purpose, commercial or other-

wise. The programs and source code referenced are provided on an as-is

basis. The author expressely disclaims any liability or responsibility

for the consequences of the use of these programs in any product, or

for any purpose whatever.

\=============================================================================/

EXTRACT.EXE -

A palette extracter for 256 color PCX files. This extracter was written

to work with files created in PC VGA mode 19 (13h), but should work

correctly with any 256 color PCX image that stores the palette as a 768

byte array at the end of the file.

Useage:   EXTRACT [-ssource-file] [-ooutput-file]

Example:  extract -sceltic.pcx -oceltic.pal

The file written by the utility is a raw palette dump containing 768

bytes of RGB color information compatible with the VGA DAC palette.

These files may be used as input to several of the other utilities

included with this package.

SOURCE CODE INCLUDED: see RGBTREE.H, RGBTREE.CPP, EXTRACT.CPP,

CLPARAM.H, CLPARAM.CPP, EXTRACT.PRJ, EXTRACT.MAK

PMTEST.EXE -

A test program for the Binary Space Partioning Tree method of matching

RGB color triplets. This program compares the performance of the tree

method to that of the "standard" brute-force sequential search method.

It reports the number of cases in which the two methods chose different

colors, the maximum disparity between the choices in percent, the

elapsed execution time for both methods, and the average number of nodes

visited per tree search.

Useage:   PMTEST [-ffilename] [-sfilename] <-tvalue>

Example:  pmtest -fceltic.pal -snewtut.pal -t7

Where -f selects the file of colors to look for, -s selects the palette

which will be searched, and the optional -t specifies the size of the

search interval (0..32).

SOURCE CODE INCLUDED: see RGBTREE.H, RGBTREE.CPP, CLPARAM.H,

CLPARAM.CPP, PMTEST.CPP, PMTEST.PRJ, PMTEST.MAK

REMAP.EXE -

This utility remaps the colors in a 256 color PC VGA mode 19 (13h) .PCX

image file to use the colors in a second file, or those from a raw

palette dump created with EXTRACT.EXE or another utility that produces

raw palette dumps. The image to be remapped is displayed after

remapping, and may be saved by pressing the 'S' or 's' keys. The file

created is a standard .PCX image file using the new colors.

Useage:   REMAP [-ssource|-ppalette] [-rtarget] <-ooutput> <-tvalue>

Example:  remap -sceltic.pcx -rnavaho.pcx -oremap.pcx -t10

remap -pceltic.pal -rnewtut.pcx -oremap.pcx -t5

Where -s selects a .PCX file containing the source colors, or -p

alternatively selects a raw palette file containing the source colors,

-r selects the .PCX to be remapped, -o selects the optional output

filename, and -t selects an optional search interval size.

SOURCE CODE INCLUDED: see RGBTREE.H, RGBTREE.CPP, CLPARAM.H,

CLPARAM.CPP, FG.H, REMAP.CPP, REMAP.PRJ, REMAP.MAK

SHOWTREE.EXE -

This utility is a simple browser for the 3D BSP trees created by the

RGBBinTree class. It builds a tree from a raw palette file and displays

it on screen in text mode. The tree can be traversed using the keyboard.

Useage:   SHOWTREE [palette file]

Example:  showtree celtic.pal

SOURCE CODE INCLUDED: see RGBTREE.H, RGBTREE.CPP, VIEWTREE.H,

VIEWTREE.CPP, SHOWTREE.CPP, SHOWTREE.PRJ, SHOWTREE.MAK

CATFADE.EXE -

Diana Gruber's solution to the "Cat and Christmas Tree" graphics problem

described in the article. Requires the image file CATMAS.PCX to be

located in the same directory as the executable.

SOURCE CODE INCLUDED: see RGBTREE.H, RGBTREE.CPP, FG.H, CATFADE.CPP,

CATFADE.PRJ, CATFADE.MAK

Where applicable, directions for the use of a specific program can be

obtained by running the program without command-line parameters.