Notes on using the 3c507 network card with ICETCP

-------------------------------------------------

The card is SOFTWARE CONFIGURED.  This means that the port address, irq,

memory address, memory size and rom address (normally not used) are

set in a configuration program called 3c507.exe which comes with the

board.

The configuration program may not immediatly recognize the board

if there are io port conflicts in the machine.  In this case the command

should be given as

3C507 /B:###

where ### is the address in hex of the board's desired port

location.  It should then be possible to bring up the configuration program

the normal way by just typing 3c507 <enter>.

The card can be configured using the menu option "set

configuration".  Possible problems are, as always:

The irq line used cannot be shared with other devices

and

The shared memory address must not conflict with other memory mapped

devices or drivers, (such as qemm-386).  This is a little more difficult

than usual because the board has a large buffer.  If you want to use it to

it's fullest, you need a 64k block of open space in the C000-EFFF area.  If

there isn't a gap that large available, then a smaller buffer (as small as

16k is possible) should be used.

Also

The zero wait state and turbo options can also cause problems.

On some machines, these will options will not work with the packet driver.

On some machines, these will also not work with NDIS. With both options

turned off, everything will work.  We recommend starting with both options

off and running the diagnostics part of the 3c507.exe program to see if

everything is working.  Then, after all software (ICE.TCP) is running,

the zero wait state and turbo options can be tried.  The diagnostics part of

3c507.exe is a good test for these options.

Even though the board is totally software configured, the packet

driver for the 3c507 (3c507.COM, unfortunately the same name as the

configuration program) needs to know all of the parameters set in the

configuration of the board.  The NDIS driver does not need these parameters.

If the packet driver fails, check the parameters for the packet driver to

verify that you entered the same ones as when you configured the board. If it

still doesn't work, check to see that the turbo and zero wait state options

are turned off.  If it still doesn't load, check for irq, port and memory

address conflicts.

Finally, the 3COM configuration program for the board WILL NOT RUN if

there are any active drivers (just freezes). If you need to reconfigure,

remove the packet or ndis drivers from your autoexec.bat or

config.sys files FIRST.

(C) 1991, James River Group, Inc, Minneapolis MN USA