Fix Disappearing Cursor Bug

For years we've put up with this bug.  Now it's fixed.

The problem occurs in any of three contexts:

* when reviewing last screens (Alt-F10),

* when switching between color and mono adapters (Alt-F7), or

* when swapping screens (say, when exiting SWAT).

The problem occurs because not all programs maintain a consistent set

of data values in the BIOS data area on which we rely (e.g., the

dependence between the cursor type and the cursor emulation bit).

The fix is to read the cursor start and end line values upon entry and

restore those values in the above circumstances.  A new routine,

GET6845 is defined for the read starting value part.  I seem to recall

that the original definition of the 6845 registers was that they were

write-only, but apparently they are now readable as well.

At the same time, while testing the different contexts in which SWAT

changes the cursor type, I noticed that the Enter command handles the

INS key, but not the XINS key, so I changed it.

SWAT_ENT.ASM	 Make XINS work the same as INS.

SWAT_VID.ASM	 Replace code which restores the old cursor type when

restoring screens with call to SET6845.

Replace code which gets old cursor type from the BIOS

data area with code which reads that value from the

6845.

Define routine GET6845 which reads 6845 register

values.