EchoMon 1.02 Wed  05-20-1992

----------------------------

New features:

- If you have an Tseng Labs ET4000 based SVGA card,  you may wish to

use EM_4000.SYS in place of EchoMon.Sys.   This will force your VGA

card into 16 bit mode,  except when accessing the monochrome adaptor.

Most VGA cards (good ones) will auto-sense that you have a monochrome

adapter installed,  and kick into 8 bit mode.  This is necessary

because of the way the IBM ATs (and clones) were designed.  This

slows down video performance considerably.  ET_4000.SYS will toggle

ET4000 based vga cards back and forth as required.   Thanks to Gerry

Rozema for the idea (and the code example).

- Mouse/Key stuffing support.  I added this the day after I released

1.00 then promptly forgot about it (about 18 months ago).  Sorry.

Look in STUFF.LZH for instructions.

- M2.CMD is a sample batch file you may wish to use.  It launches

a background program on your second monitor.  For example,

if you would normally start a program thus:

PBW help.c

Simply stick 'M2' (for "2nd Monitor") in front,  thus:

M2 PBW help.c

New Comments:

------------

- There was some confusion with the "MODE MONO" command.  You do not

need to ever use this command with echomon.  In fact,  it is strongly

suggested that you do not tell OS/2's INSTALL that you have a second

monitor.  If you already have,  change the "VIDEO_DEVICES" (in

config.sys), as desbribed later,  to remove references to VIO_IBMMPA.

- Real Herculese cards,  and many clone Herc cards,  will not function

as plain IBM compatible MPA cards.  They may not work with echomon

correctly.  This is not a shotcomming of echomon,  but rather the way

some herc cards clash with VGA cards.

- Echomon does not run DOS programs on the monochrome monitor.

- Echomon 1.01 was an internal release.

EchoMon 1.00 beta, Wed  11-07-1990

----------------------------------

OS/2 does not support two monitors very well.  If you type "MODE

MONO", you may indeed switch over to the monochrome monitor,  but

your colour monitor remains dormant until you switch back to it.

Furthermore,  any program you start while using the monochrome

monitor will only display information when it is the foreground

process.  So what OS/2 supplies you with is the ability to use

either monitor,  but not both at the same time.

EchoMon solves this problem by precisely echoing the contents of one

full screen colour session to the monochrome monitor.  Therefore,

this one session is (when it is in the foreground) running on BOTH

monitors.  The advantage is that this session continues to display

on the monochrome monitor after it looses focus and another screen

group is made active,  including Presentation Manager.  The result

is simultaneous output to both monitors.

Because of OS/2's design,  EchoMon can not redirect an OS/2 windowed

session,  only full screen sessions.

Any full screen application will run under EchoMon.  You could, for

example,  run a BBS program (such as BinkleyTerm/Maximus) in the

echoed session,  so you could always keep an eye on it.  Or you may

choose to run large jobs in that session (such as a compile),  so

that you can visually monitor it while working on something else on

your colour screen.

EchoMon requires OS/2 1.20 (csd level WR4051 or greater).

EchoMon is shareware.  It is not "CrippleWare" or "DemoWare".  The copy

you have is 100% functional and will not stop working no matter how many

times you use it,  and it does not have an expiry date.  If, however,

you decide to continue using it,  you must pay for it.

INSTALLATION

------------

EchoMon is three files:

EchoMon.DLL  -  Must be placed in one of the directories listed

in the config.sys LIBPATH statement.  If you're

unsure, copy echomon.dll to C:\OS2\DLL.

EchoMon.SYS  -  Must be installed in config.sys,  thus:

DEVICE=C:\path\ECHOMON.SYS

Where "path" is the directory where echomon.sys

is located.

You must restart your computer before this part

of EchoMon is installed in memory.

EchoMon.Exe  -  This file can be located anywhere.  You may wish

to place it on your PATH,  so that it is always

accessible.

OPERATION

---------

After installation, simply run the EchoMon.EXE program.  You will

notice your monochrome screen jump to life,  with the standard OS/2

command prompt.

Any parameters given to EchoMon.exe will be passed directly to the

command interpreter defined by COMSPEC (probably CMD.EXE),  except

as noted below.  You can therefore,  have EchoMon automatically

start a program from a batch file or the Desktop Manager.  For

example,  to "launch" CHKDSK on the monochrome monitor,  you could

use this in a batch file:

start /N EchoMon /C CHKDSK

The /N and /C cause the session to close automatically when CHKDSK

is done (the monochrome screen will retain the output from chkdsk

after the session has closed).  If you replace /C with /K,  the

session will remain open after chkdsk completes.

To "launch" a program from the PM desktop,  add a new program and

fill in "Properties" dialog in this manner:

Program Title     :  <appropriate tittle>

Path and File Name:  C:\path\ECHOMON.EXE

Parameters        :  /C CHKDSK

Run Program       :  Full Screen

One possibly undesirable side effect of this is that the Program

Starter will bring this session to the foreground when you start it

-- so both monitors will be running the same program.  To avoid

this,  start EchoMon with a first parameter of "/B".  This will

cause EchoMon to start in the background.  The /B parameter is NOT

passed to the target program.  Modifying the second example (above)

to start in the background would result in:

Program Title     :  <appropriate tittle>

Path and File Name:  C:\path\ECHOMON.EXE

Parameters        :  /B /C CHKDSK

Run Program       :  Full Screen

NOTE: With OS/2 2.00,  it is possible to start a full screen session

"minimized" (ie: the session is started in the background),  so the

/B parameter need not be used.

CAVEATS

-------

- There can only be one copy of EchoMon running at any one time.

- The /B parameter does not actually start EchoMon in the

background.  What it really does is start EchoMon in the

foreground,  then switch to the Task Manager right after

initialization,  thus leaving EchoMon in the background.

- If any programs are STARTed or DETACHed from the EchoMon session,

they must be ended before EchoMon can be restarted.  For example,

the following sequence of events will cause EchoMon to report an

error:

EchoMon              (simply start an echoed command prompt)

DETACH QH            (detach a background process)

EXIT                 (end the echoed command prompt)

EchoMon              (error: "EchoMon can only run in one session")

Before EchoMon can be restarted, the QH process would have to be

terminated.  Had the QH program been loaded in an unrelated

session, or had been DETACHed before EchoMon was first loaded,

this problem would not have occurred.

- When installing OS/2,  you DO NOT have to tell the OS/2 install

program that you have a monochrome monitor installed.  If you

have already done so,  you may have something like this in your

config.sys:

SET VIDEO_DEVICES=VIO_IBMVGA,VIO_IBMMPA

SET VIO_IBMVGA=DEVICE(BVHVGA)

SET VIO_IBMMPA=DEVICE(BVHMPA)

You may wish to remove the support for your monochrome screen and

leave just:

SET VIDEO_DEVICES=VIO_IBMVGA

SET VIO_IBMVGA=DEVICE(BVHVGA)

- EchoMon MAY work with later versions of OS/2 1.10 (let me know).

- EchoMon does NOT work with early versions of both OS/2 1.10 and

1.20.  The symptom:  All output sent via "stdout" and VioWrtTTY()

will not be visible.

- EchoMon works with MS OS/2 1.21.

- EchoMon has been tested and works with OS/2 2.00 (GA).

DISCLAIMER

----------

The Program is supplied "AS IS" without any warranty of any kind,

either expressed or  implied,  including, but not limited to, the

implied warranties of merchantabilty and fitness for a particular

purpose.

In  no  event  shall A:WARE INC be liable for any damages, either

direct or  consequential,  caused by the use, or inability to use,

this program, even  if A:WARE INC has been advised of the

possibility of such damages.

LICENSE

-------

You are granted the right to use,  at no obligation,  EchoMon for a

period of 30 days.  After that period,  you must either stop using

EchoMon or register it with A:WARE Inc.

You are free distribute, with no obligation, the EchoMon package

provided that:

a) you distribute only verbatim copies of the EchoMon package

as you receive it; and

b) it is accompanied by unmodified copies of this documentation.

You MAY:

a) Use a different compression tool to re-package the unmodified

EchoMon files before distribution.

You may NOT:

a) distribute the contents of the EchoMon package in a modified

form.  This includes adding files and/or comments to the EchoMon

package/archive.  The EchoMon archive contains exactly 8 files:

ECHOMON.SYS

ECHOMON.EXE

ECHOMON.DLL

ECHOMON.DOC

ECHOMON.REG

STUFF.LZH

EM_4000.SYS

M2.CMD

If you distribute an EchoMon package with more or less than

the above listed files,  you are in violation of this

license agreement.   If you add an ARC/ZIP/ZOO/LZH/PAK

"comment" to the EchoMon package,  you are in violation of

this license agreement.

b) distribute EchoMon in connection with any other product, service

or "value added" package,  without prior written permission from

A:WARE Inc.

c) distribute EchoMon for general use within a company or

institution without first purchasing a site license.

d) distribute EchoMon for any consideration or 'disk fee'.

Electronic Bulletin Board operators are encouraged to post EchoMon

for downloading by their users,  provided that all of the above

conditions are met.

ACKNOWLEDGMENTS

--------------

The author would like to thank Gerry Rozema for testing EchoMon,

and for providing the "/B" idea.

REGISTRATION

------------

The registration fee is US$20 (CDN$24).  Please see the EchoMon.reg

file for more information.

EchoMon is Copyright (c) 1990 A:Ware Inc.

A:WARE Inc.

6056 Cayeswood Court, Ste 100.

Mississauga Ontario

Canada

L5V 1B1

Voice: (416)858-3222  (Peter Fitzsimmons.  Inquiries only).

The most recent version of EchoMon will always be posted for

downloading on RT Lab bbs, (416)-867-9663 and 9664.  FidoNet node

number 1:250/628 (Magic FileREQuest name is ECHOMON).