8003/8013 FAMILY LAN ADAPTER SETUP PROGRAM

USER DOCUMENTATION

EZSETUP is a configuration program that operates with these

8003/8013 family Ethernet adapters:

o 8003EB  - 8-bit coaxial adapter configurable by software

only

o 8003W   - 8-bit twisted pair adapter configurable by

software only

o 8003EP  - 8-bit coaxial adapter configurable by software

or jumpers

o 8003WP  - 8-bit twisted pair adapter configurable by

software or jumpers

o 8013EP  - 16-bit coaxial adapter configurable by software

or jumpers

o 8013W   - 16-bit twisted pair adapter configurable by

software or jumpers

o 8013EW  - 16-bit coaxial and twisted pair adapter

configurable by software or jumpers

o 8013EPC - 16-bit coaxial adapter configurable by software

or jumpers

o 8013WC  - 16-bit twisted pair adapter configurable by

software or jumpers

o 8013EWC - 16-bit coaxial and twisted pair adapter

configurable by software or jumpers

These adapters contain non-volatile memory which eliminates most

of the jumper requirements of earlier adapters.  This allows

you to configure the adapter through software.

To run EZSETUP from a floppy disk, change your default drive to

the floppy drive, and type the following at the prompt:

EZSETUP

Alternatively, you can copy the EZSETUP program file to the system

hard disk and run EZSETUP from there.

You can run the EZSETUP program using either the interactive

method (where the program prompts you) or a command line.

Procedures for both methods are explained below.

1. INTERACTIVE METHOD

Type EZSETUP and follow the instructions displayed on the screen.

When you have more than one adapter which is supported by this

program installed in your system, then you see all the adapters

listed by their I/O Base addresses.

If you do have more than one adapter, but you see only one I/O

Base address listed, this means that your adapters are probably

configured to the same I/O Base address.  When this happens,

you must remove all the adapters except one from your computer,

then change each adapter's I/O Base address.  After you have

changed each adapter's address, you can add the other adapters and

configure them.

Refer to the adapter's user installation guide for further

installation and configuration instructions.

2. COMMAND LINE METHOD

The command line consists of three components: 1) the command,

EZSETUP, 2) the adapter identification, and 3) the configuration

parameters.

You only need to specify the adapter identification when you

have more than one adapter installed.  You can use either

the I/O base address or the node address.  To identify the

adapter, the command for using the node address is /node:x,

where x is the adapter's 12-digit node address.

All command line options available are listed below:

Command       Description

==============================================================

/?            Displays all EZSETUP command line options.

/factory      Forces the adapter to the default factory-set

values for the SOFT jumper position, ignoring

the specific command options for these values

(I/O base address 240, IRQ 2 Disabled, RAM base

address CE000; For 8013 adapters, CC000).

/default      Forces the adapter to the following settings

when the jumper is set to SOFT: I/O base address

240, IRQ 2, RAM base address CE000 (CC000 for

8013 adapters).

/io:x         Identifies the current I/O base address of the

adapter you want to configure.  The factory

setting for the SOFT jumper location is 240.

/irq:x        Specifies the interrupt level (IRQ).  Possible

values for x are 2, 3, 4, 5, and 7.  For 8013

adapters, possible values are 2, 3, 4, 5, 7, 10,

11, and 15.  Also enables the IRQ specified.

/newio:x      Specifies a new I/O base address.  Acceptable

values for x are 200, 220, 240, 260, 280, 2A0,

2C0, 2E0, 300, 320, 340, 360, 380, 3A0, 3C0,

and 3E0(hex).

/node:x       Identifies the node address of the adapter you

want to configure.  x is the 12-digit node address

printed on the NODE ADDRESS label attached to the

adapter.  This command is an alternative to using

the /io command to identify the adapter.  Example:

0000C024D058.

/ram:x        Specifies the RAM base address.  Possible values

for x are any hexadecimal address that is a multiple

of 8K between 80000 - FE000.  For 8013 adapters, the

value has to be a multiple of 16K between 80000 -

FFE000. To specify address above 1 Megabytes (above

FC000), 6 digits of address has to be specified. See

your adapter's user installation guide.

/rom:x        Specifies the base address of the boot ROM.

Possible values for x are any hexadecimal address

between 80000 to FC000.

Note: the address must be on the a boundary of

the ROM size.  See your adapter's user installation

guide.

/romsize:x    Specifies the size of the ROM window in Kbytes.

Possible values for x are 0 (ROM disabled), 16,

32, and 64.

/wait:x       Specifies whether or not to add wait states.

Possible values for x are y (yes) and n (no).

Most computers work with /wait set to NO.

Computers with fast I/O bus clock rates may

need /wait set to YES.  If the adapter fails

its diagnostic with /wait:n, set /wait to YES.

/netconn:x    Specifies which network connection to use.

Possible values for x are : 1 (for BNC or 10BaseT),

2 (for AUI or 10BaseT), or 3 (for Twisted Pair-

No Link Integrity). This option is valid for

8003WC, 8013WC, 8013EPC, and 8013EWC adapters.

/link_int:x   Specified whether the link integrity is enabled

disabled. Possible values for x are d (disabled)

or e (enabled). This options is valid only for

8013WP and 8013EW adapters.

The following is an example of a properly constructed command

line:

ezsetup /io:240 /newio:280 /irq:5 /ram:C0000 /rom:D0000

The command, ezsetup, starts the program. The adapter identi-

fication, /io:240, tells the system which adapter to

configure, and the remaining information tells the system

how to configure the adapter.  In this example, the I/O base

address is changed to 280, the IRQ level is changed to 5,

the RAM base address is changed to C0000, and the ROM base

address is changed to D0000.

Make sure that the configuration you select for your network

software driver matches what you select in EZSETUP.

Licensed by Western Digital for use by its customers only.

Western Digital is a registered trademark of Western Digital

Corporation. Other trademarks mentioned herein belong to

their respective companies.

Copyright (c) 1990, 1991 Western Digital Corporation

All Rights Reserved