RTS 2.0

A program to toggle the RTS line in doors

by

Bob Eyer

[73230,2620]

Nov 4, 1990

Syntax:

RTS [port]

where  port  may  be either 1 or 2.  If port is not entered on the

commandline, or if any number other than 1 or 2  is  entered,  RTS

defaults port to 1.

This  program  toggles  the  logic status of the RTS line, without

altering the DTR  line,  and  deals  directly  with  standard  IBM

hardware modem control registers for ports 1 or 2.   It  does  not

open  or  close the communications port.  RTS 2.0 may executed any

number  of times without sending invalid data to the modem control

register, and will not stay resident in memory.  Consequently, use

of this program  does  not  reduce  free  space  available  for  a

specific door program.

If  RTS is low, this program will set it high; if high it will set

the level to low.  RTS will not provide any video output, as it is

designed to be invoked  from  a  batch  file  which  controls  the

processing of a specific door program - so that the remote user is

not  aware  of  its operation.  Therefore, the Sysop must use some

other indicator program to determine the actual status of the  RTS

line, such as RS232.COM - or the RS light on the host modem.

General Discussion:

------------------

Have  you  found a door program which, when accessed through shell

or exit doors of RBBS, simply hangs remote video?  This may be the

program which solves your problem.

Most  door  programs  don't  have  RTS/CTS  problems when executed

through the shell or exit routines of RBBS.  This is  due  to  the

fact that most door programs, particularly those which are written

in  BASIC, will reopen the communications port in such a way as to

raise the RTS line.

However,  some  host programs may, under certain conditions, lower

the RTS line when dropping to DOS for  a  door  application,  even

where DTR is unaffected.

Setting the RTS line at logic low means that programs which reopen

the communications port without correcting this problem  will  not

send any information.  Typically, such programs will be written in

a  language, such as Pascal or C, which don't make any assumptions

about how the port should be opened; and some authors forget  that

not  all  host  programs allow doors to pick the RTS logic off the

host's settings.

RTS  2.0  should solve this problem.  However, don't use it unless

you're sure you have an RTS logic problem: If  your  host  program

keeps RTS high after drop to DOS, using RTS 2.0 will force it low.

DISCLAIMER:

==========

This program is circulated as public domain without any  guarantee

or  warranty;  and  the  user, by downloading this program, or any

variant thereof or by receiving it or any of its versions  in  any

other  form, agrees to accept full responsibility for its use.  It

is therefore understood that the user accepts this program or  any

previous  version  as  is.

------------------------------------------------------------------

End of documentation.