MCX11

MicroController eXecutive

for the

Motorola MC68HC11

Version 1.3

by

A.T. Barrett & Associates

11501 Chimney Rock

Houston, Texas 77035

(713)728-9688

April 1990

Support Contact:

Mike Wood

M/S OE319

Motorola, Inc.

6501 William Cannon Drive West

Austin, Texas 78735

(512)891-2717

Neither Motorola,  Inc. nor  A.T. Barrett & Associates makes

any warranty,  expressed or  implied, with  regard  to  this

material including  but not  limited to  merchantability  or

fitness for  a given  purpose.    The  information  in  this

document is  subject to  change without  notice and  neither

Motorola, Inc.  or A.T.  Barrett &  Associates  assumes  any

responsibility for  any errors which may appear herein.  Nor

shall Motorola,  Inc. or  A.T. Barrett & Associates bear any

liability  for   use   of   this   material   with   respect

compensatory,   special,   incidental,   consequential,   or

exemplary damages.

MCX11 - MicroController eXecutive for the 68HC11

TABLE OF CONTENTS

Section 1  Introduction                               Page

1.1  Background and Features........................... 1-1

1.2  MCX11 as a Software Component..................... 1-1

1.3  Target Environment................................ 1-2

1.4  Development Environment........................... 1-2

1.5  Manual Format..................................... 1-2

Section 2  MCX11 Functional Overview

2.1  What Is MCX11..................................... 2-1

2.2  Tasks............................................. 2-2

2.3  Multitasking...................................... 2-2

2.4  Priority and Scheduling........................... 2-2

2.5  Intertask Communication and Synchronization....... 2-3

2.6  Time based Functions.............................. 2-4

2.7  Interrupt Service................................. 2-4

Section 3  MCX11 Control & Data Structures

3.1 Tasks.............................................. 3-1

3.1.1 Task Identifier & Priority....................... 3-1

3.1.2 Task State Table................................. 3-2

3.1.3 Stack............................................ 3-3

3.2  Semaphores........................................ 3-3

3.3  Messages and Mailboxes............................ 3-5

3.4  Queues............................................ 3-7

3.5 Timers............................................. 3-8

Section 4  Executive Service Requests (ESRs)

4.1  Purpose........................................... 4-1

4.2  Semaphore ESRs.................................... 4-1

4.3  Message ESRs...................................... 4-2

4.4  Queue ESRs........................................ 4-2

4.5  Task Control ESRs................................. 4-2

4.6  Time Based ESRs................................... 4-2

Section 5  Alphabetical Listing of ESRs

5.1  .delay.  -  Delay a Task for a Period of Time..... 5-2

5.2  .dequeue.  -  Get an Entry from a FIFO Queue...... 5-4

5.3  .enqueue.  -  Insert Data into FIFO Queue......... 5-6

5.4  .execute.  -  Execute a Task...................... 5-9

5.5  .pend.  -  Force a Semaphore to a PENDING State.. 5-11

5.6  .purge.  -  Purge a Task's Timers................ 5-12

5.7  .receive.  -  Receive a Message.................. 5-14

5.8  .resume.  -  Resume a Task....................... 5-16

5.9  .send.  -  Send a Message to a Task.............. 5-17

5.10  .sendw.  -  Send a Message and Wait............. 5-19

5.11  .signal.  -  Signal a Semaphore................. 5-20

Page i

MCX11 - MicroController eXecutive for the 68HC11

TABLE OF CONTENTS

5.12  .suspend.  -  Suspend a Task.................... 5-22

5.13  .terminate.  -  Terminate a Task................ 5-23

5.14  .timer.  -  Start a Timer....................... 5-24

5.15  .wait.  -  Wait on Semaphore.................... 5-26

Section 6  System Configuration

6.1  System Configuration Concepts..................... 6-1

6.2  MCX11 System Table Memory Requirements............ 6-1

Section 7  Device Driver & Interrupts

7.1  Driver Concepts................................... 7-1

7.2  Interrupt Service................................. 7-1

7.2.1 Interrupt Processing Variables................... 7-2

7.2.2 Interrupt Processing Code........................ 7-4

Section 8  Advanced Topics

8.1  Other topics...................................... 8-1

Appendix A  MCX11 Distributed Source Files............. A-1

Page ii

MCX11 - MicroController eXecutive for the 68HC11

SECTION 1

INTRODUCTION

1.1  BACKGROUND AND FEATURES

MCX11, the MicroController eXecutive for the MC68HC11, is an

efficient  software   framework   for   embedded   real-time

applications using  the Motorola  MC68HC11  microcontroller.

Motorola, Inc.  makes MCX11 available free of charge via the

Freeware Bulletin  Board System.   MCX11  is written  in the

AS11 assembly  language  which  is  also  available  on  the

Motorola Freeware BBS (512/891-3733).

MCX11 provides  many features  which are designed to support

real-time applications.  These features include:

-     Multitasking for up to 126 tasks

-     Pre-emptive task scheduling by priority

-     Intertask communication and synchronization via

semaphores, messages, and queues.

-     Support for timed operations

-     Fast context switch

-     Very small RAM and ROM requirements

-     Fifteen Executive Service Request functions

1.2  MCX11 AS A SOFTWARE COMPONENT

MCX11 is  furnished in  AS11 source form and is downloadable

via  a   modem  connection  to  the  Freeware  BBS.  In  its

distribution  form,  it  is  not  executable.    It  may  be

assembled separately  or with  the application  programs and

any device  drivers peculiar  to the application.  MCX11 may

be treated as any other software library.  The user need not

know how  MCX11 performs  its functions internally.  Rather,

the user  need only  know what  functions of MCX11 to use to

achieve a  desired result.   Thus, MCX11 becomes much like a

large scale  integrated circuit  component in  the hardware.

Knowledge of what inputs produce what outputs is all that is

needed to  use  the  "chip"  successfully.  Unlike  a  chip,

however, MCX11  users are  provided with  the source code to

the kernel  so that  they can exercise complete control over

their applications.  With the  source code,  users may  even

wish to extend the functionality of MCX11 by the addition of

new services.

This manual  is not meant as a tutorial on real-time kernels

in general.  It is  intended to  explain  the  "inputs"  and

"outputs" of  MCX11 as  a software  component of  the user's

application.  In  an  effort  to  assist  the  user  in  the

Copyright (c) 1989         Page 1 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

successful employment  of MCX11,  this manual will cover the

following subjects:

-     basic concepts of MCX11 design

-     interfaces to the MCX11 services

-     system generation procedure

-     assembly procedure for MCX11 and application

source files

-     procedure for merging MCX11 with the application

object files

1.3  TARGET ENVIRONMENT

MCX11 is  designed to operate in an embedded processor, that

is, one whose functions are fixed within the application. No

assumptions are  made about  the configuration of the target

system.   It is the responsibility of the user to define the

target environment  and to insure that all necessary devices

have program support. No distinction is made between Single-

Chip Operating  mode or  Expanded Multiplexed Operating mode

in the design of MCX11. If there are special requirements in

the application for functions related to a certain operating

mode, it will be the responsibility of the user to add them.

1.4  DEVELOPMENT ENVIRONMENT

MCX11 development  requires a  computer compatible  with the

IBM PC and having at least 256K bytes of RAM, 1 floppy disk,

a keyboard, and a video monitor, an RS-232 compatible serial

I/O port, a Centronics compatible parallel printer port, and

a printer (preferably one which is Epson compatible). A hard

disk  is   very  desirable   for  development   work.   This

configuration is  needed to run the AS11 cross assembler and

serve as a debug terminal to the target assuming MC68HC11EVM

Evaluation Module is being used. Consult the EVM User Manual

for specifics  on connection tp PC. Recommended software for

the  development  environment  besides  MS-DOS  includes  an

editor,  and   the  Kermit  communications  program  (public

domain).

1.5  MANUAL FORMAT

Section 1 of this manual is simply an overview of MCX11.

Section 2 gives the functional overview of MCX11.

Section 3  details  the  organization  and  content  of  the

various MCX11 control and data structures.

Copyright (c) 1989         Page 1 - 2          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

Section 4  gives a  description  of  the  Executive  Service

Requests (ESRs)  provided by  MCX11, subdivided  into  their

classes.

Section 5  gives an  alphabetical presentation  of all MCX11

Executive Service Requests.

Section 6  deals with  the configuration  and generation  of

MCX11 systems.

Section 7  discusses device  drivers and  interuupt  service

routines.

Section 8  presents technical information on advanced topics

related to the use of MCX11.

Appendices  round   out  the  specifics  of  necessary  user

knowledge.

Copyright (c) 1989         Page 1 - 3          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

SECTION 2

MCX11 FUNCTIONAL OVERVIEW

2.1  WHAT IS MCX11

A real-time  executive provides  a software framework within

which different  processes can  operate and  gain access  to

various system resources.  Real-time systems usually consist

of several  processes, or  tasks, which need to have control

of  the  system  resources  at  varying  times  due  to  the

occurrence of  external events.  These tasks  are at various

times  competing   for  system  resources  such  as  memory,

execution time, or peripheral devices. They range from being

compute bound  to I/O  bound. Tasks which are I/O or compute

bound   cannot be allowed to monopolize a system resource if

a more important function requires the same resource.  There

must be  a way  of interrupting the operation of the task of

lesser importance  and granting  the needed  resource to the

more important task.

A multitasking   real-time  executive  promotes  an  orderly

transfer of  control from  one task  to  another  such  that

efficient usage  of the  computer's resources  is  achieved.

Orderly transfers  require that  the executive keep track of

the needed resources and the execution state of each task so

that they  can be  granted to  each task in a timely manner.

The key  word there is timely. A real-time system which does

not perform  a required  operation at  the correct  time has

failed. That  failure can have consequences which range from

the benign  to the catastrophic. Response time to a need for

executive services  and the  execution time of such services

must be  sufficiently fast  enough  so  that  no  need  goes

undetected.

One way  to  achieve  timeliness  is  the  assignment  of  a

priority to  each task.  The priority of a task is then used

to determine  its place  within the sequence of execution of

all tasks.  Tasks of  low priority  may have their execution

pre-empted by  a task  of higher priority so that the latter

can perform some time critical function.

An event  can be any stimulus which requires a reaction from

the executive  or a task. Examples of an event would include

a timer  interrupt, an alarm condition, or a keyboard input.

Events  may   originate  externally   to  the  processor  or

internally from  within the  software.  An  executive  which

responds to  these events  as  the  stimuli  for  allocating

resources is  said to  be event  driven. If response time to

any event   occurs  within a  period of  time which  can  be

Copyright (c) 1989         Page 2 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

accurately defined and guaranteed, the executive can be said

to be deterministic.

By  these  definitions,  MCX11  is  a  deterministic,  event

driven, multitasking, real-time executive.

2.2  TASKS

In MCX11, a task is a program module which exists to perform

a  defined  function  or  set  of  functions.    A  task  is

independent of  other tasks  but may establish relationships

with other tasks.  These relationships may exist in the form

of data  structures, input/output,  or other  constructs.  A

task executes when the MCX11 task dispatcher determines that

the resources  required by  the task are available.  Once it

begins running,  the task has control of all of the system's

resources.   But as  there are  other tasks in the system, a

running task  cannot  be  allowed  to  control  all  of  the

resources all  of the time.  Thus, MCX11 employs the concept

of multitasking.

2.3  MULTITASKING

Multitasking appears  to  give  the  computer  the  apparent

ability to  be performing  multiple operations concurrently.

Obviously, the  computer cannot  be doing two or more things

at once  as it  is a  sequential machine.  However, with the

functions of the system segregated into different tasks, the

effect of  concurrency can  be achieved.   In  multitasking,

each task  once  given  operating  control  either  runs  to

completion, or to a point where it must wait for an event to

occur, for  a needed  resource to become available, or until

it is  interrupted.   Efficient use  of the  computer can be

obtained by  using the  time a task might otherwise wait for

an event to occur to run another task.

This switching  from one  task to another forms the basis of

multitasking.  The result is the appearance of several tasks

being executed simultaneously.

2.4  PRIORITY AND SCHEDULING

When several  tasks can  be competing  for the  resource  of

execution time,  the problem is to determine how to grant it

so that  each gets  access to  the system in time to perform

its function.  The solution  is to assign a priority to each

task indicative of its relative importance to other tasks in

the system.   MCX11 uses a fixed priority scheme in which up

to 126  tasks may  be defined.   Tasks  which have a need to

Copyright (c) 1989         Page 2 - 2          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

respond rapidly  to events  are  assigned  high  priorities.

Those which perform functions that are not time critical are

assigned lower priorities.

It is  the priority of each task that determines where it is

to run  in the  hierarchy of  tasks.   When a  task may  run

depends  on  what  is  happening  to  the  tasks  of  higher

priority.   Tasks are  granted execution  time in  a  strict

descending order  of priority.   While executing, a task may

be interrupted  by an  event which  causes a  task of higher

priority to be runnable.  The lower  priority task is placed

into a  temporary state  of suspension and execution control

is granted to the higher priority task.  Eventually, control

is returned to the interrupted task and it is resumed at the

point of  its interruption.   Thus,  when any  task is given

execution control,  no higher  priority task  can  be  in  a

runnable state.  This is a most important point to remember.

When all  tasks are  in  an  unrunnable  state,  control  is

granted to the null task.

The null  task is  a do-nothing task which allows the system

to run in an idle mode while waiting for an event which will

resume or  start a  higher priority  task.  The null task is

always the  lowest priority task in the system and is always

runnable. It is an integral part of the kernel.

2.5  INTERTASK COMMUNICATION AND SYNCHRONIZATION

MCX11 provides  an environment whereby two or more tasks can

communicate with one another.  The three major ways in which

this  is  done  are  through  the  mechanisms  of  semaphore

signalling, message  transmission, and  queues.  A semaphore

is actually  a flag  which contains  information  about  the

state of  the associated event.  Any event which is used for

task synchronization  will be  associated with  a particular

semaphore.   The occurrence  of  a  specific  event  can  be

signalled by manipulating the semaphore associated with that

event.

Message transmission  involves the  logical transfer of data

packets from  one task  to another.   These data packets are

called "messages".   Messages  are sent  from one  task  and

placed in  the "mailbox"  of the receiving task in the order

of the  priorities of  the senders.   Messages may be of any

format recognizable  by the sender and receiver and data may

be passed  in either direction.  That is, it is possible for

two tasks to alternate the roles of sender and receiver.

A third  technique whereby  two tasks  can  communicate  and

synchronize is  via  a  first-in-first-out  (FIFO)  queueing

mechanism.   The queueing  techniques used  by MCX11 involve

Copyright (c) 1989         Page 2 - 3          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

the physical  transfer (copying)  of data  packets from  one

task to  another.   Task  synchronization  due  to  queueing

operations is automatically performed by MCX11.

2.6  TIME BASED FUNCTIONS

An MCX11  system is  configured with an interval timer using

the Real  Time Interrupt (RTI) clock as a peripheral device.

The timer  permits  task  control  on  a  timed  basis.    A

generalized scheme  using  one-shot  and  cyclic  timers  in

conjunction with  semaphores is provided.  MCX11 efficiently

manages multiple  timers using  an ordered  linked  list  of

pending timer  events. A timer for an event is inserted into

the  linked   list  in   accordance  with  its  duration.  A

differential technique  is employed  so that  the timer with

the shortest  time to expiration is at the head of the list.

Timed  events  may  even  be  co-terminous.  Directives  for

scheduling and  cancelling timed events are an integral part

of the executive.

2.7  INTERRUPT SERVICE

Support  for  a  generalized  interrupt  service  scheme  is

provided within  the MCX11  kernel. The   Interrupt  Service

Routine (ISR)  code, however,  is provided  by the User. The

rules for  writing interrupt  service routines that function

with  MCX11   are  quite   simple  and  require  only  minor

housekeeping  chores.   MCX11  also   provides  the   common

Interrupt Service  Exit function.  This logic is used by all

interrupts to  determine if  a context switch is in order as

the result  of the interrupt. If no task switch is required,

the  interrupted  task  is  resumed  at  the  point  of  the

interrupt. If,  however, a  task switch  is  in  order,  the

interrupted task's  context is saved, the new task's context

is loaded and control is given to it.

Copyright (c) 1989         Page 2 - 4          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

SECTION 3

MCX11 CONTROL & DATA STRUCTURES

In order  to understand  how MCX11  works and  how to  build

real-time application  systems around it, it is necessary to

understand how its various control and data structures work.

This  section   describes   these   structures   and   their

interrelationships. The descriptions will include:

-    tasks

-    semaphores

-    messages and mailboxes

-    queues

-    timers

The MCX11  executive services  which deal with these classes

of control  and data  structures  will  be  presented  in  a

subsequent section.

3.1 TASKS

In a real-time application using MCX11, the functions of the

system are  assigned to  various tasks. MCX11 supports up to

126 tasks in a single system. The nature of each task is, of

course, application dependent and left to the imagination of

the system designer. However, there are attributes which all

tasks share. These include:

-    a task identifier

-    a priority

-    a task state table

-    a stack

-    a mailbox

-    an entry point

3.1.1 Task Identifier & Priority

Each task is identified by a numerical identifier which is a

number from  1 - 126. The task identifier, or number, serves

as a  reference  during  executive  operations.  Task  1  is

executed before task 2 which is executed prior to task 3 and

so on.  Thus, the  task number  is inversely  related to the

task's execution  priority. The  lower the  task number, the

higher the  priority. MCX11  uses this  fixed task numbering

scheme because  it is low in overhead and quite adequate for

most applications.  The task  number permits a direct access

to the  task's state table without the need for linked lists

or other commonly used technique.

Copyright (c) 1989          Page 3 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

3.1.2 Task State Table

The state  table is  commonly referred  to as a Task Control

Block (TCB).  A TCB  in MCX11  is contained  in two separate

sections in  order to  minimize the  use of system RAM. Only

those data about the execution state of the task are kept in

RAM. Static  information about the task is kept in ROM. Both

sets of  this TCB information are kept in arrays which allow

direct access  to the  data based  on the  task number. This

makes for very quick access without the commonly wasted time

of searching a linked list.

The RAM portion of the TCB contains only volatile data about

the task and includes the following:

-    the execution state

-    the stack pointer

-    the first link in the task's mailbox

Whenever the  MCX11 dispatcher  is looking  for  a  task  to

execute, it  examines the execution state variable to see if

the  task  is  runnable  or  not.  The  execution  state  is

contained in a single byte and a value of $00 indicates that

the task  is runnable.  Any other  content in the task state

and the task is blocked from running.

The stack  pointer contains  the address of the current top-

of-stack.

The first link in the task's mailbox contains the address of

the highest  priority message  waiting to be received by the

task. If no message is waiting, the pointer contains zero.

The ROM part of the TCB contains:

-    the initial execution state

-    the entry point

-    the base address of the stack

-    the address of the RAM part of the TCB

The initial  execution state  is  specified  by  the  system

designer at  the time  of system configuration. This becomes

the initial  content of the execution state. By specifying a

value of  zero, the  task will be executed immediately after

initialization   in    accordance   with    its    priority.

Specification of a non-zero value will prevent the task from

automatically starting execution after initialization.

The entry  point is  the address  where the task is to begin

execution.

Copyright (c) 1989          Page 3 - 2          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

The base  address of  the stack  identifies that location in

RAM where the task's stack begins.

The address of the RAM part of the TCB table is included for

completeness and  is sometimes  used to save time in setting

up the address of the RAM TCB area.

3.1.3 Stack

Each task must have a stack on which the context of the task

is saved  during those periods when the task is not actually

executing. The  base address  of the  stack is stored in the

ROM portion  of the  TCB. The  stack extends  from the  base

address downward  in memory, i.e. towards address $0000, for

a number  of bytes specified by the given stack size for the

task. The  size of  each task's  stack is  dependent on many

things such  as the  maximum  depth  of  nested  subroutines

calls, the  maximum  amount  of  working  space  needed  for

temporary variables,  and the  size of any stack frames used

by the  task. At  minimum, the  size of the stack must allow

for the  storage of  a complete  context which requires nine

(9) bytes.  If XIRQ  is to  be used  in the design, an extra

nine (9) bytes must be allocated to each task's stack due to

the possibility of stacking an IRQ followed by an XIRQ. This

is possible  because the  XIRQ  cannot  be  masked  and  can

interrupt an IRQ interrupt service routine at any point.

In addition to the stacks needed by the tasks, there is also

the need  for a stack for MCX11. This system stack must have

sufficient space  to handle the maximum number of interrupts

possible  at  any  given  time,  less  one  interrupt.  Each

interrupt requires  nine bytes. MCX11 functions have a worst

case stack  requirement of  seven (7)  bytes  not  including

those bytes required for the system stack.

The sum  of all  of the  stack requirements  should  not  be

allowed to exceed available RAM or there will be trouble.

3.2  SEMAPHORES

Semaphores (event  flags) are  used to synchronize two tasks

to the  occurrence of  an asynchronous  event.  One task may

need to  wait for  the other to reach a certain point before

it can  continue.  Input/output operations are an example of

this type  of synchronization.   For instance, when an input

from an  external device  is desired by a task, it must wait

for the  input event  to occur.  When the input operation is

completed, the  device driver  signals that  the  event  has

occurred  which  indicates  the  data  is  available.    The

signalling  process  causes  the  waiting  task  to  resume,

Copyright (c) 1989          Page 3 - 3          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

presumably to process the input data. The synchronization of

the task with the event is done with the use of a semaphore.

There are  several forms  that a  semaphore may  take in the

design of  a real-time kernel.  MCX11 uses a basic semaphore

construct that  is known to handle most events and is low in

overhead.  A   semaphore  is   referenced  by   a  numerical

identifier limited  to  a  value  from  1-255.  A  semaphore

identifier of  zero has  a special  meaning. There are three

different classes of semaphores used by MCX11 as follows:

-     Named semaphores

-     Task semaphores

-     Queue semaphores

Named semaphores  are those  normally referenced by the user

in the  application code. Task and queue semaphores are used

by MCX11  internally for  the purpose  of orthogonality with

the semaphore  service functions. Task semaphores are always

referenced by the identifier of zero (0) which is translated

by MCX11  into the  proper number  for the  task  semaphore.

Queue semaphores should never be referenced by the user lest

undesirable results should occur.

An MCX11  semaphore is a single byte (8 bits) which contains

a value  representing one  of the  three possible  states in

which it can exist.  These states are:

-     PENDING

-     WAITING

-     DONE

A PENDING state indicates that the event associated with the

semaphore has  not yet  occurred and  is therefore  pending.

The WAITING  state shows that not only has the event not yet

occurred but  a task  is waiting for it to happen.  The DONE

state tells  that the  event has occurred.  MCX11 semaphores

have a  very  strict  state  transition  protocol  which  is

automatically managed  by  MCX11.    The  permissible  state

changes are shown below in Figure 3-1.

+------- PENDING------- +

|           ^           |

|   |------ |           |

v   |                   v

DONE<--------------- WAITING

Figure 3.1

Copyright (c) 1989          Page 3 - 4          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

If a  task attempts  to wait  on a semaphore which is in the

DONE state,  the  wait  does  not  occur  and  the  task  is

immediately resumed  (since the  desired event  has  already

happened)  and  the  semaphore  state  is  changed  back  to

PENDING.   If a  task attempts to wait on a semaphore in the

PENDING state,  the state  of the  semaphore is  changed  to

WAITING and  the task's  execution is  suspended  until  the

event occurs.  An attempt  to wait  on a  semaphore which is

already in the WAITING state can cause unpredictable results

and should  not be  attempted. It  should  be  considered  a

design error.

If an  attempt is  made to  signal a semaphore which is in a

PENDING state,  the semaphore is placed into the DONE state.

As this  action does not concern any task, no further action

is taken.   However,  if a  signal is sent to a semaphore in

the WAITING state, the semaphore transitions to a DONE state

and the  task waiting  on the  event is made runnable again.

The semaphore  is then  automatically set  back to a PENDING

state by MCX11.

3.3  MESSAGES AND MAILBOXES

Messages are  one of  the means by which data is transmitted

from a  sender to  a receiver task. Every task running under

MCX11  is  capable  of  being  both  a  message  sender  and

receiver. Each  task is  assigned a "mailbox" into which are

placed all  messages sent  to it. When a sender task sends a

message, it is put into the mailbox of the receiver task. If

two or more tasks are sending messages to a single receiver,

their messages  are put  into the mailbox in strict sequence

of their priorities, highest priority first.

The content  of the  message is  not actually moved from the

sender to  the receiver  when it  is sent. Instead, messages

are linked  into a threaded list according to the priorities

of their  senders. When the receiver requests receipt of the

next  message,  its  address  is  returned.  This  technique

guarantees the receiver that the highest priority message is

received first.  It is  possible,  however,  to  temporarily

suspend this  order by requesting only those messages from a

particular sender.  This is  useful when it is desirable not

to mix  messages  on  a  shared  resource,  for  example,  a

printer.

The mailbox  of a  task is  a threaded  (linked) list of all

messages sent  to but  not yet  received by  that task.  The

mailbox consists  of a pointer (link) to the message sent by

the highest  priority sender.  That message is linked to the

next highest  priority message  and it  in turn is linked to

the third  highest priority message, and so on until the end

Copyright (c) 1989          Page 3 - 5          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

of the  thread. The  last message  in a  task's mailbox will

contain a NULL (value of zero) link.

Messages may  be sent with or without waiting for a response

from the  receiver. When  a task wants to send a message and

wait until  the receiver  has processed the message, it will

include a  semaphore number  as one  of the arguments in the

MCX11   send  message  directive.  That  semaphore  is  then

associated with  the event  of completed  processing of  the

message. Once  the message is linked into the mailbox of the

receiver task,  the sender  is put  into a WAIT state on the

message processing complete event.

The receiver  removes  the  message  from  the  mailbox  and

processes it  according to  the content of the message body.

When the  receiver no  longer needs  the data in the body of

the message,  it signals  the semaphore  associated with the

completion of  message processing   making  the sender  task

runnable again  and, thereby,  allowing it  to continue  its

operation.

It should  be remembered  that the  body of  the message can

also be  used by the receiver to send a response back to the

sender. This  is a  very efficient  way of  passing data bi-

directionally between two tasks with little overhead.

If the  sending task  does not wish to wait on the action of

the receiver  or if there is no response required, a message

can be  sent without  waiting for  completion of processing.

This makes  it  possible  for  a  sender  to  send  multiple

messages to  a receiver,  or, simply do something else while

the receiver  processes the  message. However, even though a

message is  sent without  waiting for  the response,  it  is

still possible   at  a later time for the sender to wait for

the completion of processing event.

If the  receiver has completed the use of the message by the

time the  sender waits  for that  event,  the  sending  task

continues operation  without interruption.  If the  receiver

has not  yet completed processing of the message, the sender

must wait  for the  event to  occur. When it does occur, the

sender's operation is resumed.

A message is a four-element structure set up in a contiguous

area of  RAM. The  first three  elements of  the message are

used by  MCX11 to  control the  transmission. The  user need

only set up the content of the message body. The format of a

MCX11 message is as follows:

-    Mailbox link (supplied by MCX11)

-    Task number of sender (supplied by

MCX11)

Copyright (c) 1989          Page 3 - 6          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

-    Message semaphore number (optional)

-    Message body (supplied by User)

The first  element in  the message  provides the link to the

next message, if any, in the mailbox.

The second  element is  the task  number of the sender. This

field is  included so that the receiver task can selectively

receive only those messages from a specified task.

The third  element  of  the  message  contains  an  optional

message semaphore  number.   Typically, when  a sending task

wants to  send a  message and  wait until  the receiver  has

processed the  message, it will include a non-zero semaphore

number as  one  of  the  calling  arguments  for  the  MCX11

directive. The  sending task  will  then  suspend  execution

until the  receiver task  completes the  processing  of  the

message content.  When the receiver no longer needs the data

in the body of the message, it signals the message semaphore

to notify the sender that it may continue processing.

.

The last  element, the  body, of  the  message  may  contain

whatever data  is required  by  the  receiver  task  and  in

whatever format.  It may  be fixed  or variable in length as

required.

3.4  QUEUES

MCX11  supports   first-in-first-out  (FIFO)  queues  having

single or  multiple bytes per entry.  The queues can support

multiple producer/single  consumer service  but are  usually

employed in  only a  single producer/single  consumer  mode.

Queues are  referenced via  a numerical  identifier which is

unique to  each queue.   Queue  numbers are  assigned in the

system generation  procedure.    MCX11  queues  differ  from

messages in  that the  actual entry  data  rather  than  its

address is  entered into or removed from the queue.  Another

difference  is   that  the   queue  entries   represent  the

chronological order  of processing.  Also, the priorities of

the sender  or receiver are not considered as with messages.

Queues are  usually used  to handle  data such  as character

stream input/output or other data buffering.  Queues require

more overhead  as they  move the data from the source to the

queue  or   vice  versa  but  they  also  provide  automatic

synchronization  between   the  queue's   producer  and  its

consumer.   It is  recommended that  they not  be used where

high throughput rates are required.

A queue  consists of  two parts:   the  queue header and the

queue body.   The  queue header  contains  that  information

needed by the MCX11 internal routines to enable them to move

Copyright (c) 1989          Page 3 - 7          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

data into  and out of the queues properly.  The organization

of the queue header consists of the following elements:

-     Width, queue entry size (in bytes)

-     Depth, maximum length (in entries)

-     Current size of the queue (in entries)

-     Put index (in bytes; MCX11 internal use)

-     Address of queue body

-     Queue not empty semaphore number

-     Active semaphore number

The first  three elements  are  defined  during  the  system

generation process.   The  Current Size  and the  Put  Index

fields are initialized to zero (empty). Both of these fields

are  maintained  internally  by  the  MCX11  queue  handling

routines. They should never be manipulated by a user task.

Semaphores are used to signal internal events such as queue-

not- empty  or   queue-not-full. The  semaphores  for  these

events are allocated as a consecutive pair during the system

initialization process.  The queue-not-full semaphore number

is equal to the number of the queue-not-empty semaphore plus

one. A  queue can have a semaphore active only for those two

mutually exclusive  events. Consequently only the queue-not-

empty semaphore  number requires  definition  in  the  queue

header.

The active  semaphore number  field in  the queue  header is

used to  contain the number of the semaphore associated with

one of  the two possible events. If the queue is empty and a

task attempts to remove an entry, the active semaphore field

in the  queue header  is set  to contain  the number  of the

queue-not-empty semaphore. Conversely, if a task attempts to

put an  entry into a queue which is already full, the active

semaphore field  is set  to contain the number of the queue-

not-full semaphore.

By use  of these  semaphores, it  is possible  to send  data

between tasks  asynchronously but without the worry of over-

running the queue.

3.5 TIMERS

In the descriptions of time based functions to follow in the

succeeding sections,  it will be important to understand the

conventions used  by MCX11.  All timers in MCX11 are 16-bits

long. The  two time units used internally by MCX11 are ticks

and tocks.   A  tick gives  the amount of time between clock

generated system  interrupts, or  equivalently,  the  period

between clock interrupt service requests.

Copyright (c) 1989          Page 3 - 8          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

A tock  is an  integral number of ticks and is the time unit

used by  all of  the time dependent functions in MCX11.  The

tick frequency  of the  RTI clock  is set  during the system

initialization process but was determined, as was the number

of ticks  per tock,  by the  system  designer  during  MCX11

system configuration.

As an  example, suppose  the hardware clock is configured to

cause interrupts every 25 milliseconds and that 0.25 seconds

(250 milliseconds)  is the time most convenient interval for

timing events  in the application. In order to determine the

number of ticks per tock, simply divide the desired duration

of the  tock (250  ms) by  the duration  of a  tick (25 ms).

Thus, in this case, 1 tock = 10 ticks.

Timer values  are defined  as  the  number  of  clock  tocks

required to  form the  needed amount  of time.   If  a timer

value is  positive, it  is used  only once (one-shot timer).

However, if  the value  is negative,  it is used as a cyclic

timer having  a period  equal to  the 2's  complement value.

For example,  if a  clock tock  equals 250  milliseconds,  a

timer value  of 20  would produce  a once-only  period of  5

seconds.   If the  timer were  -20, the  period is  still  5

seconds but the timer is cyclic.

All timed  event operations  and data structures are handled

by MCX11. A task should not attempt to manipulate any of the

timer related control or data structures.

Copyright (c) 1989          Page 3 - 9          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

SECTION 4

EXECUTIVE SERVICE REQUESTS (ESRS)

4.1  PURPOSE

Executive Service  Requests (ESRs)  are the functions that a

real-time kernel  performs and  serve to give it its flavor.

This  section   will  describe  the  classes  of  the  MCX11

directives.  The directives are divided into five classes as

follows:

-     Semaphores

-     Messages

-     Queues

-     Task control

-     Time based

A brief  functional description of each ESR within the class

is given  along with their respective calling arguments. The

calling arguments  needed  for  each  ESR  appear  below  in

parentheses. Where  two or more arguments are required, they

are separated  by commas.  These are  not the actual calling

sequences but  are for  purposes of explanation only. Actual

calling sequences are given in Section 5.

In the  paragraphs below,  the ESR function name is enclosed

within two  periods. This  will be  the convention  used for

MCX11 to denote an ESR. The following abbreviations are used

below for the arguments:

-     SEMA      Semaphore number

-     TASK      Task number

-     QUEUE     Queue number

-     TOCK      Timer value

-     PNTR      Address pointer or data value

-     MSG       Message address

4.2  SEMAPHORE ESRS

A complete  set of  directives for  managing  semaphores  is

provided by MCX11.

.wait. (SEMA)       wait for a specific sema to be signalled

.signal. (SEMA)     signal semaphore

.pend. (SEMA)       reset semaphore state to PENDing

Copyright (c) 1989         Page 4 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

4.3  MESSAGE ESRS

The message directives provide a means of transferring large

amounts of  data between  tasks with  minimal overhead since

only pointers  (addresses)  are  passed.    Message  receipt

acknowledgement is  also provided  for task synchronization.

The format  of a  MCX11 message  and function prototypes are

noted.

.send. (MSG,TASK)        send message to a task

.sendw. (MSG,TASK,SEMA)  send & wait acknowledgement

.receive. (TASK)         receive message

4.4  QUEUE ESRS

Queue directives  provide a  means of  passing multiple byte

packets of  information between  tasks with  automatic  task

synchronization of queue full and empty conditions.

.dequeue. (QUEUE,PNTR)  remove entry from queue

.enqueue. (QUEUE,PNTR)  insert entry into queue

4.5  TASK CONTROL ESRS

The task  control directives provided by MCX11 allow for low

level  complete   control  of  tasks  and  their  respective

interactions.

.resume. (TASK)         resume specific task

.suspend. (TASK)        suspend specific task

.terminate. (TASK)      terminate specific task

.execute. (TASK)        execute task from starting point

4.6  TIME BASED ESRS

The time based directives provide for the synchronization of

tasks with  timed events.   In  addition, a generalized time

based  semaphore   scheme  for   more  advanced  time  based

requirements is provided.

.delay. (TASK,SEMA,TOCK)  delay task

.timer. (TASK,SEMA,TOCK)  schedule timer for task/sema

.purge. (TASK,SEMA,PNTR)  purge timer for task/sema

Copyright (c) 1989         Page 4 - 2          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

SECTION 5

ALPHABETICAL LISTING OF ESRs

In  the   pages  to  follow,  each  ESR  will  be  shown  in

alphabetical order.   Each  ESR will  be given in a standard

format:

Name - brief functional description

CLASS

One of the five ESR classes of which it is a member.

SYNOPSIS

The prototype  declaration including  argument  typing.

The decimal  value of  the function code is enclosed in

brackets [xx].

DESCRIPTION

A description  of what  the ESR  does, data types used,

etc.

CALLING SEQUENCE

The formal  AS11 calling  sequence  for  the  ESR.  The

following notations will be used:

ACCA = Accumulator A, 8 bits

ACCB = Accumulator B, 8 bits

IX = Index register X, 16 bits

RETURN VALUE

A description of the return values from the ESR.

EXAMPLE

One or  more typical ESR uses.  The examples assume the

syntax of AS11 cross assembler.

SEE ALSO

List  of   related  ESRs  that  could  be  examined  in

conjunction with the current ESR.

Copyright (c) 1989         Page 5 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.1  .delay.  -  DELAY A TASK FOR A PERIOD OF TIME

CLASS

Timer

SYNOPSIS

.delay. (TASK,SEMA,TOCK)       [13]

DESCRIPTION

The delay  directive places  the specified  task into a

Wait state  for the  period defined  by the TOCK value.

TASK is  a task  number in the range of 1-126. Or, TASK

may be 0 if the reference is to SELF. Thus, a task need

not know  its own  task number  to schedule a delay for

itself. SEMA  is the  semaphore number  associated with

the  expiration  of  the  timer.  It  may  be  a  named

semaphore if  its number  > 0.  Or, it  may be the task

semaphore of  the calling task if it is specified as 0.

TOCK is  a 16-bit timer. If TOCK is > 0, the timer is a

one-shot. If  TOCK <  0, the  timer is  cyclic. Caution

should be  exercised when  scheduling delays  for other

tasks.

CALLING SEQUENCE

ACCA = Task number (0 if SELF)

ACCB = Semaphore number (0 if Task's semaphore)

IX = Number of clock tocks to delay

swi

FCB  .delay.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

Assume that  the RTI  is used  as the time base using a

2**23 crystal  and the  clock TICK  is  defined  to  be

15.625 msec.  Further assume that there are 8 TICKS per

TOCK so  that a TOCK is 125 msec. If a 2.5 sec delay is

desired for  the calling  task using a named semaphore,

TIMEOUT, the request would appear as:

Copyright (c) 1989         Page 5 - 2          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

TOCK      equ   8

TOCKTIME  equ   1000/8         TOCKTIME = 125 msec

TIMEOUT   equ   3              Named semaphore #3

.

:

clra                 Task is self (ACCA=0)

ldab  #TIMEOUT       Semaphore = TIMEOUT

ldx  #2500/TOCKTIME  Timer = 20 TOCKS

swi

FCB  .delay.

.                    Point of continuation

The requesting  task would  be put into a WAIT state on

semaphore TIMEOUT  while the  2.5 sec  clock runs down.

When it  expires, TIMEOUT  would be  signalled and  the

task would  continue at  the instruction  following the

FCB declaration.

SEE ALSO

.timer., .purge.

Copyright (c) 1989         Page 5 - 3          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.2  .dequeue.  -  GET AN ENTRY FROM A FIFO QUEUE

CLASS

Queue

SYNOPSIS

.dequeue. (QUEUE,PNTR)       [7]

DESCRIPTION

Dequeue is  used to get an entry from a FIFO queue. The

number of bytes in the entry is determined by the Width

entry in  the Queue  Header. If the queue is empty, the

calling task  is placed into an EVENT WAIT state on the

queue-not-empty semaphore.   If the queue is not empty,

the oldest  entry in  the queue is removed and returned

to the  calling task. If the size of the queue entry is

greater than two bytes, the returned data is in the RAM

area beginning at the address specified by PNTR. If the

queue entry  is one  or two  bytes, PNTR is ignored and

the entry is returned in one or two registers.

CALLING SEQUENCE

ACCA= Queue number

IX = Destination address of queue entry if size > 2

swi

FCB .dequeue.

return

RETURN VALUE

If queue entry size = 1, ACCA= queue entry

If queue  entry size = 2, ACCA = byte 1 of queue entry,

and ACCB = byte 2 of queue entry

Other registers unchanged

EXAMPLE

Example #1:

There  exists   a  FIFO  queue,  DBLQ,  containing  two

character entries.  The queue  currently contains  four

entries as follows:

ab

DE

Mc

xY

Copyright (c) 1989         Page 5 - 4          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

A task  is programmed to remove the oldest entry in the

queue and  return it  for  subsequent  processing.  The

entry is dequeued by:

DBLQ      equ  4              Queue #4

.

:

ldaa  #DBLQ         ACCA = queue # 4

swi

FCB  .dequeue.

.                   Point of continuation

At the  instruction following the FCB declaration, ACCA

and ACCB will contain the two characters, ab. The queue

will now contain the three entries:

DE

Mc

xY

Example #2:

Another queue  exists, QUADQ,  which contains four byte

entries and  is currently  empty. The request to get an

entry from the queue would appear as:

QUADQ     equ   5             Queue #5

.

:

ldaa  #QUADQ        ACCA = queue # 5

ldx   #destn        IX = destination address

swi

FCB  .dequeue.

.                   Point of continuation

:

destn     ZMB   4             Where the entry is placed

:

Since there  are no entries available to dequeue, MCX11

puts the  task into  a  WAIT  state  on  the  NOT_EMPTY

semaphore associated  with  queue  #5.  The  task  will

remain in  that state  until another task puts a 4-byte

entry into  QUADQ. When  that happens, the waiting task

is put  into the RUN state. The entry is removed and is

stored at the specified destination address, destn, and

the task continues at the instruction following the FCB

declaration.

SEE ALSO

.enqueue.

Copyright (c) 1989         Page 5 - 5          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.3  .enqueue.  -  INSERT DATA INTO FIFO QUEUE

CLASS

Queue

SYNOPSIS

.enqueue. (QUEUE,PNTR)     [8]

DESCRIPTION

Enqueue inserts  an entry  into a  FIFO queue.   If the

queue is  full, the calling task is set to a WAIT state

on the queue NOT FULL condition.  When the queue is not

full, the  entry is  inserted into  the queue.  If  the

width of  the queue  entry is greater than 2 bytes, the

data to  be put  into the queue is found at the address

specified by  the contents of register IX. If the width

is 1  byte, the  actual data to be enqueued is found in

bits 8-15 of IX. If the width is equal to 2 bytes, then

byte 1  of the  entry is  found in bits 8-15 of IX, and

byte 2 is in bits 0-7 of IX.

CALLING SEQUENCE

ACCA= Queue number

IX = Source address of entry to be enqueued if size > 2

Or, IX8-15 = byte to be enqueued if size = 1

Or, IX8-15 = byte 1 and IX0-7 = byte 2 if size = 2

swi

FCB .enqueue.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

Example #1:

Using the  2-byte queue,  DBLQ, defined  above, a  task

wants to  put an  entry into  it. Assume that the queue

contents are the three entries:

DE

Mc

xY

The queue  entry size  is two  bytes which  permits the

entry to  be passed  to the  queue routine  in ACCA and

Copyright (c) 1989         Page 5 - 6          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

ACCB. The  request to  put a  two-byte entry,  LW, into

DBLQ is:

ldaa  #DBLQ         ACCA = queue # 4

ldx   #"LW"         IX = "LW"

swi

FCB  .enqueue.

.                   Point of continuation

Since the  queue has content, no task is waiting for an

entry to  dequeue; therefore, the entry is put into the

queue following  the "xY"  entry and the task continues

at the  instruction following  the FCB. The new content

of DBLQ is:

DE

Mc

xY

LW

Example #2:

Using the  queue, QUADQ,  previously  defined  but  now

containing three entries:

abcd

EFGH

iJkL

A request  to put  a new entry into the queue, MnOp, is

given by:

ldaa  #QUADQ        ACCA = queue # 5

ldx   #srcadr       IX = source address

swi

FCB  .enqueue.

.                   Point of continuation

:

srcadr    FCC   "MnOp"

:

Since the entry size is four bytes, IX must contain the

address of  the entry  to be enqueued. After continuing

at the instruction following the FCB declaration, QUADQ

contains:

abcd

EFGH

IjKl

MnOp

Copyright (c) 1989         Page 5 - 7          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

SEE ALSO

.dequeue.

Copyright (c) 1989         Page 5 - 8          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.4  .execute.  -  EXECUTE A TASK

CLASS

Task control

SYNOPSIS

.execute. (TASK)     [12]

DESCRIPTION

The execute  task directive is used to start or restart

a task from its beginning. The program counter (PC) and

stack pointer  (SP) are  initialized to  their starting

values and the specified task is made runnable.  If the

new task is of higher priority than the current task, a

context switch is performed and the new task runs, else

control is  returned to  the  caller.  The  task  being

started cannot be SELF (task number argument = 0).

CALLING SEQUENCE

ACCA= Task number

swi

FCB .execute.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

The current task wishes to cause task number 6 to begin

execution at  its starting  point.  Task  number  6  is

currently IDLE. The request appears as:

TASK6     equ   6             Task #6

.

:

ldaa  #TASK6        ACCA = task # 6

swi

FCB  .execute.

.                   Point of continuation

If TASK6  is of  higher priority, then the current task

is interrupted,  but still  runnable, and TASK6 is made

the current task. The task which scheduled TASK6 is not

given control  again until  all other  higher  priority

tasks, including  TASK6, are not runnable. When it does

continue, it will do so following the FCB declaration.

Copyright (c) 1989         Page 5 - 9          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

If TASK6 is of lower priority that the current task, no

context switch occurs and the current task continues at

the instruction following the FCB declaration.

SEE ALSO

.terminate.

Copyright (c) 1989         Page 5 - 10          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.5  .pend.  -  FORCE A SEMAPHORE TO A PENDING STATE

CLASS

Semaphore

SYNOPSIS

.pend. (SEMA)       [3]

DESCRIPTION

To understand  the use  of the  pend directive,  recall

that the  WAIT directive  will return  control  to  the

calling task  immediately if  the task requests to wait

on a semaphore found to already be in a DONE state.  In

this  sense,   semaphores  keep   a  history  of  their

associated events,  even if  no task  is waiting at the

time  of   the  signal.     If   a  task  needs  to  go

unconditionally into a Event Wait state on a semaphore,

a call should first be made to PEND followed closely by

a WAIT directive.

CALLING SEQUENCE

ACCB = Semaphore number

swi

FCB .pend.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

The current  task is  going to  make use of a semaphore

and wants  to be  certain of its state prior to use. It

forces the  semaphore into  a PENDing  condition by the

following request:

NEWSEMA   equ   14            Semaphore #14

.

:

ldab  #NEWSEMA      ACCB = semaphore # 14

swi

FCB  .pend.

.                   Point of continuation

SEE ALSO

.wait.

Copyright (c) 1989         Page 5 - 11          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.6  .purge.  -  PURGE A TASK'S TIMERS

CLASS

Timer

SYNOPSIS

.purge. (TASK,SEMA,PNTR)      [15]

DESCRIPTION

The purge  timer directive is used to remove all active

timers for  the specified  task or semaphore number.  A

specification of a zero semaphore number will purge all

semaphore timers for the given task.  When the purge is

completed, the current task is resumed.

CALLING SEQUENCE

ACCA = Task number (0 if SELF)

ACCB = Semaphore number (0 if task semaphore)

IX0-7 = Switch: 0 = purge all timers for given task

>0 = purge time for given task & sema

swi

FCB .purge.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

The most common use of this ESR is for the current task

to get  rid of  one or  all of its active timers. Since

timers are  associated with semaphores, the combination

of task  number and  semaphore number  is sufficient to

identify a  timer uniquely.  The request  to purge  the

current task's timer associated with the task semaphore

appears as:

clra                ACCA = 0 means SELF

clrb                ACCB = 0 means task sema

ldx   #1            Only purge one timer

swi

FCB  .purge.

.                   Point of continuation

Copyright (c) 1989         Page 5 - 12          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

If all  timers for a task are to be purged, the request

appears as:

clra                ACCA = 0 means SELF

ldx   #0            Purge all timers

swi

FCB  .purge.

.                   Point of continuation

SEE ALSO

.timer., .terminate.

Copyright (c) 1989         Page 5 - 13          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.7  .receive.  -  RECEIVE A MESSAGE

CLASS

Message

SYNOPSIS

.receive. (TASK)       [6]

DESCRIPTION

The receive message directive is used to fetch messages

from a  calling task's  mailbox.   Because the messages

are placed  in the mailbox in the order of the sender's

priority, they  are processed in the same sequence.  If

the receiving  task specifies  a non-zero  task number,

the first  message from  that task  will  be  returned.

With a  zero task  number, the  first  message  in  the

mailbox is  returned.   If there are no messages in the

mailbox, the  task's context  is saved, and the task is

put into  a Message  WAIT state.  When the next message

is sent  to the task it will be resumed and the attempt

to receive a message will be reissued.

CALLING SEQUENCE

ACCA =  Task number  if messages from specific task are

wanted. Or, ACCA = 0 if highest priority message

is wanted.

swi

FCB .receive.

return

RETURN VALUE

Register  IX  contains  the  address  of  the  received

message. Other registers unchanged.

EXAMPLE

Example #1:

The current  task is  a server which operates only when

another task  sends it  a  message  instructing  it  to

perform  some   function.  The   server  task  receives

messages by making a request such as:

clra                0 = next message

swi

FCB  .receive.

.                   Point of continuation

Copyright (c) 1989         Page 5 - 14          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

The server  task will  not continue  unless there  is a

message waiting  in its  mailbox. It will be put into a

WAIT state  until such  time as another task sends it a

message. No  waiting occurs  if a message is already in

the mailbox  at the  time  of  the  request.  When  the

message is available, its address is returned in the IX

register.

Example #2:

If the server wants to receive only those messages from

a specific  task, and  the task's number is to be found

in a variable named 'attach', the request is:

ldaa  attach        variable contains task #

swi

FCB  .receive.

.                   Point of continuation

:

attach    ZMB   1

The server  task will now only listen for messages from

the task  whose number is in the variable 'attach'. All

other messages  will  remain  in  the  mailbox  of  the

server. The  server will  continue  only  when  it  has

received a message from the specific task.

SEE ALSO

.send., .sendw.

Copyright (c) 1989         Page 5 - 15          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.8  .resume.  -  RESUME A TASK

CLASS

Task control

SYNOPSIS

.resume. (TASK)       [9]

DESCRIPTION

The resume  directive clears  the suspended  state of a

task caused  by a  suspend directive.   If  the resumed

task is  made runnable  and is  higher priority, a task

switch is performed.  Otherwise, control is returned to

the calling task.

CALLING SEQUENCE

ACCA = Task number

swi

FCB .resume.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

A task  which has  been  previously  suspended  may  be

resumed from the point of suspension only by a .resume.

request from another task. The request appears as:

TASK6     equ   6             Task #6

.

:

ldaa  #TASK6        ACCA = task # 6

swi

FCB  .resume.

.                   Point of continuation

Upon resuming  the given  task, a  context  switch  may

occur if  the resumed  task is  of higher priority than

the task  making the request. If not, no context switch

occurs.  The   requesting   task   continues   at   the

instruction following the FCB.

SEE ALSO

.suspend.

Copyright (c) 1989         Page 5 - 16          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.9  .send.  -  SEND A MESSAGE TO A TASK

CLASS

Message

SYNOPSIS

.send. (MSG,TASK)       [4]

DESCRIPTION

The send  message directive  inserts a message into the

target task's  mailbox in  the  order  of  the  sending

task's priority.   If  the receiving task is waiting to

receive a  message and  is of  higher priority than the

sender, a  task switch  is performed.  If the receiving

task is  of lower  priority and  waiting on  a message,

then the  receiving task is marked runnable but control

is  returned  on  a  message,  the  message  is  simply

inserted to his mailbox for later receipt.

CALLING SEQUENCE

ACCA = Task number of the receiver

ACCB = Semaphore number (task semaphore number = 0)

IX = Message address

swi

FCB .send.

return

RETURN VALUE

None.  ACCB   will  contain  the  number  of  the  task

semaphore if  ACCB was  = 0  at time  of swi. All other

registers unchanged.

EXAMPLE

A task wishing to send a message to a receiver task can

do so by the following code:

Copyright (c) 1989         Page 5 - 17          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

RCVRTSK   equ   12            Receiving task = task #12

MSGSEMA   equ   7             Use sema #7 for Message

.

:

ldaa  #RCVRTSK      ACCA = receiver task #

ldab  #MSGSEMA      ACCB = Message semaphore

ldx   #msgadr       IX = Address of message

swi

FCB  .send.

.                   Point of continuation

:

msgadr    ZMB   4             Message overhead

FCC   "THIS IS THE MESSAGE BODY"

The message  located at  'msgadr' will  be put into the

mailbox of RCVRTSK. Processing in the sending task will

resume   at   the   instruction   following   the   FCB

declaration.

SEE ALSO

.receive., .sendw.

Copyright (c) 1989         Page 5 - 18          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.10  .sendw.  -  SEND A MESSAGE AND WAIT

CLASS

Message

SYNOPSIS

.sendw. (MSG,TASK,SEMA)       [5]

DESCRIPTION

The send  message and  wait directive is similar to the

send message directive described above, except that the

sending task  is always  put into a Event Wait state on

the message  semaphore.  This allows the receiving task

to signal  the sender  when the  message processing  is

complete and/or  that there  is data  returned  in  the

message area.

CALLING SEQUENCE

ACCA = Task number of the receiver

ACCB = Semaphore number (task semaphore number = 0)

IX = Message address

swi

FCB .sendw.

return

RETURN VALUE

None.  ACCB   will  contain  the  number  of  the  task

semaphore if  ACCB was  = 0  at time  of swi. All other

registers unchanged.

EXAMPLE

The example  is the  same as for .send. except that the

ESR request  will be  .sendw. Another difference is the

WAIT state  into which the sending task is placed until

the receiving task signals the message semaphore. Until

that signal  is received,  the sending task proceeds no

further.  When   the  receiver   signals  the   message

semaphore,  the  sending  task  will  continue  at  the

instruction following the FCB.

SEE ALSO

.send., .receive.

Copyright (c) 1989         Page 5 - 19          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.11  .signal.  -  SIGNAL A SEMAPHORE

CLASS

Semaphore

SYNOPSIS

.signal. (SEMA)       [2]

DESCRIPTION

The signal  semaphore directive  sets the  state  of  a

specified semaphore  to DONE.    If  the  semaphore  is

currently in  a WAIT state, the Event Wait state of the

waiting task  is removed,  and  the  semaphore  is  set

PENDing.   If the  waiting task becomes runnable and is

of  higher  priority  than  the  signalling  task,  the

context of  the current  task is  saved, and  a context

switch is performed.

If the  state of  the semaphore  was either  PENDing or

DONE, the semaphore is placed in the DONE state and the

current task is resumed following the signal ESR.

CALLING SEQUENCE

ACCB = Semaphore number

swi

FCB .signal.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

Assume that  the current  task  wishes  to  signal  the

occurrence  of   a  particular   event.  The  event  is

associated with  a named  semaphore, EVNTSEMA. The task

signals the semaphore by the following procedure:

EVNTSEMA  equ   15            Named semaphore #15

.

:

ldab  #EVNTSEMA     ACCB = semaphore # 15

swi

FCB  .signal.

.                   Point of continuation

Copyright (c) 1989         Page 5 - 20          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

Depending on  the  state  of  semaphore  EVNTSEMA,  the

signal may  or may  not cause  a  context  switch.  The

signalling  task   will  continue   processing  at  the

instruction following the FCB declaration.

SEE ALSO

.pend., .wait.

Copyright (c) 1989         Page 5 - 21          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.12  .suspend.  -  SUSPEND A TASK

CLASS

Task control

SYNOPSIS

.suspend. (TASK)       [10]

DESCRIPTION

The suspend  directive causes  the specified task to be

placed into  a suspended  state.   The suspended  state

will remain  in force  until it is removed by a resume,

execute, or  reset  directive.    A  task  may  suspend

itself.  Task 0 indicates the SELF task.

CALLING SEQUENCE

ACCA = Task number (this may be SELF [0])

swi

FCB .suspend.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

The current  task wants  to suspend  itself until  some

indeterminate point  in the  future. To do so, it makes

the following request:

clra                ACCA = 0 means SELF

swi

FCB .suspend.

.                   Point of continuation

The task  remains in a suspended state until resumed by

a .resume.  ESR issued from another task. At that time,

the resumed  task  will  continue  at  the  instruction

following the FCB declaration.

SEE ALSO

.resume.

Copyright (c) 1989         Page 5 - 22          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.13  .terminate.  -  TERMINATE A TASK

CLASS

Task control

SYNOPSIS

.terminate. (TASK)       [11]

DESCRIPTION

The terminate  directive is  used to terminate a task's

operation.   During termination,  the task's  state  is

cleared and  the task  state is set to Terminated.  The

timer queue  is searched  and all active timers for the

task are  purged.   Also, all  messages pending for the

task are  removed.   A task  number of 0 indicates self

termination.   In all  cases, the highest priority task

will execute next.

CALLING SEQUENCE

ACCA = Task number (may be = 0 if terminating SELF)

swi

.terminate.

return

RETURN VALUE

None.  Registers   unchanged.  Returns   only  if   not

terminating SELF.

EXAMPLE

The current  task commits suicide by terminating itself

with the following procedure:

clra                ACCA = 0 means SELF

swi

FCB .terminate.

There is  no point of continuation following a suicide.

If another  task is terminated by the current task, the

current  task   will   continue   processing   at   the

instruction following the FCB declaration.

SEE ALSO

.purge.

Copyright (c) 1989         Page 5 - 23          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.14  .timer.  -  START A TIMER

CLASS

Timer

SYNOPSIS

.timer. (TASK,SEMA,TOCK)       [14]

DESCRIPTION

The timer  directive inserts  an entry of the specified

time duration  into the  timer queue.  The timer can be

cyclic or  one-shot.   Cyclic timers are indicated by a

negative  duration  while  one-shots  are  of  positive

duration.   Further  options  allow  the  timer  to  be

associated either  with a  specified task  number (0  =

SELF) or  a semaphore  number.    After  the  timer  is

inserted,  the  current  task  is  resumed.    See  the

Advanced Topic  section for task versus semaphore-based

timers.

CALLING SEQUENCE

ACCA = Task number (0 if SELF)

ACCB = Semaphore number (0 if task's semaphore)

IX = Number of clock tocks in timer. If > 0, timer is a

one-shot. If < 0, timer is cyclic.

swi

FCB .timer.

return

RETURN VALUE

None. ACCA will contain the current task number if ACCA

was = 0 at swi. Other registers unchanged.

EXAMPLE

Example #1:

Assume that the current task wants to set up a one-shot

timer of 5 seconds. Also assume that the RTI is used as

the time  base using a 2**23 crystal and the clock TICK

is defined  to be 15.625 msec. Furthermore, there are 8

TICKS per TOCK so that a TOCK is 125 msec. The setup of

a 5  second timer  associated with  a named  semaphore,

TIMEOUT, would appear as:

Copyright (c) 1989         Page 5 - 24          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

TOCK      equ   8

TOCKTIME  equ   1000/8         TOCKTIME = 125 msec

TIMEOUT   equ   3              Named semaphore #3

.

:

clra                 Task is self (ACCA=0)

ldab  #TIMEOUT       Semaphore = TIMEOUT

ldx  #5000/TOCKTIME  Timer = 40 TOCKS

swi

FCB  .timer.

.                    Point of continuation

The current  task would continue processing immediately

after the  ESR is finished because the .timer. ESR does

not cause the requestor to be put into a WAIT state nor

is there  a  context  switch  possible.  The  task  may

attempt to  wait on  the timer  semaphore at some later

time.

SEE ALSO

.purge.

Copyright (c) 1989         Page 5 - 25          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

5.15  .wait.  -  WAIT ON SEMAPHORE

CLASS

Semaphore

SYNOPSIS

.wait. (SEMA)       [1]

DESCRIPTION

The wait  directive is a fundamental function in MCX11.

It is used to make a task wait for a specified event to

occur.   The event  must be  associated with  the given

semaphore.   If the  semaphore is  found  to  be  in  a

PENDing state,  the task  is placed  into an Event Wait

state and  the semaphore is changed to WAITing, and the

context of the current task is saved.  If the semaphore

is in a DONE state, no wait occurs and the task resumes

immediately.   Upon task resumption, the wait directive

returns  the   semaphore  number  of  the  event  which

occurred.

CALLING SEQUENCE

ACCB = Semaphore number

swi

FCB .wait.

return

RETURN VALUE

None. Registers unchanged.

EXAMPLE

The current task wants to wait on a timer semaphore set

up by  a previous  .timer. request.  The semaphore is a

named semaphore, TIMEOUT. The procedure would appear as

follows:

TIMEOUT   equ   3             Semaphore #3

.

:

ldab  #TIMEOUT      ACCB = semaphore # 3

swi

FCB  .wait.

.                   Point of continuation

The current  task is put into a WAIT state on semaphore

TIMEOUT as  a result of this request. It will remain in

Copyright (c) 1989         Page 5 - 26          Motorola, Inc.

MCX11 - MicroController eXecutive for the 68HC11

this state  until another task, in this case, the clock

driver, signals  TIMEOUT  that  the  timer  period  has

elapsed.  The   task  would   then  continue   at   the

instruction following the FCB.

SEE ALSO

.pend., .signal.

Copyright (c) 1989         Page 5 - 27          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

SECTION 6

SYSTEM CONFIGURATION

6.1  SYSTEM CONFIGURATION CONCEPTS

Another configuration  concept central  to the  operation of

MCX11 is  that of  pre-definition.   Most real  time systems

used  in   embedded  operations   have   a   fairly   static

configuration.   That is to say, the equipment configuration

is not  subject to  frequent changes nor is the basic system

functionality.  Consequently, the system designer can define

the operating  environment  so  that  the  operating  system

dynamics are  avoided to  the greatest  degree possible.  In

other words,  the number of tasks concurrently executing may

change but  the relative  priorities of  tasks remain fixed.

Thus task  priorities can  be pre-assigned.   The  amount of

free memory  can be  computed  and  established,  number  of

queues and  their  sizes,  and  the  number  of  timers  and

semaphores are  all defined prior to runtime.  The result is

that MCX11  works in an established environment which yields

the effect   of  less system  overhead,  smaller  code,  and

faster execution;  all three being  desirable in a real time

system.

In order  to define  a  MCX11  system  configuration  it  is

necessary to  configure those  data and  control  structures

with  the   application  specific  information  which  MCX11

requires. The  configuration is done by creating AS11 source

code files  which define the necessary information needed to

assemble with  MCX11 source  code  to  define  a  particular

configuration.  The   following  paragraphs   describe   the

techniques.

6.2  MCX11 SYSTEM TABLE MEMORY REQUIREMENTS

MCX11 system  tables must  necessarily reside  in an area of

RAM.  This section is included to assist the system designer

in determining  the amount of ROM and RAM required for MCX11

tables. The  amount of  RAM and ROM may be computed from the

equations below  given the  following  system  configuration

information:

NTASKS    The number of tasks

NNAMSEM   The number of named semaphores

NQUEUES   The number of queues

NTIMERS   The number of timers

Copyright (c) 1989         Page 6 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

The symbols above are used by MCX11 and should be considered

reserved symbols.  These numbers  are obviously exclusive of

any RAM and ROM requirements of the application tasks.

Tasks

A Task  Control Block  occupies 5  bytes of  RAM and  7

bytes of ROM.

TASK bytes (RAM) = NTASKS * 5 + 5

TASK bytes (ROM) = NTASKS * 7

Semaphores

Each semaphore  requires 1  byte of  RAM.  Total system

area required  for semaphore  control is  the number of

named semaphores  + the  number of  tasks + 2 times the

number of queues.

SEMA bytes (RAM) = NNAMSEM + NTASKS + 2 * NQUEUES

Queues

Each queue  consists of  a queue  header and  the queue

body.   Each queue header requires 3 bytes of RAM and 6

bytes of ROM.  Each defined queue body requires an area

of RAM  defined by the product of its Width (W) and its

Depth (D).

QUEUE bytes (RAM) = NQUEUES * 3 + Sum(Wi * Di),

where Wi is the width and Di

is the depth of the ith queue,

and i ranges from 1 - NQUEUES.

QUEUE bytes (ROM) = NQUEUES * 6

Clock

Timers require  8 bytes  of RAM  each.   The total area

required for  timer control  and storage is the product

of 8 and the number of defined timers.

TIMER bytes (RAM) = NTIMERS * 8

Copyright (c) 1989         Page 6 - 2          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

SECTION 7

DEVICE DRIVER & INTERRUPTS

7.1  DRIVER CONCEPTS

Device  drivers   are  program   modules  which  provide  an

organized software  interface between  a physical device and

the application programs using the device.  The intent is to

mask the  specifics of  the device's  hardware peculiarities

from the application software.  By doing so, the application

code need  only conform  to the  protocol which  the  device

driver expects  in order to perform a function.  Each device

driver may  have a  protocol unique to its function, i.e., a

disk driver  has different  functional requirements  from an

analog-to-digital converter.

In order  to accommodate the wide range of devices which are

found in  real-time systems, the device drivers in MCX11 are

structured as  tasks.   As a  task, a  device driver has the

most flexible  environment with  complete access  to  system

resources via  the Executive  Service Requests.  Actually, a

device driver  usually consists  of two  parts:   the driver

task, and  the interrupt  service routine (ISR).  The latter

is used when the device can cause an interrupt to signal the

completion of a function or the availability of the device.

An ISR  exists to  handle an exception in the normal flow of

processing.   The general  philosophy of MCX11 drivers is to

minimize the time spent in the interrupt service routine and

let the  task portion  handle the  real work  of the driver.

This design  concept places  no restrictions on the designer

for either  the ISR or the task functionality. The structure

of the  ISR is  explained more  completely in  the following

section.

7.2  INTERRUPT SERVICE

The  classic   interrupt   processing   philosophy   is   to

acknowledge the  interrupt and  save the  current  processor

context.   Then a branch is made to a software routine where

the code  services the  specific interrupt  and performs any

required operations.  When the  interrupt service routine is

complete, control  of the  main program  is resumed  at  the

point of  interruption.   MCX11 uses  a modified  version of

that sequence.

The  MC68HC11  microcontroller  always  pushes  the  current

processor context of nine (9) bytes on the active stack when

an interrupt  occurs. That having been done, it acknowledges

Copyright (c) 1989         Page 7 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

the interrupt  and vectors automatically to the specific ISR

written for  that event.  The user must initialize each two-

byte interrupt  vector location  specific to his application

as part of the initialization code.

MCX11  presets   the  vector  locations  for  the  following

interrupts. All  others not  specifically  reserved  by  the

MC68HC11 are available to the user.

Event               Address

Clock (RTI)          $FFF0-$FFF1

MCX11 ESR request    $FFF6-$FFF7

Power On/Reset       $FFFE-$FFFF

MCX11 uses a modified version of the classical ISR approach.

The sequence is the same until the return to the interrupted

program is  made at  the  end  of  the  ISR.  In  MCX11,  an

interrupt may  be the  event for which a task is waiting. If

so, a  notification of  the event's occurrence must be given

to the  task. But  the possibility  exists that  the waiting

task has  a  higher  priority  than  the  interrupted  task.

Notification of  the event should remove the wait state from

the higher  priority task  thus making it runnable, assuming

it is not otherwise blocked.

At that  point, a  decision has  to be  made as  to  how  to

continue. Whichever  task is  of higher  priority,  the  one

interrupted or the one which was waiting, it will be the one

to resume  processing. If  it is  the interrupted  task,  it

resumes at  the point  of interruption  as  in  the  classic

sequence. If  the latter  task  is  of  higher  priority,  a

context switch  is performed in which it is made the current

task. Its  processor context  is restored from its stack and

the task  resumes at  the location  following the  ESR which

caused it to wait. The interrupted task remains runnable and

will continue at the point of its interruption at some later

time when it becomes the highest priority runnable task.

MCX11 provides  a common  exit routine  to  which  all  ISRs

should branch  if there  is the  possibility  of  a  context

switch as  the  result  of  the  interrupt.  The  function's

primary responsibility is to signal a specified semaphore of

the occurrence  of an  event and  to dispatch control to the

highest priority  task which is runnable after the interrupt

has been serviced.

7.2.1 Interrupt Processing Variables

In order  for MCX11 interrupt servicing to operate properly,

it is  necessary to  observe a  few rules of the road. There

Copyright (c) 1989         Page 7 - 2          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

are some  system variables  which are  used during interrupt

processing in  the ISR  and in  the common ISR exit routine.

These variables  must be used only in the ISR and should not

be used  elsewhere. A  listing of  the variables and a brief

description of  them follows. Their addresses are also given

as a  displacement from the base address of the MCX11 system

variables, MCXVAR.  MCXVAR may be located at any RAM address

but must  be predefined  and assembled with the MCX11 kernel

code.

intlvl   (address MCXVAR+11)

This byte  variable is  a  counter  of  the  number  of

interrupts stacked  but not  yet processed  completely.

When an  interrupt occurs in a task, intlvl will always

contain a value of zero. The ISR code must increment it

by 1  to indicate  that an ISR is in process. If a task

calls the  MCX11 kernel,  intlvl is  incremented  to  a

value of 1. If an interrupt were to occur while control

is in  the MCX11 kernel, intlvl would be incremented to

2 by the ISR, etc.

MCX11  allows  the  existence  of  interruptible  ISRs;

therefore, intlvl can assume any value up to the number

of possible interrupts in the system. As interrupts are

processed by  the common  ISR  exit  logic,  intlvl  is

decremented. A  resumption of  processing at  the  task

level can  only occur  when the  decremented  value  of

intlvl becomes  zero.  Otherwise,  the  non-zero  value

indicates the  existence of an interrupt which has been

incompletely serviced.  In that  case, the  common  ISR

exit logic  returns control  to the interrupted ISR and

resumes  processing   at  that   point.  Thus,   nested

interrupts and  re-entrant ISRs  are  handled  cleanly,

quickly, and automatically by MCX11.

curtsk  (address MCXVAR+5)

This byte  variable contains  the number of the current

task. It  is to be considered a READ-ONLY variable. The

user should  NEVER write to this variable. It is needed

during  ISR   processing  to  determine  if  the  MCX11

Dispatcher was interrupted.

curtcb  (address MCXVAR+6)

This  2-byte  variable  contains  the  address  of  the

current  task's  Task  Control  Block  (TCB).  The  TCB

contains an  entry for  the task's stack pointer. MCX11

interrupt  processing   requires  that  the  ISR  which

increments intlvl  to a  value of  1 must also save the

Copyright (c) 1989         Page 7 - 3          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

current task's  stack pointer in its TCB. The variable,

curtcb, is READ-ONLY.

7.2.2 Interrupt Processing Code

All ISRs  which have  the capability  of causing  a  context

switch should  incorporate the  following routine as in-line

code in  the ISR.  If the  ISR is  to be  allowed to  enable

interrupts, this code should precede the CLI instruction. As

a rule  it should  occur early  in the  ISR  but  there  are

exceptions. (Refer  to the  MCX11 clock  driver's ISR, whose

entry address  has the  symbol rtiisr, for a good example of

an exception to the rule.)

Note that the label, notlvl0, used in the code segment below

will need  to be changed for each ISR. The symbols ACTSP and

SYSTACK are  not variables  and should not be changed. ACTSP

is equated to the value of 1 and represents the displacement

into the  TCB where  the task's  stack  pointer  is  stored.

SYSTACK is the location of the base of the System Stack area

and is defined in the System Configuration source code file,

SYSTEM.AS.

tst   intlvl    Test for task level interrupt

bne   notlvl0   Branch if not task level int

tst   curtsk    See if Dispatcher interrupted

beq   notlvl0   Branch if so

ldx   curtcb    Get task's TCB address

sts   ACTSP,x   Save task's stack pointer

lds   #SYSTACK  Change to System Stack

notlvl0   inc   intlvl    Increment the interrupt level

After this  point, the  CLI instruction may be inserted

if desired.

The interrupt  service code  follows. An important factor in

interrupt  processing  throughput  is  the  length  of  time

interrupts are  disabled during  the  ISR.    In  all  ISRs,

interrupts must  be disabled long enough to perform the code

segment above and only then should interrupts be re-enabled.

Ideally,  interrupt   routines  should   be  performed  with

interrupts enabled to allow other interrupts to be serviced.

The exit  from the  ISR routine is via a branch to the MCX11

common Interrupt Service Routine Exit routine located at the

address having  the symbol,  isrrtn. This routine allows for

optionally signalling  a semaphore  during the exit process.

The ISR  may wish  to signal  a  semaphore  that  the  event

associated with  the interrupt  has occurred.  To do so, the

Copyright (c) 1989         Page 7 - 4          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

ISR simply  puts the number of the semaphore to be signalled

into the  B-Accumulator and the executes a branch to isrrtn.

From there  on, the  further processing  of the interrupt is

performed automatically  by MCX11. As an example, assume the

ISR wants  to signal a semaphore and that the label used for

the semaphore  is EVNTSEMA. Then the concluding code segment

in the ISR is

ldab  #EVNTSEMA   Load ACCB with semaphore #

jmp   isrrtn      Jump to exit logic.

If no  semaphore is to be signalled during the exit routine,

the ISR  should make  sure that the B-Accumulator is cleared

before branching to isrrtn.

Copyright (c) 1989         Page 7 - 5          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

SECTION 8

ADVANCED TOPICS

8.1  OTHER TOPICS

Other topics  of interest  will be  added to this section as

they are researched and documented.

Copyright (c) 1989         Page 8 - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

APPENDIX A

MCX11 DISTRIBUTED SOURCE FILES

MCX11 is  distributed as  a collection of files. Those files

and their purposes are briefly described below.

CLKDRVR.AS

The AS11  source code  of  the  RTI  clock  driver  and  RTI

Interrupt Service Routine.

ESREQU.AS

These are  just the  equates for  MCX11  ESRs  included  for

completeness.

MCX.AS

This is the AS11 source code of the MCX11 kernel. Unless you

are expanding  the functions  of  the  kernel  or  fixing  a

reported bug, this file probably won't change too much.

SYSTEM.AS

This AS11 source code file contains the application specific

information and  should be  studied very  closely.  It  also

contains initialization  code  which  may  be  necessary  to

change to meet the needs of a given application. The file is

pre-configured  with   the  specifications  needed  for  the

demonstration programs in file TEST.AS.

TEST.AS

This is  an AS11  source code  file containing 4 tasks which

demonstrate the  use of all of the MCX11 ESRs. It doesn't do

anything practical except to give the user a limited look at

how to accomplish certain operations.

VECTORS.AS

AS11 source  code containing  all of  the  vectors  used  by

MCX11.  This  file  may  need  to  be  edited  for  a  given

application.

ASSEMBLE.BAT

A MS-DOS  Batch file  used to  assemble MCX11  and the  TEST

programs. It also produces a listing file.

MANUAL.DOC

Copyright (c) 1989         Page A - 1          Motorola, Inc.

MCX11 - MicroController eXecutive for the MC68HC11

The MCX11 User's Manual is contained in this file. It can be

printed out directly to any ASCII compatible printer.

README.DOC

Some notes about getting started with MCX11.

Copyright (c) 1989         Page A - 2          Motorola, Inc.