INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG97/M2611

September 1997

Status:	 	Proposal

Title:		DMIF Implementation on Local Area Networks which Provide a Non-Guaranteed Quality of Service

Author:	Zvi Lifshitz (Optibase Ltd.)

SUMMARY

This proposal describes the way by which MPEG-4 terminals communicate over Local Area Networks (LAN) which do not provide a guaranteed Quality of Service.

The LAN over which the terminals communicate, may be a single segment or ring, or it may be multiple segments with complex topologies, including the Internet. The possible means by which quality of service might be assured on such types of LANs/Internet-clouds is beyond the scope of this proposal.

This proposal is based on ITU-T Recommendation H.323: “Visual Telephone Systems and Equipment for Local Area Networks which Provide a Non-Guaranteed Quality of Service”.  Like H.323, it makes use of H.255.0: "Media Stream Packetization and Synchronization for Visual Telephone Systems on Non-Guaranteed Quality of Service LANs", and H.245: “Control of communications between Visual Telephone Systems and Terminal Equipment”.  However, the use of H.225.0 and H.245 by MPEG-4 terminals require that these Recommendations will be amended.  This document proposes required changes to these Recommendations.  These kind of changes needed to be communicated through liaison with ITU-T.

Table of Contents

1. Normative References

2. Definitions

3. General

4. Implementation of Multicast Services

5. Call Signaling

6. RTP/RTCP

6.1 Payload Type

6.2 Time Stamps

6.3 RTCP

7. Control Functions

7.1 Master-Slave Determination

7.2 Capability Exchange

7.3 Logical Channel Signaling

7.4 Bi-directional Logical Channel Signaling

7.5 Close Logical Channel Signaling

7.6 Round Trip Delay Determination

7.7 Maintenance Loop Signaling

7.8 Elementary Streams Request

7.9 Stream Map Table Exchange

7.10 Decoder Ready Signaling

8. Proposed H.245 Extensions

9. Broadcast

# Normative References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this proposal.

[1]	Working Draft 5.0 of ISO/IEC 14496-1.

[2]	DMIF (Delivery Multimedia Integration Framework) ISO/IEC 14496-6 WD 1.0.

[3]	ITU-T Recommendation H.323 (1996): "Visual Telephone Systems and Equipment for Local Area Networks which Provide a Non-Guaranteed Quality of Service".

[4]	ITU-T Recommendation H.225.0 (1996): "Media Stream Packetization and Synchronization for Visual Telephone Systems on Non-Guaranteed Quality of Service LANs ".

[5]	ITU-T Recommendation H.245 (1995): "Control of communications between Visual Telephone Systems and Terminal Equipment".

[6]	ITU-T Recommendation Q.931 (1993): "Digital Subscriber Signaling System No.  1 (DSS 1) - ISDN User-Network Interface Layer 3 Specification for Basic Call Control".

# Definitions

For the purposes of this Proposal the definitions given in ISO/IEC 14496-1 WD [1], ISO/IEC 14496-6 WD [2], H.323 [3], H.225.0 [4] and H.245 [5] apply along with those in this section.

Unicast Service:  A unicast service is an instance of service, as defined in [2], when two terminals are exchanging control information and media streams through point-to-point channels.

Multicast Service:  A multicast service is an instance of service, as defined in [2], when one terminal sends media streams to multiple receivers.  The control information is exchanged between the sender and each of the receivers through point-to-point channels, and at least some media streams are transmitted via multicast channels.

Broadcast Service:  A broadcast service is an instance of service, as defined in [2], when one terminal sends media streams to multiple receivers, and both control information and media streams are transmitted through multicast channels.  There is no backchannel from a receiver to the sender.

# General

Unicast and multicast services are implemented in a similar way as H.323 conferences, with few changes that are recommended in this Proposal.  In the context of this Proposal, there is no difference between these two services, since H.323, H.225.0 and H.245 apply to both.  As a matter of convenience, both are referred hereinafter as multicast services.

The case of broadcast service had been left for further study in H.323, and was addressed later in ITU-T H.332.  Many aspects of H.332 do not fit the DMIF requirements.  This Proposal suggests principles for implementing broadcast services, but details are left for further study.

# Implementation of Multicast Services

A multicast service includes the following components:

- Call signaling.

- Transmission of media streams.

- Control.

These components are implemented in a way similar to H.323, as following:

Call signaling: Use the procedure described in H.225.0 for call signaling, with minimal changes as described in chapter 5.  It should be noted that in order to start a call, the originator must have some identification of the called party, such as an IP address or a ConferenceID.  The way by which this identifier is acquired is outside the scope of this Proposal.  [Editor: However, I am not convinced that this issue is sufficiently covered in [1] and/or [2]. I would recommend the editors of these documents to examine this issue carefully.]

Transmission of media streams:  The way media streams are packetized and the transport protocol which is used depends on their defined QoS.  If a stream requires a reliable channel, a native reliable channel will be used (e.g., a TCP channel on TCP/IP), and access units will use the native packetization of that protocol.  When the stream does not require a reliable channel, the mechanism of RTP/RTCP shall be used as defined in H.225.0.  Few issues relating to RTP/RTCP which are unique to DMIF are discussed in chapter 6.

Control:  To perform control actions such as exchanging capabilities or opening channels, H.245 functions shall be used.  The use of these functions is not identical to H.323, and the differences are described in chapter 7.  The use of H.245 also requires some additions to H.245, which are specified in section 8.

# Call Signaling

Call signaling is the first stage in the establishment of a service.  As result of that stage, the two terminals will have two reliable channels open between them.  One channel will serve for further call notifications throughout the life of the application, e.g., requesting a gatekeeper for bandwidth increase, as define in H.323 [3].  The other channel will be used for exchanging control information.

The procedure of call signaling is defined in H.225.0 [4].  This recommendation requires minimal changes to meet DMIF requirements, as following:

- In 7.2.2.1  Bearer Capability:  Layer 1 protocol - octet #5: Currently there are allowed values for G.711 and H.323.  We need a value for MPEG-4.

- In 7.6  H.225.0 RAS Message Common Parts: The SupportedProtocols structure should be extended as following:

SupportedProtocols 	::= CHOICE

{

nonStandardData	NonStandardParameter,

h310		H310Caps,

h320		H320Caps,

h321		H321Caps,

h322		H322Caps,

h323		H323Caps,

h324		H324Caps,

voice		VoiceCaps,

t120-only	T120OnlyCaps,

...

mpeg-4		MPEG4Caps

}

- In 7.15  Status Request Messages:  The infoRequestResponse structure should be extended as following:

InfoRequestResponse	::=SEQUENCE --(IRR)

{

nonStandardData	NonStandardParameter OPTIONAL,

requestSeqNum	RequestSeqNum,

endpointType	EndpointType,

endpointIdentifier	EndpointIdentifier,
	rasAddress	TransportAddress,

callSignalAddress	SEQUENCE OF TransportAddress,

endpointAlias	SEQUENCE OF AliasAddress OPTIONAL,

perCallInfo	SEQUENCE OF SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

callReferenceValue	CallReferenceValue,

conferenceID		ConferenceIdentifier,

originator		BOOLEAN OPTIONAL,

audio			SEQUENCE OF RTPSession OPTIONAL,

video			SEQUENCE OF RTPSession OPTIONAL,

data				SEQUENCE OF TransportChannelInfo OPTIONAL,

h245			TransportChannelInfo,

callSignaling		TransportChannelInfo,

callType		CallType,

bandWidth		BandWidth,

callModel		CallModel,

...

mpeg4NonReliable	SEQUENCE OF RTPSession,

mpeg4Reliable		SEQUENCE OF TransportChannelInfo

} OPTIONAL,

...

}

# RTP/RTCP

The use of RTP/RTCP for transmission of media streams which do not require reliable channels are described in H.255.0 [4].  This Proposal adopts those recommendations, with the following comments.

## Payload Type

The use of RTP requires the determination of a Payload Type (PT) for each media stream.  PTs can be static or dynamic.  The static PTs are a set of numbers which has been standardized by ITU for specific media types.  Dynamic PTs are negotiated by H.245 control functions.

PTs serve for three purposes:

- It enables the interleaving of several media streams in one RTP channel.  This has no significance for DMIF which has its own mechanism for similar purpose (FlexMux).

- It facilitates the operation of mixers and translators which can base their action on the PT.  [Editor: Does MPEG-4 have any interest in this?  If so, a set of static PTs have to be registered with ITU-T, and the Object Descriptor we find in MPEG-4 needs to include an optional extra field for payload type.]

- It helps filtering out alien packets in multicast channels.  For this purpose, the use of dynamic PTs will do.  In addition, we propose to register one payload type, for “MPEG-4 stream”.

## Time Stamps

H.225.0 recommends the use of the Time Stamp (TS) field in RTP headers for achieving synchronization.  For same purpose, MPEG-4 defines a set of time stamps handled by the adaptation layer, which is out of the scope of DMIF.  This Proposal recommends that RTP TSs should serve for different purpose.  A time stamp should specify the transmission time of a packet.  This value can then be used by the receiver to filter out packets which arrived at a delay that is not acceptable according to the QoS parameter of the channel.

## RTCP

H.225.0 requires the use of an RTCP channel for each open RTP channel.  RTCP channels help in analyzing the QoS achieved on the RTP channels, and provide some limited inband signaling capabilities.  The former can be useful for MPEG-4 channels.  MPEG-4 requires the definition of QoS for each media stream.  Based on that information the stream consumer may decide which stream to request for.  If the QoS that is actually achieved for a specific channel is below the one required, the audio or visual results might be unacceptable.  Using RTCP, DMIF may analyze the de-facto QoS achieved for a channel, and notify the application if the required QoS has not been matched.  Actions taken by applications in these cases are implementation specific.  One useful option would be to close the channel and require an alternative stream (if available) with a lower QoS.  [Editor: I don’t remember mechanism for that defined in DAI.  I believe it would be useful to add it].

# Control Functions

Successful completion of a call signaling procedure results in the establishment of a reliable channel between the two endpoints which is used by H.323 to exchange H.245 control functions (logical channel 0).  DMIF terminals shall follow this mechanism.

The H.245 Control Function uses the H.245 Control Channel to carry end-to-end control messages governing operation of the DMIF entity, including capabilities exchange, opening and closing of logical channels, and flow control messages.  In addition, DMIF defines operations which do not exist in H.323.  These include request for media-streams and exchange of stream-map-tables (SMTs).

H.245 defines a collection of signaling functions for media-type dependent control.  Unlike H.323, DMIF does not make use of these functions, because they are considered above the DMIF layer.  [Editor: This Proposal recommends H.245 extensions needed to establish an application-signaling channel.  Systems has yet to define the syntax and semantics for the use of this channel].

H.245 specifies a number of independent protocol entities which support endpoint to endpoint signaling.  A protocol entity is specified by its syntax (messages), semantics, and a set of procedures which specify the exchange of messages and the interaction with the user.  DMIF endpoints shall support the syntax, semantics, and procedures of the following protocol entities:

- Master/slave determination

- Capability Exchange

- Logical Channel Signaling

- Bi-directional Logical Channel Signaling

- Close Logical Channel Signaling

- Round Trip Delay Determination

- Maintenance Loop Signaling

The use of these for DMIF might require few extensions to H.245.  These are explained later on in this chapter and detailed syntax is proposed in section 8.

In addition, DMIF requires the introduction of the following protocol entities:

- Elementary Streams Request

- Stream Map Table Exchange

- Decoder Ready Signaling

We propose two alternative ways to carry on this kind of DMIF  requests.  One is to extend H.245 so it will support DMIF signaling.  The other is the use of a dedicated channel over which DMIF endpoints can exchange DMIF-specific information, in internal format and syntax.  The first way would save resources, and is also easier to implement.  The second way has the advantage that revisions of DMIF signaling can be made independently of H.245 and ITU.

The H.245 extensions proposed in section 8 support these two alternatives.  H.245 protocol entities for DMIF signaling is described later on in the current chapter.

DMIF terminals shall be capable of parsing all H.245 MultimediaSystemControlMessage messages, and shall send and receive all messages needed to implement required functions and those optional functions which are supported by the terminal. DMIF terminals shall send the functionNotSupported message in response to any unrecognized request, response, or command messages that it receives.

The use of protocol entities in DMIF is described in the following paragraphs.

## Master-Slave Determination

DMIF uses H.245 Master Slave Determination procedures during the establishment of a service. The semantics and syntax of these procedures are identical to H.323 [3].

## Capability Exchange

Capabilities Exchange procedures are used during the establishment of a service to ensure that the two terminals may interoperate. Both multiplex (i.e. transport layer) capabilities and media capabilities are exchanged.  The semantics and syntax of multiplex capabilities are like in H.323, but the media capabilities differ substantially.  Instead of referring specific media and decoder types, MPEG-4 profiles are exchanged.  This requires H.245 extensions which are detailed in section 8.

## Logical Channel Signaling

During the establishment of a service, a reliable channel is optionally created for application signaling. This can be either uni-directional of bi-directional channel. [Editor: Mechanism for that is not yet defined in DMIF].

Media streams channels may be opened anytime during the session by the sending endpoint.  These can be reliable or unreliable (RTP), unicast or multicast.

Logical Channel Signaling procedures are used for the creation of channels. Unlike H.323 which identifies channels by their media types, DMIF has no knowledge of the media attribute of each stream.  Streams are identified only by their ES_ids.  This requires H.245 extensions which are proposed in section 8.

In addition, the sending endpoint may distribute information about available multicast channels and their contents, with the use of the communicationsModeCommand function. This also requires H.245 extensions as specified in section 8.

[Editor: It is not clear whether the channel for the BIFS is automatically created during the service setup, or the application needs to explicitly ask for it (as ES_id #0).  IMO, It shall be explicitly requested, because media streams do not need to reside on same terminal as the BIFS stream, and therefore it is possible to have services with no BIFS stream.]

H.245 procedures for logical channel signaling make use of a logical channel number.  This corresponds to the DMIF concept of Channel Association Tag [Editor: I think. Maybe I am confusing the tag names].

## Bi-directional Logical Channel Signaling

The only case in which this can be required is for the creation of the application signaling channel, as explained in 7.3.  H.245 extensions needed for that are specified in section 8.

## Close Logical Channel Signaling

This follows the rules of H.323.  No H.245 extensions are needed.

## Round Trip Delay Determination

This follows the rules of H.323.  No H.245 extensions are needed.

## Maintenance Loop Signaling

This follows the rules of H.323.  No H.245 extensions are needed.

## Elementary Streams Request

As opposed to H.323, when sending endpoints may open channels based and their knowledge of the exchanged capabilities, DMIF terminals need to explicitly request channels which contain specific elementary streams.  For that purpose, a new protocol entity is proposed - Elementary Stream Request, which makes use of two new functions: elementaryStreamRequest and elementaryStreamResponse.  A syntax for these functions is proposed in section 8.

## Stream Map Table Exchange

Stream Map Tables (SMTs) is a concept which does not exist in H.323.  DMIF requires mechanism for exchanging SMTs.  Two protocol entities are introduced for this purpose. The first is initiated by the receiving side, which sends streamMapTableRequest and receives either streamMapTableResponse or streamMapTableReject.  The second is a one-way entity which is initiated by the sending side, which sends streamMapTableAddIndication function to inform about new (or modified) entries in the list, and streamMapTableRemoveIndication to inform about removal of entries from the list.

## Decoder Ready Signaling

DMIF permits the establishments of channels before the associated media stream are actually required.  This enables setting up the infrastructure of the whole session before any media stream starts to flow, which would result in a better performance. For this purpose, DMIF needs a separate procedure for signaling that decoder is ready to accept data. It is proposed to perform this with a new function - decoderReadyCommand.  Syntax is proposed in section 8.

# Proposed H.245 Extensions

This section proposes extensions to ITU-T Recommendation H.245 Version 2 from June 1996.  In case a newer version already exists, the syntax described in this section should be corrected to prevent collision with the new version.

The function syntax described subsequently includes the full syntax of each extended structure.  The proposed extensions are marked with an underline.

RequestMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

masterSlaveDetermination	MasterSlaveDetermination,

terminalCapabilitySet	TerminalCapabilitySet,

openLogicalChannel	OpenLogicalChannel,

closeLogicalChannel	CloseLogicalChannel,

requestChannelClose	RequestChannelClose,

multiplexEntrySend	MultiplexEntrySend,

requestMultiplexEntry	RequestMultiplexEntry,

requestMode	RequestMode,

roundTripDelayRequest	RoundTripDelayRequest,

maintenanceLoopRequest	MaintenanceLoopRequest,

...,

communicationModeRequest	CommunicationModeRequest,

conferenceRequest	ConferenceRequest,

h223AnnexAReconfiguration	H223AnnexAReconfiguration,

elementaryStreamRequest	ElementaryStreamRequest

streamMapTableRequest	StreamMapTableRequest

}

ResponseMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

masterSlaveDeterminationAck	MasterSlaveDeterminationAck,

masterSlaveDeterminationReject	MasterSlaveDeterminationReject,

terminalCapabilitySetAck	TerminalCapabilitySetAck,

terminalCapabilitySetReject	TerminalCapabilitySetReject,

openLogicalChannelAck	OpenLogicalChannelAck,

openLogicalChannelReject	OpenLogicalChannelReject,

closeLogicalChannelAck	CloseLogicalChannelAck,

requestChannelCloseAck	RequestChannelCloseAck,

requestChannelCloseReject	RequestChannelCloseReject,

multiplexEntrySendAck	MultiplexEntrySendAck,

multiplexEntrySendReject	MultiplexEntrySendReject,

requestMultiplexEntryAck	RequestMultiplexEntryAck,

requestMultiplexEntryReject	RequestMultiplexEntryReject,

requestModeAck	RequestModeAck,

requestModeReject	RequestModeReject,

roundTripDelayResponse	RoundTripDelayResponse,

maintenanceLoopAck	MaintenanceLoopAck,

maintenanceLoopReject	MaintenanceLoopReject,

...,

communicationModeResponse	CommunicationModeResponse,

conferenceResponse	ConferenceResponse,

h223AnnexAReconfigurationAck	H223AnnexAReconfigurationAck,

h223AnnexAReconfigurationReject	H223AnnexAReconfigurationReject,

elementaryStreamResponse	ElementaryStreamResponse,

streamMapTableResponse	StreamMapTableResponse,

streamMapTableReject	StreamMapTableReject

}

CommandMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

maintenanceLoopOffCommand	MaintenanceLoopOffCommand,

sendTerminalCapabilitySet	SendTerminalCapabilitySet,

encryptionCommand	EncryptionCommand,

flowControlCommand	FlowControlCommand,

endSessionCommand	EndSessionCommand,

miscellaneousCommand	MiscellaneousCommand,

...,

communicationModeCommand	CommunicationModeCommand,

conferenceCommand	ConferenceCommand,

decoderReadyCommand	StreamAssociationTagList

}

IndicationMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

functionNotUnderstood	FunctionNotUnderstood,

masterSlaveDeterminationRelease	MasterSlaveDeterminationRelease,

terminalCapabilitySetRelease	TerminalCapabilitySetRelease,

openLogicalChannelConfirm	OpenLogicalChannelConfirm,

requestChannelCloseRelease	RequestChannelCloseRelease,

multiplexEntrySendRelease	MultiplexEntrySendRelease,

requestMultiplexEntryRelease	RequestMultiplexEntryRelease,

requestModeRelease	RequestModeRelease,

miscellaneousIndication	MiscellaneousIndication,

jitterIndication	JitterIndication,

h223SkewIndication	H223SkewIndication,

newATMVCIndication	NewATMVCIndication,

userInput	UserInputIndication,

...,

h2250MaximumSkewIndication	H2250MaximumSkewIndication,

mcLocationIndication	MCLocationIndication,

conferenceIndication	ConferenceIndication,

vendorIdentification	VendorIdentification,

functionNotSupported	FunctionNotSupported,

streamMapTableAddIndication	StreamMapTable,

streamMapTableRemoveIndication	StreamAssociationTagList

}

Capability		::=CHOICE

{

nonStandard	NonStandardParameter,

receiveVideoCapability	VideoCapability,

transmitVideoCapability	VideoCapability,

receiveAndTransmitVideoCapability	VideoCapability,

receiveAudioCapability	AudioCapability,

transmitAudioCapability	AudioCapability,

receiveAndTransmitAudioCapability	AudioCapability,

receiveDataApplicationCapability	DataApplicationCapability,

transmitDataApplicationCapability	DataApplicationCapability,

receiveAndTransmitDataApplicationCapability	DataApplicationCapability,

h233EncryptionTransmitCapability	BOOLEAN,

h233EncryptionReceiveCapability	SEQUENCE

{

h233IVResponseTime	INTEGER (0..255),	-- units milliseconds

...

},

...,

conferenceCapability	ConferenceCapability,

receiveMPEG4Capability	MPEG4Capability,

transmitMPEG4Capability	MPEG4Capability,

receiveAndTransmitMPEG4Capability	MPEG4Capability

}

MPEG4Capability	::=SEQUENCE

{

profile		INTEGER(0..127),

fromProfileLevel	INTEGER(0..127),

toProfileLevel	INTEGER(0..127),

dmifCapability	DMIFCapability OPTIONAL,

-- indicates the capability to use a DMIF signaling channel

…

}

DMIFCapability

{

…

}

DataType		::=CHOICE

{

nonStandard	NonStandardParameter,

nullData	NULL,

videoData	VideoCapability,

audioData	AudioCapability,

data		DataApplicationCapability,

encryptionData	EncryptionMode,

...,

mpeg4Data	MPEG4Data,

mpeg4ApplicationSignaling	NULL,

dmifSignaling	NULL

}

MPEG4Data	::=CHOICE

{

singleStream	SEQUENCE

{

streamAssociationTag	StreamAssociationTag,

uuData	UUData OPTIONAL

},

flexMuxedStreams	FlexMuxTable,

…

}

UUData			::=CHOICE

{

esID		ES_id,

…

}

FlexMuxTable	::=SET SIZE(1..256) OF FlexMuxTableEntry

FlexMuxTableEntry	::=SEQUECE

{

streamAssociationTag	StreamAssociationTag

flexChannelNumber	INTEGER(0..255),

uuData	UUData OPTIONAL

}

StreamAssociationTag	::=INTEGER(0..65535) 	   --[Editor: Is this enough?]

ES_id			::=INTEGER(0..65535) 	   --[Editor: Is this enough?]

CommunicationModeTableEntry	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(1..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

terminalLabel	TerminalLabel OPTIONAL, -- if not present,

-- it refers to all participants

-- in the conference

sessionDescription	BMPString (SIZE(1..128)), -- Basic ISO/IEC 10646-1 (Unicode)

dataType	CHOICE

{

videoData	VideoCapability,

audioData	AudioCapability,

data	DataApplicationCapability,

...

mpeg4Data	MPEG4Data,

},

mediaChannel	TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,  -- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

...

}

ElementaryStreamRequest	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

elementaryStreams	SET SIZE(1..65535) OF ES_id,

sendNow	BOOLEAN,

-- if TRUE decoderReadyCommand is not needed

…

}

ElementaryStreamResponse	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

rejectedElementaryStreams	SET SIZE(0..65535) OF ElementaryStreamReject

}

ElementaryStreamReject	::=SEQUENCE

{

esID		ES_id,

reason		CHOICE

{

notAvailable	NULL,

inadequateQoS	NULL,

notAccessible	NULL,

protected,	NULL,

…

}

}

StreamMapTableRequest	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

elementaryStreams	SET SIZE(1..65535) OF ES_id OPTIONAL,

--if used only ES in the list are required

…

}

StreamMapTableResponse	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

streamMapTable	StreamMapTable

}

StreamMapTable	::=SET SIZE(1..65535) OF StreamMapTableEntry

StreamMapTableEntry	::=SEQUENCE

{

streamAssociationTag	StreamAssociationTag,

esID		ES_id

}

StreamMapTableReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

reason		CHOICE

{

noneIsKnown	NULL,

unavailable	NULL,

notAccessible	NULL

…

}

StreamAssociationTagList	::=SET SIZE(1..65535) OF StreamAssociationTag

# Broadcast

Broadcast services use only multicast channels.  The lack of back channels does not allow any kind of mutual signaling.  This also means that all the channels are unreliable.

The case of broadcast service over LAN has not been fully studied yet.  However, the following principles may be proposed:

- Media streams will use RTP/RTCP as in the case of multicast services.

- BIFS streams will be transmitted as a carousel, also over RTP.  RTCP is not required.

- A channel whose address is retrieved by a way which is out of the scope of this Proposal will be used for transmission of information about other channels. Exact syntax of this information is TBD. It should, however, be similar to the H.245 CommunicationModeTable structure.  This also will be transmitted as a carousel.  The use of RTP is not required here.  Bare UDP packets will be enough for this purpose.