PKTDLL v1.2

~~~~~~~~~~~

PKTDLL provides a Packet Driver interface over Digital Equipment Corp.'s

Datalink (DLL) specification drivers.  This is a small TSR shim which

translates application Packet Driver calls into appropriate DLL calls.

PKTDLL should work over any of the current v4.1 DLL and earlier drivers

(DLLDEPCA, DLLNDIS, etc.), making it a simple matter to add Packet Driver

applications to an existing PATHWORKS network environment (v5.0 should

most likely work as well - except for Novell 802.3 support - see below).

Simply add a line with "PKTDLL" into your AUTOEXEC.BAT after the commands

to start the network.  If you feel really adventurous you could even

"LOADHIGH PKTDLL" (it works fine!).

PKTDLL was renamed from DLLPKT to be consistent with the naming conventions

used within Digital Equipment Corporation and the left to right reading

habits of the western world.  If you feel stubborn about this issue, DOS

does have a file renaming command.

I have not included support for any advanced functionality such as

multicast and promiscuous mode.  If there are significant grumblings, I

will re-examine these issues, however, I really don't see the need.  This

extra functionality is better served by existing utilities which deal

directly with the DLL.

There is an assumption that the DECnet flavour of PATHWORKS is being used.

Having two different TCP/IP stacks simultaneously on a single machine would

be somewhat strange (perhaps it would be ok with multiple network cards).

Also, PKTDLL is currently setup as a Class 1 Packet Driver (ethernet).  I

will soon be examining the changes required to support Token Ring.  I have

also made this a so called high performance Packer Driver, but there has

been a minimum of testing with the high performance functions.  This may

sound somewhat tentative, however, I have been using the driver regularly

for quite some time and have found no problems.

If you have the desire to modify this code or find items needing repair,

please send me your requests and/or updates.  I am closer to DEC's

engineering group and should be able to ensure that PKTDLL functions

according to the DLL specification, hopefully keeping so called quality at

a high level.  Please also forward any other comments to me.  I am quite

interested in how people may be using this driver.

As with any other packet driver, to use most network applications within

MS Windows, one must employ the services of WINPKT or PKTMUX.

The following packages have been successfully tested with PKTDLL:

WATTCP                          NCSA Telnet

KA9Q                            CUTCP

Kermit                          QVTNET

Trumpet - News Reader           PDIPX from Intel

Trumpet - TCPDRV                IPX from BYU

PKTMUX                          WINPKT

New in PKTDLL 1.2:

~~~~~~~~~~~~~~~~~~

1)      Specifying a protocol type of "0" as the first protocol type on

the command line will stop any protocols from being held open for

Windows.  This should not be necessary in most circumstances, as

the protocols are always available to DOS applications.  There may

be a very tiny performance gain.

2)      Specifying "/s" on the command line will stop all upcalls from

PKTDLL (similar to "TERMIN/S").  This combined with a MARK/RELEASE

type program will safely allow the unloading of a normally non-

unloadable packet driver based program (eg. IPX).  This can and

will ungracefully mess up any running packet driver applications

(ie. not recommended except for very specific uses).

3)      The command line parsing is somewhat more freeform, though the

switches must still be at the end of the line.  The order of the

numbers is less important and the vector does not have to be

specified in order to change the protocol types anymore.

4)      A problem with Novell 802.3 support over v4.1 DLLDEPCA has been

fixed.  Novell 802.3 packets have now been successfully tested

over DLLDEPCA and DLLNDIS (still no progress on v5.0 DLL support).

Thanks for feedback to David Denholm.

Usage:                  PKTDLL [packet_int] [proto_type...] [/n] [/s] [/u]

~~~~~~

packet_int      -       This is the Packet Driver interrupt to use.  The

default interrupt is 61 HEX.  The number will be

interpreted as hexadecimal whether or not it is

prefixed by "0x".  This same rule applies to the

protocol type following.

range: 60 - 7F

proto_type      -       This is a list of protocol types to hold open for

Windows access.  The DLL must have any protocols

which it will use in Windows opened prior to

entering Windows.  The protocols in this list will

be opened and held in limbo until accessed by an

application.  There is no such restriction on Packet

Driver applications outside of Windows.

The default is to open protocols 0800, 0806, and

8035 (IP, ARP, and RARP).  This list may replaced

by a list of your own choosing.  Be aware that the

DLL only supports 8 portals, several of which are

often used by PATHWORKS.  MOP and LoopBack are

temporarily disabled when PKTDLL is loaded but are

restored on exiting PKTDLL.  A value of 0 will

disable this feature.

range: 0 or 05EF - FFFE

/n              -       "/n" will select 802.3 framing for Novell's IPX.

Most public domain IPX stacks use Novell's 8137

DIX protocol type.  This will convert these packets

to 802.3 on the wire.  This functionality will not

work with the new PATHWORKS V5.x DLL drivers being

developed - ie. only DIX will be supported (the API

is changing and I am feeling very lazy).

/s              -       "/s" will stop all upcalls from PKTDLL, allowing

a MARK/RELEASE type program to safely unload a

non-unloadable program like IPX.  PKTDLL remains

in memory and available for use.

/u              -       "/u" will unload the driver releasing both its

memory and it reserved protocols, provided there

are none in use.  If an alternate interrupt has

been chosen, it must be specified when unloading

the driver.

Examples:               PKTDLL

Loads the Packet Driver at interrupt 61 with protocols

IP, ARP and RARP active for Windows.

PKTDLL 61 7301 7302 /n

WINPKT 0x60 0x61

Loads the Packet Driver at Interrupt 61 with protocols

7301 and 7302 (just made them up!) active for Windows.

Also, protocol 8137 will be converted to 802.3.

The WINPKT line is necessary to run most packet driver

applications within Windows.  If you feel adventurous

try PKTMUX v1.2c (it actually works).

PKTDLL 60 /u

Unloads the Packer Driver loaded at interrupt 60.

Have Fun,

Brian Angus

angus@trcoa.enet.dec.com

(416) 597-3268