������������������������������������������������������������������������������ͻ

�                      CrossFire Assembler, By Mark Parry                      �

�                                  80x86.DOC                                   �

������������������������������������������������������������������������������ͼ

This document explains the Intel 80x86 processors/instruction set from the

perspective of a Motorola 680x0 programmer. It is not a intended as a complete

guide to programming the 80x86 range of processors. You should also look at the

document "FILETYPE.DOC" to see how various output file formats are supported,

especially for types ".COM", ".EXE", ".SYS" and ".EXL".

������������������������������������������������������������������������������Ŀ

�                                 Introduction                                 �

��������������������������������������������������������������������������������

The reason for CrossFire? Originally, I wanted to write a 680x0 assembler on the

PC, to take advantage of the much faster processor speed. I could of probably

have coped with the annoying quirks of PC assemblers, but I could just not get

my head around the opposite syntax of the instructions. For this reason, I had

to write an 80x86 assembler that I could work with. CrossFire's version of 80x86

is as close to 680x0 as I could make it.

Example: store value (41 hex) in memory on different assemblers

MOV	BYTE PTR [BX],41h	TASM/MASM/etc.

MOV.B	[BX],41h		XFire 80x86 (normal syntax)

MOVE.B	[BX],#$41		XFire 80x86 (normal syntax)

MOVE.B	#$41,[BX]		XFire 80x86 (reverse syntax)

MOVE.B	#$41,(A0)		XFire 680x0

Being used to programming the 68000, it was a major culture shock switching to

the 80x86. The assemblers do not help, they seem to be lumbered with a lot of

unnecessary and awkward features.

I don't like having to keep modifying segment registers. Earlier versions of the

assembler had to, and it was a nightmare. To try to make life easier for myself,

I now program the 80x86 in one of two ways;

if all the code, data and buffers will fit in 64k, I can write the

program in a 16-bit segment. ('.COM')

if code, data and buffers will not fit in 64k, or I need to use expanded

memory, I will use a DOS extender and write the program in a 32-bit

segment. I now tend to write all programs using this method, as I have a

library of routines that handle memory, input, output and files. The DOS

extender seems very stable, and works under DPMI, VCPI and XMS. ('.EXE')

A 32-bit segment works similar to the 68000. With reverse syntax and the

equivalent instructions (the branches, set conditionally, shifts, CLR, TST,

SWAP, etc.) 80386 code can look remarkably similar to 68000. I find 68000

branches, set conditionally and shifts more logical, and CLR, TST and SWAP make

source code more readable. Using MOVE/MULU/MULS/DIVU/DIVS/ADDX/SUBX/JSR/EOR/EXG

instead of MOV/MUL/IMUL/DIV/IDIV/ADC/SBB/CALL/XOR/XCHG are just a matter of

preference.

BTST, BCHG, BCLR and BSET (only with an immediate bit number) have been included

from before 80386- instructions were added. However, I still only use BT, BTC,

BTR and BTS if I use a bit field or a register bit number. xBCHG, xBCLR and

xBSET are included because they take less than half the bytes of BCHG, BCLR and

BSET (but they do not set the flags properly). xBTST is included for parity.

Use of the extra instructions is optional, all of the 80x86 instructions can

still be used (I like having the choice). However, BSR (bit scan reverse) and

LSL (load segment limit) have been renamed (@BSR and @LSL), because of the 680x0

instructions BSR (branch to subroutine) and LSL (logical shift left).  BTTST

(BT), BTCLR (BTR), BTCHG (BTC), BTSET (BTS), BTSF (BSF) and BTSR (@BSR) were

added because I prefer the syntax.

I tried to make the assembler as flexible as possible. There are a lot of

duplicate instructions, and the same address mode can be specified in a number

of ways. There are a lot of ways to do the same thing, for example :-

LODSD = LODS.D = LODSL = LODS.L

[BX] = (BX)

CS:[BX] = CS:(BX) = [CS:BX] = (CS:BX)

offset[address] = [offset,address] = [offset+address]

$12 = 12h = 12xh

$ABCD = 0ABCDh = 0ABCDxh

%100 = 100b = 100xb

Although the assembler will recognize round brackets on 80x86 address modes,

(BX) is considered the same as [BX] for example, you can only use square

brackets for direct memory addressing; [100] is allowed, (100) is not. It is

therefore recommended that you use only square brackets for 80x86 address modes.

������������������������������������������������������������������������������Ŀ

�                 Differences Between CrossFire and MASM/TASM                  �

��������������������������������������������������������������������������������

For the purposes of this document, MASM and TASM are considered the same.

CrossFire does not LINK.

No need to define segments.

No need to define procedures as near or far.

BYTE PTR, WORD PTR, etc. are not used (.B/.W are).

Immediate numbers need a hash (#) before them, for example :-

ADD	AL,4			(MASM/TASM)

ADD	AL,#4			(XFire, normal 80x86 syntax)

Label offsets are handled differently :-

MOV	AX,OFFSET LABEL		(MASM/TASM)

MOV	AX,#LABEL		(XFire, normal 80x86 syntax)

The way you define a label on MASM/TASM affects the way it is used, example:-

(MASM/TASM)

LABEL	DB	0

MOV	LABEL,1			move byte to LABEL

LABEL	DW	0

MOV	LABEL,1			move word to LABEL

MOV	BYTE PTR LABEL,1	override LABEL type, move byte

MOV	WORD PTR LABEL,1	override LABEL type, move word

(XFire, normal 80x86 syntax)

MOV.B	[LABEL],#1		move byte to LABEL

MOV.W	[LABEL],#1		move word to LABEL

CrossFire reports errors and warnings using the same method as TASM. Additional

REPORTs can be made for the following conditions :-

report PSEUDO-BRANCH	pseudo branch instructions

report PSEUDO-BIT	pseudo BIT instructions (BTST/BCHG/BCLR/BSET...

...xBTST/xBCHG/xBCLR/xBSET)

report PSEUDO-xBIT	pseudo xBIT instructions

(xBTST/xBCHG/xBCLR/xBSET)

report PSEUDO-MOVE	pseudo MOVE instructions

report PSEUDO-STACK	pseudo instructions that use the stack

report PSEUDO-ALL	all pseudo instructions

report OVERRIDE		segment override used (CS: DS: ES: FS: GS: SS:)

report SEGMENT		segment register used (CS DS ES FS GS SS)

report STACK		instruction uses / might use the stack

report STACK?		instruction might use the stack

report 16/32		instruction works differently in

16/32-bit segments

report PREFIX-66	operation size switch ($66) used

report PREFIX-67	address mode switch ($67) used

report STRING		string operations (CMPS LODS MOVS...

...SCAS STOS INS OUTS)

report BIT		bit operations (BT BTC BTR BTS)

report SS:		address from stack segment (SS:)

report 80x86		all 80x86 reports

for example, to report instructions that will/can use the stack:-

report	stack

MOV	#12,SP		; no report (XFire does not considered this

;			to be using the stack)

.CALL	CALL	.CALL		; will report 'instruction uses the stack'

IDIV	BX		; will report 'instruction might use the stack'

to report only instructions that will use the stack:-

report	stack,stack?-

MOV	#12,SP		; no report (XFire does not considered this

;			to be using the stack)

.CALL	CALL	.CALL		; will report 'instruction uses the stack'

IDIV	BX		; will report nothing

������������������������������������������������������������������������������Ŀ

�                              Processor Support                               �

��������������������������������������������������������������������������������

The following instructions are provided for Intel processor support :-

16BIT	�

32BIT	� set segment size

8086		8086n		8086r		�

286		286n		286r		� n=normal syntax

386		386n		386r		� r=reverse syntax

486		486n		486r    	�

However, the preferred method is to use the OPT (or OPTION) command, example :-

OPTION	16-BIT	USE16	�

OPTION	32-BIT	USE32	� set segment size

OPT	8086	8086n	8086r 	86	86n	86r	�

OPT	8088	8088n	8088r 	88	88n	88r	�

OPT	80186	80186n	80186r	186	186n	186r	� set

OPT	80188	80188n	80188r	188	188n	188r	� processor

OPT	80286	80286n	80286r	286	286n	286r	� type

OPT	80386	80386n	80386r	386	386n	386r	�

OPT	80486	80486n	80486r	486	486n	486r	�

OPT	80586	80586n	80586r	586	586n	586r	�

OPT	.COM	�

OPT	.EXE	� set output

OPT	.SYS	� program type

OPT	.EXL	�

Multiple options can be passed on a single line, for example :-

OPTION	86r,16-bit,.EXE

All processor support OPTIONs can also be passed as command line switches, for

example :-

XFIRE source.ext dest.ext /86r /16-bit /.exe

New processor support may only be provided by OPT (or OPTION) command and the

command line in the future.

As far as CrossFire is concerned, 80186 instructions are identical to 8086. I

have seen some documentation that say some extra instructions (e.g. PUSH #imm)

were introduced on the 80188, but most reckon that they were introduced on the

80286, and this seems to be how TASM handles them. Also 8088 and 80188

processors are considered to be the same as 8086 and 80186.

Trying to assemble instructions that require a better processor only generate a

warning, for example :-

OPT	8086

CLR.L	eAX

will assemble the instruction, but gives the warning '80386 processor'.

������������������������������������������������������������������������������Ŀ

�                                Document Guide                                �

��������������������������������������������������������������������������������

CrossFire accepts the following number forms :-

hexadecimal		$1234		1234h		1234xh		0x1234

decimal			1234		1234d		1234xd

binary			%1010		1010b		1010xb

character constants	'xyz'		"xyz"		�xyz�

reverse binary		%%1010 = 5	%%0001 = 8

k kilo (*1024)		10k = 10240	$10k = 16384

M meg (*1048576)	10M = 10485760

Examples in this file use $1234 (hexadecimal), 1234 (decimal), %1010 (binary).

Symbol Definitions

{286-}		80286 or better expected.

{386-}		80386 or better expected.

{486-}		80486 or better expected.

{586-}		80586 (pentium) or better expected.

{SS:}		memory address uses the stack segment (SS:) by default.

{stack}		instruction uses the stack.

{stack?}	instruction may use the stack.

{pm}		protected mode instruction.

{priv}		privileged instruction.

{prefix}	this instruction is a prefix for the next instruction.

{680x0}		equivalent syntax for same/similar instruction on the 680x0.

{!!!}		an 80x86/680x0 hybrid instruction.

<>		not equal

<		less than

>		greater than

@ins		instruction clashes with 680x0 instruction, so is renamed.

[mem]		memory address, see addressing modes.

680x0:		information significant to 680x0 programmers.

Also:		related instructions.

All examples and instruction forms are in reverse (680x0) format. 80x86 refers

to longs (L) as double words (D). Both forms are acceptable to the assembler.

The instructions are listed in alphabetical order. However, conditional branches

(Jc, Bcc) and set conditionally (SETcc, Scc) have their own sections, with a

minimal entry in the main list.

Pseudo instructions are not actually a single processor instruction, but are

assembled as multiple instructions. For example, MOVE.W	[BX],[DI] is assembled

as PUSH.W [BX], POP.W [DI]

{680x0} and {!!!} instructions are not 80x86 instructions, but are provided for

convenience (EXG, MOVE, CLR, TST, the branches, for example). All are also

listed separately after the 80x86 instructions set.

680x0:	The flags are unchanged or only partly changed on certain instructions:-

NOT, MOVE(=MOV); no flags set.

DIVU(=DIV), DIVS(=IDIV); all flags undefined.

MULU(=MUL), MULS(=IMUL); only overflow (OF) and carry (CF) are defined.

ROL, ROR, ROXL(=RCL), ROXR(=RCR); only set overflow(OF) and carry(CF).

INC, DEC; the carry(CF) is not changed.

The shift and rotate instructions (SAL/ASL, SAR/ASR, SHL/LSL, SHR/LSR, ROL, ROR,

RCL/ROXL, RCR/ROXR) effects on flags were reported differently depending on the

reference material, so I tested the effects for all cases and used the results

in this file. These tests were performed on an Intel Pentium processor, and may

not be the same on other processors (but they SHOULD be).

������������������������������������������������������������������������������Ŀ

�                               80x86 Registers                                �

��������������������������������������������������������������������������������

The Data Registers

bit	31              15      7      0

<------------ eAX ------------->  eAX bits 0-31			{386-}

<----- AX ----->   AX bits 0-15

<--AH--><--AL-->   AH bits 8-15  AL bits 0-7

<------------ eBX ------------->  eBX bits 0-31			{386-}

<----- BX ----->   BX bits 0-15

<--BH--><--BL-->   BH bits 8-15  BL bits 0-7

<------------ eCX ------------->  eCX bits 0-31			{386-}

<----- CX ----->   CX bits 0-15

<--CH--><--CL-->   CH bits 8-15  CL bits 0-7

<------------ eDX ------------->  eDX bits 0-31			{386-}

<----- DX ----->   DX bits 0-15

<--DH--><--DL-->   DH bits 8-15  DL bits 0-7

<------------ eBP ------------->  eBP bits 0-31			{386-}

<----- BP ----->   BP bits 0-15

<------------ eSP ------------->  eSP bits 0-31			{386-}

<----- SP ----->   SP bits 0-15

<------------ eSI ------------->  eSI bits 0-31			{386-}

<----- SI ----->   SI bits 0-15

<------------ eDI ------------->  eDI bits 0-31			{386-}

<----- DI ----->   DI bits 0-15

(e)AX	accumulator

(e)BX	base register

(e)CX	count register

(e)DX	data register

(e)BP	base pointer (from stack)

(e)SP	stack pointer

(e)SI	source index

(e)DI	destination index

The Segment Registers

Segment registers are 16-bits (word). However, PUSHing/POPing a segment register

on/off the stack in a 32-bit segment uses 32-bits (long) by default, and segment

registers can be moved to/from 32-bit registers (the upper 16-bits are 0).

CS:	code segment

DS:	data segment

ES:	extra segment

SS:	stack segment

FS:	another data segment 						{386-}

GS:	another data segment						{386-}

System Registers

All system registers are 32-bits (longs).

CR0, CR2, CR3			control registers			{386-}

TR3, TR4, TR6, TR7		test registers				{386-}

DR0, DR1, DR2, DR3, DR6, DR7	debug registers				{386-}

There is more information on debug registers at the end of the file.

The Program Counter (Instruction Pointer, (e)IP)

<------------ eIP ------------->  eIP bits 0-31			{386-}

<----- IP ----->   IP bits 0-15

The (e)FLAGS Register

<----------- eFLAGS ----------->  eFLAGS bits 0-31		  {386-}

<---- FLAGS --->   FLAGS bits 0-15

0	CF	carry flag

1

2	PF	parity flag

3

4	AF	auxiliary carry

5

6	ZF	zero flag

7	SF	sign flag

8	TF	trap flag (single step trace)

9	IF	interrupt flag

10	DF	direction flag

11	OF	overflow flag

12	IOPL    �							{286-}

13	IOPL	� input/output privilege level				{286-}

14	NT	nested task flag					{286-}

15

16	RF	resume flag						{386-}

17	VM	virtual 8086 mode flag (virtual mode)			{386-}

Input/output privilege level (IOPL) is used for protection in a multitasking

environment. IOPL level 0 is system level operation, IOPL level 3 is the most

protected. The level controls the programs ability to perform things such as

IN and OUT, setting the interrupt flag and modifying the IOPL by changing the

(e)FLAGS register.

The nested test flag (NT) signifies if the current task was called by another.

It is set by the system and cannot be modified by the program.

The resume flag (RF) is used by on-line debuggers to resume operation after a

debug exception.

������������������������������������������������������������������������������Ŀ

�                                Address Modes                                 �

��������������������������������������������������������������������������������

16-bit Address Modes

[BX]		[BX,SI]

[BP] {SS:}      [BX,DI]

[SI]            [BP,SI] {SS:}

[DI]            [BP,DI] {SS:}		[address]

An 8/16-bit sign extended offset can optionally be used. To use an 8 bit offset,

use 'offset.b'. [address] is a 16-bit memory address. The default segment is

DS:, or SS: if BP is used. Examples :-

[BX]						from DS:[BX]

CS:[BX]						from CS:[BX]

12.b[BX]					from DS:[BX+12]

[BX,BP]		[BX+BP]				from SS:[BX+BP]

12.b[BX,BP]	12.b[BX+BP]	[BX,BP,12.b]	from SS:[BX+BP+12]

CS:12[BX,BP]	CS:[BX,BP,12]	[CS:BX,BP,12]	from CS:[BX+BP+12]

12[BX,12]	[BX,12,12]			from DS:[BX+24]

12[12]						from DS:[24]

ES:[12]						from ES:[12]

32-bit Address Modes			{386-}

32-bit address modes are not as limited, as any data register can be used.

Base register	eAX, eBX, eCX, eDX, eSI, eDI, eBP {SS:}, eSP {SS:}

Index register	eAX, eBX, eCX, eDX, eSI, eDI, eBP {SS:}

The index register can be scaled (*2, *4 or *8)

An 8/32-bit sign extended offset can optionally be used (although some

address mode combinations cannot use an 8 bit offset). To use an 8 bit offset,

use 'offset.b'. [address] is a 32-bit memory address. The default segment is

DS:; or SS: if eSP or eBP are used. Examples :-

[eAX]						from DS:[eAX]

[eAX*2]		[eAX,eAX]			from DS:[eAX*2]

[eAX*4]						from DS:[eAX*4]

CS:12[eAX]	12[CS:eAX]	[CS:eAX,12]	from CS:[eAX+12]

12[eAX,eBX*4]	[12,eAX,eBX*4]	[eBX*4,eAX,12]	from DS:[(eAX*4)+eBX+12]

12[eBP]		[eBP,12]			from SS:[eBP+12]

12[eSP]		[eSP,12]			from SS:[eSP+12]

[12]						from DS:[12]

Segment overrides (CS:, DS:, ES:, FS:, GS:, SS:) can be used on all address

modes (but they are ignored for LEA).

������������������������������������������������������������������������������ͻ

�                            80x86 Instruction Set                             �

������������������������������������������������������������������������������ͼ

AAA (ASCII adjust after addition)

Syntax:	AAA

Flags:	auxiliary carry (AF), carry (CF);

overflow (OF), sign (SF), zero (ZF), parity (PF) are undefined.

Adjust unpacked BCD in AL after addition.

--------------------------------------------------------------------------------

AAD (ASCII adjust before division)

Syntax:	AAD

Flags:	sign (SF), zero (ZF), parity (PF);

overflow (OF), auxiliary carry (AF), carry (CF) are undefined.

Adjust unpacked BCD in AX before division.

--------------------------------------------------------------------------------

AAM (ASCII adjust after multiplication)

Syntax:	AAM

Flags:	sign (SF), zero (ZF), parity (PF);

overflow (OF), auxiliary carry (AF), carry (CF) are undefined.

Adjust packed BCD in AL into AX after multiplication.

--------------------------------------------------------------------------------

AAS (ASCII adjust after subtraction)

Syntax:	AAS

Flags:	auxiliary carry (AF), carry (CF);

overflow (OF), sign (SF), zero (ZF), parity (PF) are undefined.

Adjust unpacked BCD in AL after subtraction.

--------------------------------------------------------------------------------

ADC (add with carry)

ADDX (add extended) {680x0}

Syntax:	ADC destination,source		Reverse: ADC source,destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

Add the source and the carry flag (CF) to the destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

ADD (addition)

Syntax:	ADD destination,source		Reverse: ADD source,destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

Add the source to the destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

ADDR: (address mode switch) {!!!} {386-} {prefix}

Syntax:	ADDR:

Flags:	Unaffected

Switch the address mode (between 16/32-bit) for the next instruction.

--------------------------------------------------------------------------------

AND (logical and)

Syntax:	AND destination,source		Reverse: AND source,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Logically AND source with destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

ARPL (adjust requested privileged level) {286-} {pm}

Syntax:	ARPL destination,source		Reverse: ARPL source,destination

Flags:	zero (ZF).

If RPL (requested privilege level) of destination < RPL of source, set

the zero flag (ZF) and copy source RPL to destination RPL; Otherwise,

clear the zero flag (ZF). The destination is either a word register or a

memory address, containing a segment selector. The RPL is contained in

bits 0-1 of the segment selector.

Instruction forms:		byte		word		long

ARPL to register				AX,BX

ARPL to memory					AX,[mem]

--------------------------------------------------------------------------------

BCHG (bit test and change) {680x0} {stack}

Syntax:	BCHG destination,#bit		Reverse: BCHG #bit,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Test #bit of the destination, then change it. This is a pseudo

operation.

Instruction forms:		byte		word		long

change register bit		#bit,AL		#bit,AX		#bit,eAX

=	TEST #xxx,AL	TEST #xxx,AX	TEST #xxx,eAX

PUSHF		PUSHF		PUSHF

XOR #xxx,AL	XOR #xxx,AX	XOR #xxx,eAX

POPF		POPF		POPF

change memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	TEST #xxx,[mem]	TEST #xxx,[mem]	TEST #xxx,[mem]

PUSHF		PUSHF		PUSHF

XOR #xxx,[mem]	XOR #xxx,[mem]	XOR #xxx,[mem]

POPF		POPF		POPF

where #xxx is a bit mask, created from #bit.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	xBCHG can be used to change a bit, without testing the bit first.

Also:	BTC, xBCHG

--------------------------------------------------------------------------------

BCLR (bit test and clear) {680x0} {stack}

Syntax:	BCLR destination,#bit		Reverse: BCLR #bit,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Test #bit of the destination, then clear it. This is a pseudo operation.

Instruction forms:		byte		word		long

clear register bit		#bit,AL		#bit,AX		#bit,eAX

=	TEST #xxx,AL	TEST #xxx,AX	TEST #xxx,eAX

PUSHF		PUSHF		PUSHF

AND #yyy,AL	AND #yyy,AX	AND #yyy,eAX

POPF		POPF		POPF

clear memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	TEST #xxx,[mem]	TEST #xxx,[mem]	TEST #xxx,[mem]

PUSHF		PUSHF		PUSHF

AND #yyy,[mem]	AND #yyy,[mem]	AND #yyy,[mem]

POPF		POPF		POPF

where #xxx is a bit mask, created from #bit,

#yyy is the inverse or #xxx.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	xBCLR can be used to clear a bit, without testing the bit first.

Also:	BTR, xBCLR

--------------------------------------------------------------------------------

BOUND (array index bound check) {286-} {stack?}

Syntax:	BOUND register,[mem]		Reverse: BOUND [mem],reg

Flags:	Unaffected

Check the register against upper and lower boundary, and forces

interrupt 05 if out of range.

16-bit (word) register :- If the register is < the word at [mem] or >

the word at [mem+2], force interrupt 05.

32-bit (long) register :- If the register is < the long at [mem] or >

the long at [mem+4], force interrupt 05.

Instruction form:		byte		word		long

BOUND						[mem],AX	[mem],eAX

--------------------------------------------------------------------------------

BSET (bit test and set) {680x0} {stack}

Syntax:	BSET destination,#bit		Reverse: BSET #bit,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Test #bit of the destination, then set it. This is a pseudo operation.

Instruction forms:		byte		word		long

set register bit		#bit,AL		#bit,AX		#bit,eAX

=	TEST #xxx,AL	TEST #xxx,AX	TEST #xxx,eAX

PUSHF		PUSHF		PUSHF

OR #xxx,AL	OR #xxx,AX	OR #xxx,eAX

POPF		POPF		POPF

set memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	TEST #xxx,[mem]	TEST #xxx,[mem]	TEST #xxx,[mem]

PUSHF		PUSHF		PUSHF

OR #xxx,[mem]	OR #xxx,[mem]	OR #xxx,[mem]

POPF		POPF		POPF

where #xxx is a bit mask, created from #bit.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	xBSET can be used to change a bit, without testing the bit first.

Also:	BTS, xBSET

--------------------------------------------------------------------------------

BSF (bit scan forward) {386-}

BTSF (bit scan forward) {!!!} {386-}

Syntax:	BSF dest register,source	Reverse: BSF source,dest register

Flags:	zero (ZF).

Scan the source from right to left for the first non-zero bit. If the

source is zero, the zero flag (ZF) is set, and the destination register

is undefined. If the source is not zero, the zero flag (ZF) is cleared,

and the position of the first non-zero bit is stored in the destination

register.

Instruction forms:		byte		word		long

scan register					AX,BX		eAX,eBX

scan memory					[mem],BX	[mem],eBX

Also:	@BSR

--------------------------------------------------------------------------------

@BSR (bit scan reverse) {386-}

BTSR (bit scan reverse) {!!!} {386-}

Syntax:	@BSR dest register,source	Reverse: @BSR source,dest register

Flags:	zero (ZF).

Scan the source from left to right for the first non-zero bit. If the

source is zero, the zero flag (ZF) is set, and the destination register

is undefined. If the source is not zero, the zero flag (ZF) is cleared,

and the position of the first non-zero bit is stored in the destination

register.

Instruction forms:		byte		word		long

scan register					AX,BX		eAX,eBX

scan memory					[mem],BX	[mem],eBX

Also:	BSF

--------------------------------------------------------------------------------

BSWAP (byte swap) {486-}

Syntax:	BSWAP register

Flags:	Unaffected

Changes the byte order of the register.

Instruction form:		byte		word		long

BSWAP						AX		eAX

Also:	REV,REVERSE

--------------------------------------------------------------------------------

BT (bit test) {386-}

BTTST (bit test) {!!!} {386-}

Syntax:	BT destination,bit		Reverse: BT bit,destination

Flags:	carry (CF).

Copy the defined bit of the destination to the carry flag (CF).

Instruction forms:		byte		word		long

immediate bit of register			#bit,AX		#bit,eAX

immediate bit of memory {1}			#bit,[mem]	#bit,[mem]

register bit of register			BX,AX		eBX,eAX

register bit of memory {1,2}			BX,[mem]	eBX,[mem]

{1}	Size (.W or .L/.D) is needed. The default is byte (.W).

{2}	[mem] is actually a bit field. Word version can address from [mem-32k]

to [mem+(32k-1)], Long from [mem-2G] to [mem+(2G-1)]. Bit 0 is bit 0 of

[mem+0], bit 8 is bit 0 of [mem+1], bit -1 is bit 7 of [mem-1].

680x0:	The carry flag (CF) is used, NOT the zero flag (ZF).

--------------------------------------------------------------------------------

BTC (bit test with complement) {386-}

BTCHG (bit test with change) {!!!} {386-}

Syntax:	BTC destination,bit		Reverse: BTC bit,destination

Flags:	carry (CF).

Copy the defined bit of the destination to the carry flag (CF), then

complement (change) that bit of the destination.

Instruction forms:		byte		word		long

immediate bit of register			#bit,AX		#bit,eAX

immediate bit of memory {1}			#bit,[mem]	#bit,[mem]

register bit of register			BX,AX		eBX,eAX

register bit of memory {1} {2}			BX,[mem]	eBX,[mem]

{1}	Size (.W or .L/.D) is needed. The default is byte (.W).

{2}	[mem] is actually a bit field. Word version can address from [mem-32k]

to [mem+(32k-1)], Long from [mem-2G] to [mem+(2G-1)]. Bit 0 is bit 0 of

[mem+0], bit 8 is bit 0 of [mem+1], bit -1 is bit 7 of [mem-1].

680x0:	The carry flag (CF) is used, NOT the zero flag (ZF).

--------------------------------------------------------------------------------

BTR (bit test with reset) {386-}

BTCLR (bit test with clear) {!!!} {386-}

Syntax:	BTR destination,bit		Reverse: BTR bit,destination

Flags:	carry (CF).

Copy the defined bit of the destination to the carry flag (CF), then

reset (clear) that bit of the destination.

Instruction forms:		byte		word		long

immediate bit of register			#bit,AX		#bit,eAX

immediate bit of memory {1}			#bit,[mem]	#bit,[mem]

register bit of register			BX,AX		eBX,eAX

register bit of memory {1} {2}			BX,[mem]	eBX,[mem]

{1}	Size (.W or .L/.D) is needed. The default is byte (.W).

{2}	[mem] is actually a bit field. Word version can address from [mem-32k]

to [mem+(32k-1)], Long from [mem-2G] to [mem+(2G-1)]. Bit 0 is bit 0 of

[mem+0], bit 8 is bit 0 of [mem+1], bit -1 is bit 7 of [mem-1].

680x0:	The carry flag (CF) is used, NOT the zero flag (ZF).

--------------------------------------------------------------------------------

BTS (bit test with set) {386-}

BTSET (bit test with set) {!!!} {386-}

Syntax:	BTS destination,bit		Reverse: BTS bit,destination

Flags:	carry (CF).

Copy the defined bit of the destination to the carry flag (CF), then set

that bit of the destination.

Instruction forms:		byte		word		long

immediate bit of register			#bit,AX		#bit,eAX

immediate bit of memory {1}			#bit,[mem]	#bit,[mem]

register bit of register			BX,AX		eBX,eAX

register bit of memory {1} {2}			BX,[mem]	eBX,[mem]

{1}	Size (.W or .L/.D) is needed. The default is byte (.W).

{2}	[mem] is actually a bit field. Word version can address from [mem-32k]

to [mem+(32k-1)], Long from [mem-2G] to [mem+(2G-1)]. Bit 0 is bit 0 of

[mem+0], bit 8 is bit 0 of [mem+1], bit -1 is bit 7 of [mem-1].

680x0:	The carry flag (CF) is used, NOT the zero flag (ZF).

--------------------------------------------------------------------------------

BTST (bit test) {680x0}

Syntax:	BTST destination,#bit		Reverse: BTST #bit,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Test #bit of the destination. The destination is left unchanged. This is

a pseudo operation.

Instruction forms:		byte		word		long

test register bit		#bit,AL		#bit,AX		#bit,eAX

=	TEST #xxx,AL	TEST #xxx,AX	TEST #xxx,eAX

test memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	TEST #xxx,[mem]	TEST #xxx,[mem]	TEST #xxx,[mem]

where #xxx is a bit mask, created from #bit.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Also:	BT, xBTST

--------------------------------------------------------------------------------

CALL (procedure call) {stack}

JSR (jump to subroutine) {680x0} {stack}

Syntax:	CALL label

Flags:	Unaffected

PUSHes instruction pointer (e)IP (and code segment (CS) for far CALL)

onto the stack, and continue execution at 'label'.

Examples: BX = 100, value at [100] = 1234, at [102] = 5678. 16-bit segment.

CALL register {1}		CALL	BX		call routine at 100

CALL indirect address		CALL{2}	[BX]		�

CALL{2}	[100]		� call routine at 1234

branch to subroutine at label	CALL	label		(.W/.L/.D is optional)

CALL FAR address		CALL{2}	segment:address

CALL.F	segment:address

CALL FAR indirect address	CALL.F	[BX]		� call routine at 1234,

CALL.F	[100]		� in segment 5678.

{1}	In a 16-bit segment; if you use a 32-bit register, an exception will

occur if the register value is greater than a 16-bit value, that is:

CALL eBX	will cause an exception if eBX >= 64k.

In a 32-bit segment; if you use a 16-bit register, the address jumped to

will obviously by within the first 64k of the segment.

{2}	You can specify .W or .L/.D to force a switch to a 16 or 32-bit segment

instruction, but use with care.

Also:	RET, CALLF, RETF, BSR, RTS

--------------------------------------------------------------------------------

CALLF (call FAR procedure) {!!!} {stack}

JSRF (jump to FAR subroutine) {!!!} {stack}

Syntax:	CALLF label

Flags:	Unaffected

Call subroutine, use far address or far indirect memory address.

Examples: BX = 100, value at [100] = 1234, at [102] = 5678. 16-bit segment.

call FAR address		CALLF	segment:address

call FAR indirect address 	CALLF	[BX]		� call routine at 1234,

CALLF	[100]		� in segment 5678.

Notes:	You can specify .W or .L/.D to force a switch to a 16 or 32-bit segment

instruction, but use with care.

Also:	CALL, RETF

--------------------------------------------------------------------------------

CBW (convert byte to word)

Syntax:	CBW

Flags:	Unaffected

Extend sign bit of AL through AH.

Also:	MOVSX, CWD, CWDE, CDQ

--------------------------------------------------------------------------------

CDQ (convert double to quad) {386-}

Syntax:	CDQ

Flags:	Unaffected

Extend sign bit of eAX through eDX.

Also:	MOVSX, CBW, CWD, CWDE

--------------------------------------------------------------------------------

CLC (clear carry)

Syntax:	CLC

Flags:	carry (CF) = 0.

Clear the carry flag (CF).

--------------------------------------------------------------------------------

CLD (clear direction flag)

Syntax:	CLD

Flags:	direction (DF) = 0.

Clear the direction flag (direction = forwards, increment).

--------------------------------------------------------------------------------

CLI (clear interrupt flag)

Syntax:	CLI

Flags	interrupt flag (IF) = 0.

Clear interrupt flag, disabling interrupts.

--------------------------------------------------------------------------------

CLR (clear operand) {680x0}

Syntax:	CLR operand

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Clear the operand. This is a pseudo operation.

Instruction forms:		byte		word		long

CLR register (XOR reg,reg)	AL		AX		eAX

CLR memory (AND #0.b,[mem]) {1}	[mem]		[mem]		[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

CLTS (clear task switched flag) {286-} {priv}

Syntax:	CLTS

Flags:	Unaffected

Clears the task switched flag in the machine status word (MSW).

--------------------------------------------------------------------------------

CMC (complement the carry)

Syntax:	CMC

Flags:	carry (CF).

Complement (invert) the carry flag (CF).

--------------------------------------------------------------------------------

CMP (compare)

Syntax:	CMP destination,source		Reverse: CMP source,destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

Set status flags as if source were subtracted from destination. The

destination is not affected.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

CMPSB CMPS.B (compare string byte)

CMPSW CMPS.W (compare string word)

CMPSD CMPS.D CMPSL CMPS.L (compare string long) {386-}

Syntax:	Inherent

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

This is a string operation. Set the status flags as if the byte, word

or long at DS:[(e)SI] were subtracted from ES:[(e)DI]. Then, depending

on the direction flag (DF), increment or decrement (e)SI and (e)DI by

1, 2 or 4. The source segment can be overridden. SI, DI and CX (if

repeat) are used for a 16-bit segment. eSI, eDI and eCX (if repeat) are

used for a 32-bit segment.

REPZ or REPE can be used as a prefix to repeat the operation if the

source and destination are the same; REPNZ or REPNE can be used as a

prefix to repeat the operation if the source and destination are not the

same; both use (e)CX as the repeat counter.

Examples:

Remember, DS: is the source segment, ES: is the destination segment.

Assume the direction flag is forward (DF=0, CLD).

to find the first difference in 1000 bytes of data :-

16-bit segment			32-bit segment

MOVE.W	#source,SI  DS:[SI]	MOVE.L	#source,eSI   DS:[eSI]

MOVE.W	#dest,DI    ES:[DI]	MOVE.L	#dest,eDI     ES:[eDI]

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

REPZ				REPZ

CMPSB				CMPSB

BEQ.S	no differences		BEQ.S	no differences

(e)CX = remain count - 1, (e)SI and (e)DI = different byte + 1

; no differences

(e)CX = 0, (e)SI = source+1000 , (e)DI = dest+1000

to find the first match in 1000 bytes of data :-

MOVE.W	#source,SI  DS:[SI]	MOVE.L	#source,eSI	DS:[eSI]

MOVE.W	#dest,DI    ES:[DI]	MOVE.L	#dest,eDI	ES:[eDI]

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

REPNZ				REPNZ

CMPSB				CMPSB

BNE.S	no match		BNE.S	no match

(e)CX = remain count - 1, (e)SI and (e)DI = matched byte + 1

; no matches

(e)CX = 0, (e)SI = source+1000 , (e)DI = dest+1000

Do NOT rely on (e)CX = 0 to signify there are no differences / matches;

(e)CX will also = 0 if the last byte is the only difference / match.

to override the source segment (the destination cannot be overridden)

compare byte from CS:[(e)SI] with byte at ES:[(e)DI] :-

CS:				CS:

CMPSB				CMPSB

Notes:	The destination segment cannot be overridden.

Watch out for segment wraps in a 16-bit segment!

680x0:	If you use the decrement mode, it is POST decrement (decrement after

the data is copied).

Also:	REPE, REPZ, REPNE, REPNZ, CLD, STD

--------------------------------------------------------------------------------

CMPXCHG (compare and exchange) {486-}

Syntax:	CMPXCHG destination,source	Reverse: CMPXCHG source,destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

Compare AL/AX/eAX with the destination. If they are equal, the

destination is loaded with the source; otherwise AL/AX/eAX is loaded

with the destination.

Instruction forms:		byte		word		long

register to register		CL,BL		CX,BX		eCX,eBX

register to memory		CL,[mem]	CX,[mem]	eCX,[mem]

--------------------------------------------------------------------------------

CS: (segment override) {!!!} {prefix}

Syntax:	CS:

Flags:	Unaffected

Override the default segment for the next instruction.

--------------------------------------------------------------------------------

CWD (convert word to double)

Syntax:	CWD

Flags:	Unaffected

Extend sign bit of AX through DX.

Also:	MOVSX, CBW, CWDE, CDQ

--------------------------------------------------------------------------------

CWDE (convert word to double, extended) {386-}

Syntax:	CWDE

Flags:	Unaffected

Extend sign bit of AX through eAX.

Also:	MOVSX, CBW, CWD, CDQ

--------------------------------------------------------------------------------

DAA (decimal adjust after addition)

Syntax:	DAA

Flags:	sign (SF), zero (ZF), auxiliary carry (AF), parity (PF), carry (CF);

overflow (OF) is undefined.

Adjust packed BCD in AL after addition.

--------------------------------------------------------------------------------

DAS (decimal adjust after subtraction)

Syntax:	DAS

Flags:	sign (SF), zero (ZF), auxiliary carry (AF), parity (PF), carry (CF);

overflow (OF) is undefined.

Adjust packed BCD in AL after subtraction.

--------------------------------------------------------------------------------

DEC (decrement)

Syntax:	DEC destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF)

Decrement (subtract 1 from) the destination.

Instruction forms:		byte		word		long

register			AL		AX		eAX

memory {1}			[mem]		[mem]		[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

680x0:	The carry flag (CF) is NOT affected!

--------------------------------------------------------------------------------

DIV (unsigned division) {stack?}

DIVU (division, unsigned) {680x0} {stack?}

Syntax:	DIV AL/AX/eAX,divisor		Reverse: DIV divisor,AL/AX/eAX

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF) are undefined.

For byte, divide AX by divisor, AL = quotient, AH = remainder.

For word, divide DX:AX by divisor, AX = quotient, DX = remainder.

For long, divide eDX:eAX by divisor, eAX = quotient, eDX = remainder.

Instruction forms:		byte		word		long

register			BL		BX		eBX           �

BL,AL		BX,AX		eBX,eAX       �

BL,AH:AL	BX,DX:AX	eBX,eDX:eAX   �

BL,AL:AH	BX,AX:DX	eBX,eAX:eDX   �

memory			    {1}	[mem]		[mem]		[mem]         �

[mem],AL	[mem],AX	[mem],eAX     �

[mem],AH:AL	[mem],DX:AX	[mem],eDX:eAX �

[mem],AL:AH	[mem],AX:DX	[mem],eAX:eDX �

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

DS: (segment override) {!!!} {prefix}

Syntax:	DS:

Flags:	Unaffected

Override the default segment for the next instruction.

--------------------------------------------------------------------------------

ENTER (create stack frame) {286-}

Syntax:	ENTER #parameters,#nest depth

Flags:	Unaffected

#nest depth determines the number of stack frame pointers that are

being copied from the current stack frame (using eBP) into the new stack

frame. eBP then points to the new stack frame. #parameters is the amount

of bytes reserved from the stack for local variables.

Example:

ENTER	#word,#byte

Also:	LEAVE

--------------------------------------------------------------------------------

ESC (escape)

Syntax:	ESC source

Flags:	Unaffected

Provides access to the data bus for coprocessors. The CPU treats this

as a NOP, but places the memory operand on the bus.

Example:

ESC	[BX]

--------------------------------------------------------------------------------

ES: (segment override) {!!!} {prefix}

Syntax:	ES:

Flags:	Unaffected

Override the default segment for the next instruction.

--------------------------------------------------------------------------------

FS: (segment override) {!!!} {prefix} {386-}

Syntax:	FS:

Flags:	Unaffected

Override the default segment for the next instruction.

--------------------------------------------------------------------------------

FWAIT (event wait)

Syntax:	FWAIT

Flags:	Unaffected

The CPU enters wait state until the coprocessor signals it has finished

its operation. This is used to prevent the CPU from accessing memory

that may be temporarily needed by the coprocessor. FWAIT is the same

instruction as WAIT.

--------------------------------------------------------------------------------

GS: (segment override) {!!!} {prefix} {386-}

Syntax:	GS:

Flags:	Unaffected

Override the default segment for the next instruction.

--------------------------------------------------------------------------------

HLT (halt CPU)

Syntax:	HLT

Flags:	Unaffected

Stop instruction execution and place the CPU into halt state (an enabled

interrupt or NMI will resume execution at next the instruction).

--------------------------------------------------------------------------------

IDIV (signed division) {stack?}

DIVS (division, signed) {680x0} {stack?}

Syntax:	IDIV AL/AX/eAX,divisor		Reverse: IDIV divisor,AL/AX/eAX

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF) are undefined.

For byte, divide AX by divisor, AL = quotient, AH = remainder.

For word, divide DX:AX by divisor, AX = quotient, DX = remainder.

For long, divide eDX:eAX by divisor, eAX = quotient, eDX = remainder.

Instruction forms:		byte		word		long

register			BL		BX		eBX           �

BL,AL		BX,AX		eBX,eAX       �

BL,AH:AL	BX,DX:AX	eBX,eDX:eAX   �

BL,AL:AH	BX,AX:DX	eBX,eAX:eDX   �

memory			    {1}	[mem]		[mem]		[mem]         �

[mem],AL	[mem],AX	[mem],eAX     �

[mem],AH:AL	[mem],DX:AX	[mem],eDX:eAX �

[mem],AL:AH	[mem],AX:DX	[mem],eAX:eDX �

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

IMUL (signed multiplication) {stack?}

MULS (multiplication, signed) {680x0} {stack?}

Syntax:	IMUL AL/AX/eAX,source		Reverse: IMUL source,AL/AX/eAX

register,source			      source,register

register,#value			      #value,register

register,source,#value		      #value,source,register

Flags:	overflow (OF), carry (CF);

sign (SF), zero (ZF), auxiliary carry (AF), parity (PF) are undefined.

For byte, AX = AL * source. For word, DX:AX = AX * source. For long,

eDX:eAX = eAX * source.

If the destination register is specified, as AL / AX / eAX

(not AH:AL / DX:AX / eDX:eAX), the new address mode is assumed :-

For word or long, register = register * source			{386-}

register * #value			{286-}

register * source * #value		{286-}

Instruction forms:		byte		word		long

register			BL		BX		eBX           �

BL,AL		BX,AX		eBX,eAX       �

BL,AH:AL	BX,DX:AX	eBX,eDX:eAX   �

BL,AL:AH	BX,AX:DX	eBX,eAX:eDX   �

memory			    {1}	[mem]		[mem]		[mem]         �

[mem],AL	[mem],AX	[mem],eAX     �

[mem],AH:AL	[mem],DX:AX	[mem],eDX:eAX �

[mem],AL:AH	[mem],AX:DX	[mem],eAX:eDX �

register = source * register			[mem],BX	[mem],eBX

CX,BX		eCX,eBX

register = #value * register			#imm,BX		#imm,eBX

register = #value (signed byte) * register	#imm.b,BX	#imm.b,eBX

register = #value * source			#imm,CX,BX	#imm,eCX,eBX

#imm,[mem],BX	#imm,[mem],eBX

register = #value (signed byte) * source	#imm.b,CX,BX	#imm.b,eCX,eBX

#imm.b,[mem],BX	#imm.b,[mem],eBX

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

IN (input from port)

Syntax:	IN AL/AX,eAX,port  or  IN port,AL/AX/eAX

Flags:	Unaffected

Input value from port into AL/AX/eAX. Port can be either be immediate,

from 0-255, or from DX (only 10 bits, 0-1023, are used by the PC).

Instruction forms:		Byte		Word		Long

immediate port		    {1}	port		port		port          �

port,AL		port,AX		port,eAX      �

AL,port		AX,port		eAX,port      �

DX port			    {1}	DX		DX		DX            �

AL		AX		eAX           �

DX,AL		DX,AX		DX,eAX        �

AL,DX		AX,DX		eAX,DX        �

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

INC (increment)

Syntax:	INC destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF)

Increment (add 1 to) the destination.

Instruction forms:		byte		word		long

register			AL		AX		eAX

memory {1}			[mem]		[mem]		[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

680x0:	The carry flag (CF) is NOT affected!

--------------------------------------------------------------------------------

INSB INS.B (input string byte from port) {286-}

INSW INS.W (input string word from port) {286-}

INSD INS.D INSL INS.L (input string long from port) {386-}

Syntax:	Inherent

Flags:	Unaffected

This is a string operation. Input the byte, word or long from port DX,

storing at ES:[(e)DI. Then, depending on the direction flag (DF),

increment or decrement (e)DI by 1, 2 or 4. REP can be used as a prefix

to repeat the operation, using (e)CX as the count. DI and CX (if repeat)

are used for a 16-bit segment. eDI and eCX (if repeat) are used for a

32-bit segment.

Examples:

Remember, ES: is the destination segment.

Assume the direction flag is forward (DF=0, CLD).

store byte from port DX at ES:[(e)DI]

INSB

store 1000 bytes from port DX into buffer

16-bit segment			32-bit segment

MOVE.W	#buffer,DI   ES:[DI]	MOVE.L	#buffer,eDI   ES:[eDI]

MOVE.W	#port,DX		MOVE.W	#port,DX

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

REP				REP

INSB				INSB

(e)CX = 0, (e)DI = buffer+1000

Notes:	The destination segment cannot be overridden.

Watch out for segment wraps in a 16-bit segment!

680x0:	If you use the decrement mode, it is POST decrement (decrement after

the data is copied).

Also:	IN, REP, CLD, STD

--------------------------------------------------------------------------------

INT (interrupt) {stack}

Syntax:	INT #n

Flags:	Unaffected

Initiate a software interrupt, from 0-255. The flags are PUSHed onto

the stack, clearing the trap (TF) and interrupt (IF) flags, PUSHing CS

then (e)IP onto the stack and loading CS:(e)IP from the interrupt vector

table.

Example:

INT	#$21			MS-DOS function call

--------------------------------------------------------------------------------

INT3 (interrupt #3) {!!!} {stack}

Syntax:	INT3

Flags:	Unaffected

Same as using INT #3, but this saves a byte.

Also:	INT

--------------------------------------------------------------------------------

INTO (interrupt on overflow) {stack?}

Syntax:	INTO

Flags:	Unaffected

Generate interrupt #04 if overflow flag (OF) is set.

Also:	INT

--------------------------------------------------------------------------------

INVD (invalidate cache) {486-}

Syntax:	INVD

Flags:	Unaffected

Flushes CPU internal cache. Issues special function bus cycle which

indicates to flush external caches. Data in the write back external

cache is lost.

--------------------------------------------------------------------------------

INVLPG (invalidate translation look aside buffer entry) {486-}

Syntax:	???

Flags:	Unaffected

Invalidates a single page table entry in the translation look aside

buffer. INTEL warns that this instruction may be implemented differently

on future processors.

--------------------------------------------------------------------------------

IRET (return from interrupt) {stack}

IRETW IRET.W (word return from interrupt) {stack}

IRETD IRET.D IRETL IRET.L (long return from interrupt) {stack} {386-}

Syntax:	Inherent

Flags:	loaded from the stack.

The return address is POPed from the stack into	the program counter

((e)IP), followed by the code segment (CS, word or long depending on

word or long operation), followed by the (e)FLAGS register.

Notes:	You can specify .W or .L/.D to force a switch to a word or long

operation, but use with care.

Also:	INT, INTO, INT3

--------------------------------------------------------------------------------

Jc (jump conditionally)

Bcc (branch conditionally) {680x0}

JA,JAE,JB,JBE,JC,JE,JG,JGE,JL,JLE,JNA,JNAE,JNB,JNBE,JNC

JNE,JNG,JNGE,JNL,JNLE,JNO,JNP,JNS,JNZ,JO,JP,JPE,JPO,JS,JZ

BCC,BCS,BEQ,BGE,BGT,BHI,BHS,BLE,BLO,BLS,BLT,BMI,BNE,BPL,BRA,BSR,BVC,BVS

See "Conditional Branches" after this section.

--------------------------------------------------------------------------------

JCX0 JCXE JCXZ (jump if CX=0)

JeCX0 JeCXE JeCXZ (jump if eCX=0) {386-}

Syntax:	JCX0 label

Flags:	Unaffected

Branch to label if (e)CX=0. See "Conditional Branches" after this

section.

--------------------------------------------------------------------------------

JMP (unconditional jump)

Syntax:	JMP label

Flags:	Unaffected

Jump unconditionally, to label, register, indirect memory address or to

FAR address.

Examples: BX = 100, value at [100] = 1234, at [102] = 5678. 16-bit segment.

jump to register {1}		JMP	BX		jump to 100

jump to indirect address	JMP{2}	[BX]		�

JMP{2}	[100]		� jump to 1234.

branch short to address		JMP.S	label

branch to address		JMP	label		(.W/.L/.D is optional)

jump to FAR address		JMP{2}	segment:address

JMP.F	segment:address

jump to FAR indirect address	JMP.F	[BX]		� jump to address 1234,

JMP.F	[100]		� in segment 5678.

{1}	In a 16-bit segment; if you use a 32-bit register, an exception will

occur if the register value is greater than a 16-bit value, that is:

JMP eBX		will cause an exception if eBX >= 64k.

In a 32-bit segment; if you use a 16-bit register, the address jumped to

will obviously by within the first 64k of the segment.

{2}	You can specify .W or .L/.D to force a switch to a 16 or 32-bit segment

operation, but use with care.

Also:	JMPF, BRA

--------------------------------------------------------------------------------

JMPF (unconditional jump, FAR) {!!!}

Syntax:	JMPF label

Flags:	Unaffected

Jump FAR unconditionally, to the far address or far indirect address.

Examples: BX = 100, value at [100] = 1234, at [102] = 5678. 16-bit segment.

jump to FAR address		JMPF	segment:address

jump to FAR indirect address 	JMPF	[BX]		� jump to address 1234,

JMPF	[100]		� in segment 5678.

Notes:	You can specify .W or .L/.D to force a switch to a 16 or 32-bit segment

operation, but use with care.

Also:	JMP

--------------------------------------------------------------------------------

LAHF (load AH from FLAGS)

Syntax:	LAHF

Flags:	Unaffected

Copies bits 0-7 of the status flags into AH.

bit 7:	sign

bit 6:	zero

bit 4:	auxiliary carry

bit 2:	parity

bit 0:	carry

The other bits of AH are undefined and should not be used.

--------------------------------------------------------------------------------

LAR (load access rights) {286-} {pm}

Syntax:	LAR destination,source		Reverse: LAR source,destination

Flags:	zero (ZF).

If segment selector for source is visible and a valid descriptor type,

copy the access rights bytes from source to destination and set the zero

flag (ZF); Otherwise, clear the zero flag (ZF).

Instruction forms:		byte		word		long

LAR from register				AX,BX		eAX,eBX

LAR from memory					[mem],BX	[mem],eBX

--------------------------------------------------------------------------------

LDS (load register and DS)

Syntax:	LDS register,source		Reverse: LDS source,register

Flags:	Unaffected

Load register and DS from memory. Word or Long only.

Examples:

LDS	[mem],AX	load AX from [mem], load DS from [mem+2]

LDS	[mem],eAX	load eAX from [mem], load DS from [mem+4]

--------------------------------------------------------------------------------

LEA (load effective address)

Syntax:	LEA register,effective address	Reverse: LEA effective address,register

Flags:	Unaffected

Set register to effective address. Word or Long only. Segment overrides

in the effective address are ignored.

Examples:

LEA	n[BX],AX		AX = BX + n

LEA	[n],AX			AX = n (but use MOVE #n,AX)

LEA	n[eBX],AX		works, the upper 16-bits are ignored

LEA	n[eBX*4,eCX],eAX	eAX = (eBX*4) + eCX + n

LEA	n[eBX],eAX		eAX = (eBX*1) + n	*1

LEA	n[eBX*2],eAX		eAX = (eBX*2) + n	*2

LEA	n[eBX*2,eBX],eAX	eAX = (eBX*3) + n	*3

LEA	n[eBX*4],eAX		eAX = (eBX*4) + n	*4

LEA	n[eBX*4,eBX],eAX	eAX = (eBX*5) + n	*5

LEA	n[eBX*8],eAX		eAX = (eBX*8) + n	*8

LEA	n[eBX*8,eBX],eAX	eAX = (eBX*9) + n	*9

LEA	n[AX],eAX		works, the upper 16-bits are zero

--------------------------------------------------------------------------------

LEAVE (exit procedure) {286-}

Syntax:	LEAVE

Flags:	Unaffected

Removes current stack frame by restoring (e)BP and (e)SP to their

condition before the procedure was entered.

Also:	ENTER

--------------------------------------------------------------------------------

LES (load register and ES)

Syntax:	LES register,source		Reverse: LES source,register

Flags:	Unaffected

Load register and ES from memory. Word or Long only.

Examples:

LES	[mem],AX	load AX from [mem], load ES from [mem+2]

LES	[mem],eAX	load eAX from [mem], load ES from [mem+4]

--------------------------------------------------------------------------------

LFS (load register and FS) {386-}

Syntax:	LFS register,source		Reverse: LFS source,register

Flags:	Unaffected

Load register and FS from memory. Word or Long only.

Examples:

LFS	[mem],AX	load AX from [mem], load FS from [mem+2]

LFS	[mem],eAX	load eAX from [mem], load FS from [mem+4]

--------------------------------------------------------------------------------

LGDT (load global descriptor table) {286-} {pm}

Syntax:	LGDT memory

Flags:	Unaffected

Load the global descriptor table (GDT) from memory.

Example:

LGDT	[mem]

--------------------------------------------------------------------------------

LGS (load register and GS) {386-}

Syntax:	LGS register,source		Reverse: LGS source,register

Flags:	Unaffected

Load register and GS from memory. Word or Long only.

Examples:

LGS	[mem],AX	load AX from [mem], load GS from [mem+2]

LGS	[mem],eAX	load eAX from [mem], load GS from [mem+4]

--------------------------------------------------------------------------------

LIDT (load interrupt descriptor table) {286-} {pm}

Syntax:	LIDT memory

Flags:	Unaffected

Load the interrupt descriptor table (IDT) from memory.

Example:

LIDT	[mem]

--------------------------------------------------------------------------------

LLDT (load local descriptor table) {286-} {pm}

Syntax:	LLDT source

Flags:	Unaffected

Load the local descriptor table (LDT) from source.

Instruction forms:		byte		word		long

LLDT from register				AX

LLDT from memory				[mem]

--------------------------------------------------------------------------------

LMSW (load machine status word) {286-} {pm}

Syntax:	LMSW source

Flags:	Unaffected

Load the machine status word (MSW) from source.

Instruction forms:		byte		word		long

LMSW from register				AX

LMSW from memory				[mem]

--------------------------------------------------------------------------------

LOCK (lock bus) {prefix}

Syntax:	LOCK

Flags:	Unaffected

Assert bus lock (LOCK#) signal for the duration of the next instruction.

This is used to	prevent two processors from updating the same memory

location. The next instruction can be a bit test, xchg, addition,

subtraction or logical operation; or XCHG, MOVE, IN, OUT; depending on

which reference material you believe.

--------------------------------------------------------------------------------

LODSB LODS.B (load string byte)

LODSW LODS.W (load string word)

LODSD LODS.D LODSL LODS.L (load string long) {386-}

Syntax:	Inherent

Flags:	Unaffected

This is a string operation. Copy the byte, word or long from DS:[(e)SI]

to AL, AX or eAX. Then, depending on the direction flag (DF), increment

or decrement (e)SI by 1, 2 or 4. The source segment can be overridden.

SI is used for a 16-bit segment, eSI is used for a 32-bit segment.

Examples:

load byte from DS:[(e)SI] into AL :-

LODSB

load byte from CS:[(e)SI] into AL :-

CS:

LODSB

Notes:	REPeat can be used, but seems of little use (loading 1000 bytes from

memory into AL, for example).

Watch out for segment wraps in a 16-bit segment!

680x0:	If you use the decrement mode, it is POST decrement (decrement after

the data is copied).

Also:	REP, CLD, STD

--------------------------------------------------------------------------------

LOOP (loop always)

Syntax:	LOOP label

Flags:	Unaffected

Decrement (e)CX and branch to label if (e)CX is not zero. See

"Conditional Branches" after this section.

--------------------------------------------------------------------------------

LOOPE (loop if equal)

LOOPZ (loop if zero)

Syntax:	LOOPE label

Flags:	Unaffected

Decrement (e)CX and branch to label if (e)CX is not zero and the zero

flag (ZF) is set. See "Conditional Branches" after this section.

--------------------------------------------------------------------------------

LOOPNE (loop if not equal)

LOOPNZ (loop if not zero)

Syntax:	LOOPNE label

Flags:	Unaffected

Decrement (e)CX and branch to label if (e)CX is not zero and the zero

flag (ZF) is clear. See "Conditional Branches" after this section.

--------------------------------------------------------------------------------

@LSL (load segment limit) {286-} {pm}

Syntax:	@LSL destination,source		Reverse: @LSL source,destination

Flags:	zero (ZF).

If segment selector of source is visible and valid, copy the segment

limit from source to destination and set the zero flag (ZF); Otherwise,

clear the zero flag (ZF).

Instruction forms:		byte		word		long

@LSL from register				AX,BX		eAX,eBX

@LSL from memory				[mem],BX	[mem],eBX

--------------------------------------------------------------------------------

LSS (load register and SS) {386-}

Syntax:	LSS register,source		Reverse: LSS source,register

Flags:	Unaffected

Load register and SS from memory. Word or Long only.

Examples:

LSS	[mem],AX	load AX from [mem], load SS from [mem+2]

LSS	[mem],eAX	load eAX from [mem], load SS from [mem+4]

--------------------------------------------------------------------------------

LTR (load task register) {286-} {pm}

Syntax:	LTR source

Flags:	Unaffected

Load the task register from source and mark the task as busy.

Instruction forms:		byte		word		long

LTR from register				AX

LTR from memory					[mem]

--------------------------------------------------------------------------------

MOV (move)

MOVE (move) {680x0}

Syntax:	MOV destination,source		Reverse: MOV source,destination

Flags:	Unaffected

Copy value from source to destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

segment register to register	 		DS,AX		ES,eAX

segment register to memory {2}			FS,[mem]	GS,[mem]

register to segment register {3}		AX,DS		eAX,ES

memory to segment register {2,3}		[mem],FS	[mem],GS

segment register to segment register {4}	DS,ES		FS,GS

control register to register {386-}				CR0,eAX

register to control register {386-}				eAX,CR2

debug register to register {386-}				DR0,eAX

register to debug register {386-}				eAX,DR1

test register to register {386-}				TR3,eAX

register to test register {386-}				eAX,TR4

memory to memory {1,5}		[mem],[mem]	[mem],[mem]	[mem],[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

{2}	Size (.W or .L/.D) is optional. The default is Word (.W) for a 16-bit

segment, Long (.L/.D) for a 32-bit segment.

{3}	Segment register CS: cannot be modified this way.

{4}	This is a pseudo operation. The source segment register is PUSHed onto

the stack, and then the destination segment register is POPed from the

stack. The destination segment register cannot be CS: or SS:.

{5}	This is a pseudo operation. For a Byte operation, AX/eAX is PUSHed onto

the stack, AL is read from the source, stored at the destination, then

AX/eAX is POPed from the stack. For a Word or Long operation, the source

is PUSHed on the stack, then POPed from the stack to the destination.

680x0:	Flags are NOT affected.

--------------------------------------------------------------------------------

MOVSB MOVS.B (move string byte)

MOVSW MOVS.W (move string word)

MOVSD MOVS.D MOVSL MOVS.L (move string word) {386-}

Syntax:	Inherent

Flags:	Unaffected

This is a string operation. Copy the byte, word or long from DS:[(e)SI]

to ES:[(e)DI]. Then, depending on the direction flag (DF), increment or

decrement (e)SI and (e)DI by 1, 2 or 4. REP can be used as a prefix to

repeat the operation, using (e)CX as the count. The source segment can

be overridden. SI, DI and CX (if repeat) are used for a 16-bit segment.

eSI, eDI and eCX (if repeat) are used for a 32-bit segment.

Examples:

Remember, DS: is the source segment, ES: is the destination segment.

Assume the direction flag is forward (DF=0, CLD).

to copy 1000 bytes of data :-

16-bit segment			32-bit segment

MOVE.W	#source,SI  DS:[SI]	MOVE.L	#source,eSI	DS:[eSI]

MOVE.W	#dest,DI    ES:[DI]	MOVE.L	#dest,eDI	ES:[eDI]

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

REP				REP

MOVSB				MOVSB

(e)CX = 0, (e)SI = source+1000 , (e)DI = dest+1000

to override the source segment (the destination cannot be overridden)

copy byte from CS:[(e)SI] to ES:[(e)DI] :-

CS:				CS:

MOVSB				MOVSB

to override the destination with a repeated move string operation

copy 1000 bytes from CS:[(e)SI] to ES:[(e)DI] :-

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

REP				REP

CS:				CS:

MOVSB				MOVSB

Notes:	REP is assembled as REPZ. REPZ or REPNZ could also be used.

The destination segment cannot be overridden.

Watch out for memory overlaps and segment wraps in a 16-bit segment!

680x0:	If you use the decrement mode, it is POST decrement (decrement after

the data is copied).

Also:	REP, CLD, STD

--------------------------------------------------------------------------------

MOVSX (move with sign extend) {386-}

Syntax:	MOVSX destination,source	Reverse: MOVSX source,destination

Flags:	Unaffected

Sign extend source to a larger destination register. Copy data from

source to the destination register, extending the sign bit to fill

the destination.

Instruction forms:

byte register to word register		MOVSX	AL,BX

byte register to long register		MOVSX	AL,eBX

word register to long register		MOVSX	AX,eBX

byte memory to word register		MOVSX	[mem],BX

byte memory to long register {1}	MOVSX.b	[mem],eBX

word memory to long register {1}	MOVSX.w	[mem],eBX

{1}	Size (.B / .W) is needed. The default is byte (.B).

680x0:	MOVSX can be used like EXT.

Also:	MOVZX, CBW, CWD, CWDE, CDQ

--------------------------------------------------------------------------------

MOVZX (move with zero extend) {386-}

Syntax:	MOVZX destination,source	Reverse: MOVZX source,destination

Flags:	Unaffected

Zero extend source to a larger destination register. Copy data from

source to the destination register, clearing the upper bits of the

destination.

Instruction forms:

byte register to word register		MOVZX	AL,BX

byte register to long register		MOVZX	AL,eBX

word register to long register		MOVZX	AX,eBX

byte memory to word register		MOVZX	[mem],BX

byte memory to long register {1}	MOVZX.b	[mem],eBX

word memory to long register {1}	MOVZX.w	[mem],eBX

{1}	Size (.B / .W) is needed. The default is byte (.B).

Also:	MOVSX

--------------------------------------------------------------------------------

MUL (unsigned multiplication) {stack?}

MULU (multiplication, unsigned) {680x0} {stack?}

Syntax:	MUL AL/AX/eAX,source		Reverse: MUL source,AL/AX/eAX

Flags:	overflow (OF), carry (CF);

sign (SF), zero (ZF), auxiliary carry (AF), parity (PF) are undefined.

For byte, AX = AL * source. For word, DX:AX = AX * source. For long,

eDX:eAX = eAX * source.

Instruction forms:		byte		word		long

register			BL		BX		eBX           �

BL,AL		BX,AX		eBX,eAX       �

BL,AH:AL	BX,DX:AX	eBX,eDX:eAX   �

BL,AL:AH	BX,AX:DX	eBX,eAX:eDX   �

memory			    {1}	[mem]		[mem]		[mem]         �

[mem],AL	[mem],AX	[mem],eAX     �

[mem],AH:AL	[mem],DX:AX	[mem],eDX:eAX �

[mem],AL:AH	[mem],AX:DX	[mem],eAX:eDX �

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

NEG (negate destination)

Syntax:	NEG destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

Subtracts the destination from 0.

Instruction forms:		byte		word		long

register			AL		AX		eAX

memory {1}			[mem]		[mem]		[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

NOP (no operation)

Syntax:	NOP

Flags:	Unaffected

No operation. This is the same as XCHG AL,AL.

--------------------------------------------------------------------------------

NOT (logical NOT)

Syntax:	NOT destination

Flags:	Unaffected

Invert each bit of destination.

Instruction forms:		byte		word		long

register			AL		AX		eAX

memory {1}			[mem]		[mem]		[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

680x0:	The flags are unaffected!

--------------------------------------------------------------------------------

OPSIZE: (operation size switch) {!!!} {386-} {prefix}

Syntax:	OPSIZE:

Flags:	Unaffected

Switch the operation size (between 16/32-bit) for the next instruction.

--------------------------------------------------------------------------------

OR (logical OR)

Syntax:	OR destination,source		Reverse: OR source,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Logically OR source with destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

OUT (output data to port)

Syntax:	OUT port,AL/AX,eAX  or  OUT AL/AX/eAX,port

Flags:	Unaffected

Output value from AL/AX/eAX to port. Port can be either be immediate,

from 0-255, or from DX (only 10 bits, 0-1023, are used by the PC).

Instruction forms:		Byte		Word		Long

immediate port		    {1}	port		port		port

AL,port		AX,port		eAX,port

port,AL		port,AX		port,eAX

DX port			    {1}	DX		DX		DX

AL		AX		eAX

AL,DX		AX,DX		eAX,DX

DX,AL		DX,AX		DX,eAX

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

OUTSB OUTS.B (out string byte) {286-}

OUTSW OUTS.W (out string word) {286-}

OUTSD OUTS.D OUTSL OUTS.L (out string long) {386-}

Syntax:	Inherent

Flags:	Unaffected

This is a string operation. Output the byte, word or long from

DS:[(e)SI] to port DX. Then, depending on the direction flag (DF),

increment or decrement (e)SI by 1, 2 or 4. The source segment can be

overridden. REP can be used as a prefix to repeat the operation, using

(e)CX as the count. SI and CX (if repeat) are used for a 16-bit segment.

eSI and eCX (if repeat) are used for a 32-bit segment.

Examples:

Remember, DS: is the source segment.

Assume the direction flag is forward (DF=0, CLD).

output byte from DS:[(e)SI] to port DX :-

OUTSB

output byte from CS:[(e)SI] to port DX :-

CS:

OUTSB

output 1000 bytes from buffer to port DX

16-bit segment			32-bit segment

MOVE.W	#buffer,SI   DS:[SI]	MOVE.L	#buffer,eSI   DS:[eSI]

MOVE.W	#port,DX		MOVE.W	#port,DX

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

REP				REP

OUTSB				OUTSB

(e)CX = 0, (e)SI = buffer+1000

Notes:	Watch out for segment wraps in a 16-bit segment!

680x0:	If you use the decrement mode, it is POST decrement (decrement after

the data is copied).

Also:	OUT, REP, CLD, STD

--------------------------------------------------------------------------------

POP (pop data from stack) {stack}

Syntax:	POP destination

Flags:	Unaffected

POPs data from stack to the destination. Word or Long only.

Instruction forms:		byte		word		long

register					AX		eAX

memory {1}					[mem]		[mem]

segment register {2}				DS		ES

{1}	Size (.W or .L/.D) is needed. The default is Word (.W) for a 16-bit

segment, Long (.L/.D) for a 32-bit segment.

{2}	Size (.W or .L/.D) is optional. The default is Word (.W) for a 16-bit

segment, Long (.L/.D) for a 32-bit segment.

Also:	PUSH, POPM

--------------------------------------------------------------------------------

POPA (pop all data registers) {286-} {stack}

POPAW POPA.W (pop all word data registers) {286-} {stack}

POPAD POPA.D POPAL POPA.L (pop all long data registers) {386-} {stack}

Syntax:	Inherent

Flags:	Unaffected

POPs all the data registers from the stack. Word or Long only. For POPA,

words are used for a 16-bit segment, longs for a 32-bit segment. The POP

order is :-

words:			longs:

1st	DI   0[SP] 		eDI   0[eSP]

2nd	SI   2[SP] 		eSI   4[eSP]

3rd	BP   4[SP] 		eBP   8[eSP]

4th	SP   6[SP] 		eSP  12[eSP]	ignored!!!

5th	BX   8[SP] 		eBX  16[eSP]

6th	DX  10[SP] 		eDX  20[eSP]

7th	CX  12[SP] 		eCX  24[eSP]

8th	AX  14[SP] 		eAX  28[eSP]

16 bytes from stack	32 bytes from stack

Also:	PUSHA, POPM

--------------------------------------------------------------------------------

POPF (pop (e)FLAGS from stack) {stack}

POPFW POPF.W (pop FLAGS from stack) {stack}

POPFD POPF.D POPFL POPF.L (pop eFLAGS from stack) {386-} {stack}

Syntax:	Inherent

Flags:	loaded from the stack.

POPs the (e)FLAGS register from the stack. Word or Long only.

For POPF, FLAGS are POPed for a 16-bit segment, eFLAGS for a 32-bit

segment.

Note:	For (e)FLAGS, VM (virtual mode) and RF (resume flag) are not changed.

IOPL (input output priority level) and IF (interrupt flag) are only

changed if the task's privilege level is high enough.

Also:	PUSHF

--------------------------------------------------------------------------------

POPM (pop multiple registers from stack) {!!!} {stack}

Syntax:	POPM register list

Flags:	Unaffected

POP the registers in the register list from the stack. The registers

are POPed in a specific order, so do not need to be listed in order.

Registers can be separated by ','s or '/'s. The POP order is :-

first:	eAX,eBX,eCX,eDX,eBP,eSI,eDI

AX,BX,CX,DX,BP,SI,DI

DS,ES,FS,GS			:last

Examples:

POPM	AX,BX,CX,DX	with	AX = [SP+0]

BX = [SP+2]

CX = [SP+4]

DX = [SP+6]

POPM	DI,SI,CX,DX	with	CX = [SP+0]

DX = [SP+2]

SI = [SP+4]

DI = [SP+4]

--------------------------------------------------------------------------------

PUSH (push data onto stack) {stack}

Syntax:	PUSH source

Flags:	Unaffected

PUSHes data from source onto the stack. Word or Long only.

Instruction forms:		byte		word		long

register					AX		eAX

memory {1}					[mem]		[mem]

immediate {1} {286-}				#imm		#imm

immediate, sign extended byte {1} {286-}	#imm.b		#imm.b

segment register {2}				CS		DS

{1}	Size (.W or .L/.D) is needed. The default is Word (.W) for a 16-bit

segment, Long (.L/.D) for a 32-bit segment.

{2}	Size (.W or .L/.D) is optional. The default is Word (.W) for a 16-bit

segment, Long (.L/.D) for a 32-bit segment.

Also:	POP, PUSHM

--------------------------------------------------------------------------------

PUSHA (pop all data registers) {286-} {stack}

PUSHAW PUSHA.W (pop all word data registers) {286-} {stack}

PUSHAD PUSHAL PUSHA.D PUSHA.L (pop all long data registers) {386-} {stack}

Syntax:	Inherent

Flags:	Unaffected

PUSHes all the data registers onto the stack. Word or Long only. For

PUSHA, words are used for a 16-bit segment, longs for a 32-bit segment.

The PUSH order is :-

words:			longs:

1st	AX  14[SP] 		eAX  28[eSP]

2nd	CX  12[SP] 		eCX  24[eSP]

3rd	DX  10[SP] 		eDX  20[eSP]

4th	BX   8[SP] 		eBX  16[eSP]

5th	SP   6[SP] 		eSP  12[eSP]

6th	BP   4[SP] 		eBP   8[eSP]

7th	SI   2[SP] 		eSI   4[eSP]

8th	DI   0[SP] 		eDI   0[eSP]

16 bytes onto stack	32 bytes onto stack

Also:	POPA, PUSHM

--------------------------------------------------------------------------------

PUSHF (push (e)FLAGS onto stack) {stack}

PUSHFW PUSHF.W (push FLAGS onto stack) {stack}

PUSHFD PUSHFL PUSHF.D PUSHF.L (push eFLAGS onto stack) {386-} {stack}

Syntax:	Inherent

Flags:	Unaffected

PUSHes the (e)FLAGS register onto the stack. Word or Long only.

For PUSHF, FLAGS are PUSHed for a 16-bit segment, eFLAGS for a 32-bit

segment.

Also:	POPF

--------------------------------------------------------------------------------

PUSHM (push multiple registers onto stack) {!!!} {stack}

Syntax:	PUSHM register list

Flags:	Unaffected

PUSH the registers in the register list onto the stack. The registers

are PUSHed in a specific order, so do not need to be listed in order.

Registers can be separated by ','s or '/'s. The PUSH order is :-

first:	GS,FS,ES,DS

DI,SI,BP,DX,CX,BX,AX

eDI,eSI,eBP,eDX,eCX,eBX,eAX	:last

Examples:

PUSHM	AX,BX,CX,DX	with	[SP+0] = AX

[SP+2] = BX

[SP+4] = CX

[SP+6] = DX

PUSHM	DI/SI/CX/DX	with	[SP+0] = CX

[SP+2] = DX

[SP+4] = SI

[SP+4] = DI

--------------------------------------------------------------------------------

RCL (rotate through carry left)

ROXL (rotate with extend left) {680x0}

Syntax:	RCL destination,count		Reverse: RCL count,destination

Flags:	overflow (OF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) is set if the sign (SF) changes,

if count > 1, overflow (OF) is undefined (but see notes:).

Rotate destination left, through the carry flag (CF), by count

positions. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

rotate register by 1		BL		BX		eBX

rotate memory by 1 {1}		[mem]		[mem]		[mem]

rotate register by CL		CL,BL		CL,BX		CL,eBX

rotate memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

rotate register by count	#count,BL	#count,BX	#count,eBX

rotate memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

--------------------------------------------------------------------------------

RCR (rotate through carry right)

ROXR (rotate with extend right) {680x0}

Syntax:	RCR destination,count		Reverse: RCR count,destination

Flags:	overflow (OF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) is set if the sign (SF) changes,

if count > 1, overflow (OF) is undefined (but see notes:).

Rotate destination right, through the carry flag (CF), by count

positions. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

rotate register by 1		BL		BX		eBX

rotate memory by 1 {1}		[mem]		[mem]		[mem]

rotate register by CL		CL,BL		CL,BX		CL,eBX

rotate memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

rotate register by count	#count,BL	#count,BX	#count,eBX

rotate memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

--------------------------------------------------------------------------------

REP (repeat string operation) {prefix}

Syntax:	REP, or REP string operation

Flags:	Unaffected, but the string operation may set the flags.

{1} If (e)CX <> 0, perform the string operation, then decrement (e)CX

and repeat from {1}. The string operation (MOVS, STOS, INS or OUTS) can

be specified on the same line, or else it must be the next instruction.

CX is used in a 16-bit segment, eCX is used in a 32-bit segment.

Examples:

REP	MOVSB		the string operation on the same line

REP

MOVSB			the string operation as the next instruction

Notes:	REP is assembled as REPE. SCAS and CMPS could be the string instruction,

but for clarity you should use REPE or REPZ.

Also:	MOVS, STOS, INS, OUTS

--------------------------------------------------------------------------------

REPE, REPZ (repeat string operation if equal) {prefix}

Syntax:	REPE, or REPE string operation

Flags:	Unaffected, but the string operation may set the flags.

{1} If (e)CX <> 0, perform the string operation, then decrement (e)CX.

If the zero flag (ZF) is set, repeat from {1}. The string operation

(CMPS or SCAS) can be specified on the same line, or else it must be the

next instruction. CX is used in a 16-bit segment, eCX is used in a 32

bit segment.

Examples:

REPE	CMPSB		the string operation on the same line

REPZ

SCASB			the string operation as the next instruction

Notes:	The string operation could be MOVS, STOS, INS or OUTS, but would be

repeated regardless of the zero flag (ZF).

Also:	REPNE, REPNZ, CMPS, SCAS

--------------------------------------------------------------------------------

REPNE, REPNZ (repeat string operation if not equal) {prefix}

Syntax:	REPNE, or REPNE string operation

Flags:	Unaffected, but the string operation may set the flags.

{1} If (e)CX <> 0, perform the string operation, then decrement (e)CX.

If the zero flag (ZF) is clear, repeat from {1}. The string operation

(CMPS or SCAS) can be specified on the same line, or else it must be the

next instruction. CX is used in a 16-bit segment, eCX is used in a 32

bit segment.

Examples:

REPNE	CMPSB		the string operation on the same line

REPNZ

SCASB			the string operation as the next instruction

Notes:	The string operation could be MOVS, STOS, INS or OUTS, but would be

repeated regardless of the zero flag (ZF).

Also:	REPE, REPZ, CMPS, SCAS

--------------------------------------------------------------------------------

RET.F (return, FAR) {stack}

RET RET.S (return, near) {stack}

RETW RET.W (word return, near) {stack}

RETD RET.D RETL RET.L (long return, near) {386-} {stack}

Syntax:	RET, or RET #value

Flags:	Unaffected

Return from subroutine. The return address is POPed from the stack into

the program counter ((e)IP). The optional #value, if specified, is then

added to (e)SP. If size is not specified, word is used for a 16-bit

segment, long is used for a 32-bit segment.

Examples:

RET		return from subroutine.

RET.F		return from FAR subroutine (see RETF).

RET	#8	return from subroutine, tidy stack by 8.

Notes:	You can specify .W or .L/.D to force a switch to a 16 or 32-bit segment

operation, but use with care.

Also:	RTS, RETF, CALL, CALLF

--------------------------------------------------------------------------------

RETF (return, FAR) {stack}

RETFW RETF.W (word return, FAR) {stack}

RETFD RETF.D RETFL RETF.L (long return, FAR) {386-} {stack}

Syntax:	RETF, or RETF #value

Flags:	Unaffected

Return from FAR subroutine. The return address is POPed from the stack

into the program counter ((e)IP), followed by the code segment (CS, word

or long depending on the segment operation type). The optional #value,

if specified, is then added to (e)SP. If size is not specified, word is

used for a 16-bit segment, long is used for a 32-bit segment.

Examples:

RETF		return from FAR subroutine.

RETF	#8	return from FAR subroutine, tidy stack by 8.

Notes:	You can specify .W or .L/.D to force a switch to a 16 or 32-bit segment

operation, but use with care.

Also:	CALLF, JSRF

--------------------------------------------------------------------------------

REV (reverse byte order) {!!!}

REVERSE (reverse byte order) {!!!}

Syntax:	REV register

Flags:	overflow (OF), carry (CF), unless register is AX,BX,CX or DX.

Reverse the byte order of the word/long register. This is a pseudo

operation.

Examples:

REV AX, BX, CX, DX		XCHG	AL,AH	BL,BH	CL,CH	DL,DH

REV SP, BP, SI, DI		ROR.W	#8,SP	#8,BP	#8,SI	#8,DI

REV eAX, eBX, eCX, eDX		XCHG	AL,AH	BL,BH	CL,CH	DL,DH

ROR.L	#16,eAX	#16,eBX	#16,eCX	#16,eDX

XCHG	AL,AH	BL,BH	CL,CH	DL,DH

REV eSP, eBP, eSI, eDI		ROR.W	#8,SP	#8,BP	#8,SI	#8,DI

ROR.L	#16,eSP	#16,eBP	#16,eSI	#16,eDI

ROR.W	#8,SP	#8,BP	#8,SI	#8,DI

--------------------------------------------------------------------------------

ROL (rotate left)

Syntax:	ROL destination,count		Reverse: ROL count,destination

Flags:	overflow (OF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) is set if the sign (SF) changes,

if count > 1, overflow (OF) is undefined (but see notes:).

Rotate destination left, from high order bit to low order bit, by count

positions. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

rotate register by 1		BL		BX		eBX

rotate memory by 1 {1}		[mem]		[mem]		[mem]

rotate register by CL		CL,BL		CL,BX		CL,eBX

rotate memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

rotate register by count	#count,BL	#count,BX	#count,eBX

rotate memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

--------------------------------------------------------------------------------

ROR (rotate right)

Syntax:	ROR destination,count		Reverse: ROR count,destination

Flags:	overflow (OF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) is set if the sign (SF) changes,

if count > 1, overflow (OF) is undefined (but see notes:).

Rotate destination right, from low order bit to high order bit, by count

positions. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

rotate register by 1		BL		BX		eBX

rotate memory by 1 {1}		[mem]		[mem]		[mem]

rotate register by CL		CL,BL		CL,BX		CL,eBX

rotate memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

rotate register by count	#count,BL	#count,BX	#count,eBX

rotate memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

--------------------------------------------------------------------------------

RTS (return from subroutine) {680x0}

Syntax:	RTS

Flags:	Unaffected

Return from subroutine. The return address is POPed from the stack into

the program counter ((e)IP).

--------------------------------------------------------------------------------

SAHF (store AH into FLAGS)

Syntax:	SAHF

Flags:	sign (SF), zero (ZF), auxiliary carry (AF), parity (PF), carry (CF) are

loaded from AH.

Store AH into status flags.

bit 7:	sign

bit 6:	zero

bit 4:	auxiliary carry

bit 2:	parity

bit 0:	carry

The other bits of the status flag are unaffected.

--------------------------------------------------------------------------------

SAL (shift arithmetic left)

ASL (arithmetic shift left) {680x0}

Syntax:	SAL destination,count		Reverse: SAL count,destination

Flags:	overflow (OF), sign (SF), zero (ZF), parity (PF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) =1 if the sign (SF) changes, =0 if not,

if count > 1, overflow (OF) is undefined (but see notes:);

auxiliary carry (AF) is undefined.

Shift destination left by count positions, shifting zero's in on the

right. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

shift register by 1		BL		BX		eBX

shift memory by 1 {1}		[mem]		[mem]		[mem]

shift register by CL		CL,BL		CL,BX		CL,eBX

shift memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

shift register by count		#count,BL	#count,BX	#count,eBX

shift memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

--------------------------------------------------------------------------------

SAR (shift arithmetic right)

ASR (arithmetic shift right) {680x0}

Syntax:	SAR destination,count		Reverse: SAR count,destination

Flags:	overflow (OF), sign (SF), zero (ZF), parity (PF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) is cleared (the sign does not change),

if count > 1, overflow (OF) is undefined (but see notes:);

auxiliary carry (AF) is undefined.

Shift destination right by count positions, copying the sign bit on the

left. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

shift register by 1		BL		BX		eBX

shift memory by 1 {1}		[mem]		[mem]		[mem]

shift register by CL		CL,BL		CL,BX		CL,eBX

shift memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

shift register by count		#count,BL	#count,BX	#count,eBX

shift memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

When tested, the parity flag (PF) does not seem to be set correctly :-

MOVE.B	#%11,AL

ASR	AL

when shifted, eAX has an odd number of set bits, so parity (PF) should

be clear; however, in all cases parity (PF) is set.

--------------------------------------------------------------------------------

SBB (subtract with borrow)

SBC (subtract with carry) {!!!}

SUBX (subtract extended) {680x0}

Syntax:	SBB destination,source		Reverse: SBB source,destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

Subtract the source and the carry flag (CF) from the destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

SCASB SCAS.B (scan string byte)

SCASW SCAS.W (scan string word)

SCASD SCAS.D SCASL SCAS.L (scan string long) {386-}

Syntax:	Inherent

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF)

and carry (CF).

This is a string operation. Set the status flags as if the byte, word or

long at ES:[(e)DI] were subtracted from AL, AX or eAX. Then, depending

on the direction flag (DF), increment or decrement (e)SI and (e)DI by

1, 2 or 4. DI and CX (if repeat) are used for a 16-bit segment. eDI and

eCX (if repeat) are used for a 32-bit segment.

REPZ or REPE can be used as a prefix to repeat the operation if the

register and destination are the same; REPNZ or REPNE can be used as a

prefix to repeat the operation if the register and destination are not

the same; both use (e)CX as the repeat counter.

Examples:

Remember, ES: is the destination segment.

Assume the direction flag is forward (DF=0, CLD).

compare byte at ES:[(e)DI] with AL, setting flags :-

SCASB

scan to the end of a string, maximum length of string = 1000 bytes :-

16-bit segment			32-bit segment

MOVE.W	#string,DI		MOVE.L	#string,eDI

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

CLR.B	AL			CLR.B	AL

REPNZ				REPNZ

SCASB				SCASB

BNE.S	end not found		BNE.S	end not found

(e)CX = remain count - 1, (e)DI = string end + 1

; end not found

(e)CX = 0 , (e)DI = dest+1000

string	db	'this is a string',0

Do NOT rely on (e)CX = 0 to signify the end was not found, (e)CX will

also = 0 if the last byte is the end of the string.

Notes:	The destination segment cannot be overridden.

Watch out for segment wraps in a 16-bit segment!

680x0:	If you use the decrement mode, it is POST decrement (decrement after

the data is copied).

Also:	REPE, REPZ, REPNE, REPNZ, CLD, STD

--------------------------------------------------------------------------------

SETcc (set byte conditionally) {386-}

Scc {680x0} (set byte conditionally) {386-}

SETA,SETAE,SETB,SETBE,SETC,SETE,SETG,SETGE,SETL,SETLE

SETNA,SETNAE,SETNB,SETNBE,SETNC,SETNE,SETNG,SETNGE,SETNL

SETNLE,SETNO,SETNP,SETNS,SETNZ,SETO,SETP,SETPE,SETPO,SETS,SETZ

SCC,SCS,SEQ,SF,SGE,SGT,SHI,SHS,SLE,SLO,SLS,SLT,SMI,SNE,SPL,ST,SVC,SVS

See "Set Byte Conditionally" after this section.

--------------------------------------------------------------------------------

SGDT (store global descriptor table) {286-} {pm}

Syntax:	SGDT memory

Flags:	Unaffected

Store the global descriptor table (GDT) into memory.

Example:

SGDT	[mem]

--------------------------------------------------------------------------------

SHL (shift left)

LSL (logical shift left) {680x0}

Syntax:	SHL destination,count		Reverse: SHL count,destination

Flags:	overflow (OF), sign (SF), zero (ZF), parity (PF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) =1 if the sign (SF) changes, =0 if not,

if count > 1, overflow (OF) is undefined (but see notes:);

auxiliary carry (AF) is undefined.

Shift destination left by count positions, shifting zero's in on the

right. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

shift register by 1		BL		BX		eBX

shift memory by 1 {1}		[mem]		[mem]		[mem]

shift register by CL		CL,BL		CL,BX		CL,eBX

shift memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

shift register by count		#count,BL	#count,BX	#count,eBX

shift memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

--------------------------------------------------------------------------------

SHLD (shift left double) {386-}

LSLD (logical shift left double) {!!!} {386-}

Syntax:	SHLD destination,source,count	Reverse: SHLD count,source,destination

Flags:	carry (CF), parity (PF), sign (SF), zero (ZF);

overflow (OF), auxiliary flag (AF) are undefined.

Shift the destination left by count positions, copying from the left

of source to the right of destination. The source is unchanged. Count

can be by register CL or an immediate count.

Instruction forms:		byte		word		long

shift into register				CL,AX,BX	AX,eAX,eBX

#count,AX,BX	#count,eAX,eBX

shift into memory				CL,AX,[mem]	CL,eAX,[mem]

#count,AX,[mem]	#count,eAX,[mem]

--------------------------------------------------------------------------------

SHR (shift right)

LSR (logical shift right) {680x0}

Syntax:	SHR destination,count		Reverse: SHR count,destination

Flags:	overflow (OF), sign (SF), zero (ZF), parity (PF), carry (CF);

if count = 0, flags are unchanged,

if count = 1, overflow (OF) =1 if the sign (SF) changes, =0 if not,

if count > 1, overflow (OF) is undefined (but see notes:);

auxiliary carry (AF) is undefined.

Shift destination right by count positions, shifting zero's in on the

left. Count can be one, register CL or an immediate count {286-}.

Instruction forms:		byte		word		long

shift register by 1		BL		BX		eBX

shift memory by 1 {1}		[mem]		[mem]		[mem]

shift register by CL		CL,BL		CL,BX		CL,eBX

shift memory by CL {1}		CL,[mem]	CL,[mem]	CL,[mem]

shift register by count		#count,BL	#count,BX	#count,eBX

shift memory by count {1}	#count,[mem]	#count,[mem]	#count,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Notes:	The overflow (OF) seems to be set correctly for all shift counts.

When tested, the parity flag (PF) does not seem to be set correctly :-

MOVE.B	#%11,AL

LSR	AL

when shifted, eAX has an odd number of set bits, so parity (PF) should

be clear; however, in all cases parity (PF) is set.

--------------------------------------------------------------------------------

SHRD (shift right double) {386-}

LSRD (logical shift right double) {!!!} {386-}

Syntax:	SHRD destination,source,count	Reverse: SHRD count,source,destination

Flags:	carry (CF), parity (PF), sign (SF), zero (ZF);

overflow (OF), auxiliary flag (AF) are undefined.

Shift the destination right by count positions, copying from the right

of source to the left of destination. The source is unchanged. Count

can be by register CL or an immediate count.

Instruction forms:		byte		word		long

shift into register				CL,AX,BX	AX,eAX,eBX

#count,AX,BX	#count,eAX,eBX

shift into memory				CL,AX,[mem]	CL,eAX,[mem]

#count,AX,[mem]	#count,eAX,[mem]

--------------------------------------------------------------------------------

SIDT (store interrupt descriptor table) {286-} {pm}

Syntax:	SIDT memory

Flags:	Unaffected

Store the interrupt descriptor table (IDT) into memory.

Example:

SIDT	[mem]

--------------------------------------------------------------------------------

SLDT (store local descriptor table) {286-} {pm}

Syntax:	SLDT destination

Flags:	Unaffected

Store the local descriptor table (LDT) into destination.

Instruction forms:		byte		word		long

SLDT into register				AX

SLDT into memory				[mem]

--------------------------------------------------------------------------------

SMSW (store machine status word) {286-} {pm}

Syntax:	SMSW destination

Flags:	Unaffected

Store the machine status word (MSW) into destination.

Instruction forms:		byte		word		long

SMSW into register				AX

SMSW into memory				[mem]

--------------------------------------------------------------------------------

SS: (segment override) {!!!} {prefix}

Syntax:	SS:

Flags:	Unaffected

Override the default segment for the next instruction.

--------------------------------------------------------------------------------

STC (set carry)

Syntax:	STC

Flags	carry (CF) = 1.

Set the carry flag (CF).

--------------------------------------------------------------------------------

STD (set direction flag)

Syntax:	STD

Flags:	direction (DF) = 1.

Set the direction flag (direction = backwards, decrement).

--------------------------------------------------------------------------------

STI (set interrupt flag)

Syntax:	STI

Flags	interrupt flag (IF) = 1.

Set the interrupt flag, enabling interrupts.

--------------------------------------------------------------------------------

STOSB STOS.B (store string byte)

STOSW STOS.W (store string word)

STOSD STOS.D STOSL STOS.L (store string long) {386-}

Syntax:	Inherent

Flags:	Unaffected

This is a string operation. Store AL, AX or eAX at ES:[(e)DI]. Then,

depending on the direction flag (DF), increment or decrement (e)DI by

1, 2 or 4. REP can be used as a prefix to repeat the operation, using

(e)CX as the count. SI, DI and CX (if repeat) are used for a 16-bit

segment. eSI, eDI and eCX (if repeat) are used for a 32-bit segment.

Examples:

Remember, ES: is the destination segment.

Assume the direction flag is forward (DF=0, CLD).

store AL at ES:[(e)DI] :-

STOSB

clear a block of 1000 bytes :-

16-bit segment			32-bit segment

MOVE.W	#dest,DI   ES:[DI]	MOVE.L	#dest,eDI   ES:[eDI]

MOVE.W	#1000,CX		MOVE.L	#1000,eCX

CLR.B	AL			CLR.B	AL

REP				REP

STOSB                           STOSB

(e)CX = 0, (e)DI = dest+1000

Notes:	The destination segment cannot be overridden.

Watch out for segment wraps in a 16-bit segment!

680x0:	If you use the decrement mode, it is POST decrement (decrement after

the data is copied).

Also:	REP, CLD, STD

--------------------------------------------------------------------------------

STR (store task register) {286-} {pm}

Syntax:	STR source

Flags:	Unaffected

Store the task register into destination.

Instruction forms:		byte		word		long

STR into register				AX

STR into memory					[mem]

--------------------------------------------------------------------------------

SUB (subtract)

Syntax:	SUB destination,source		Reverse: SUB source,destination

Flags:	overflow (OF), sign (SF), zero (ZF), auxiliary carry (AF), parity (PF),

carry (CF).

Subtract the source from the destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

SWAP (swap register halves) {680x0}

Syntax:	SWAP register

Flags:	overflow (OF), carry (CF), unless register is AX,BX,CX or DX.

Swap the register halves. This is a pseudo operation.

Examples:

SWAP AX, BX, CX, DX		XCHG	AL,AH	BL,BH	CL,CH	DL,DH

SWAP SP, BP, SI, DI		ROR.W	#8,SP	#8,BP	#8,SI	#8,DI

SWAP eAX, eBX, eCX, eDX		ROR.L	#16,eAX	#16,eBX	#16,eCX	#16,eDX

SWAP eBP, eSP, eSI, eDI		ROR.L	#16,eBP	#16,eSP	#16,eSI	#16,eDI

--------------------------------------------------------------------------------

TEST (test bit pattern)

Syntax:	TEST destination,source		Reverse: TEST source,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Set flags the same as for AND, leaving the destination unchanged.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

TST (test operand) {680x0}

Syntax:	TST operand

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0,

auxiliary carry (AF); auxiliary carry (AF) is undefined for register.

Compare the operand with zero. This is a pseudo operation.

Instruction forms:		byte		word		long

TST register (OR reg,reg)	AL		AX		eAX

TST memory (CMP #0.b,[mem]) {1}	[mem]		[mem]		[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

--------------------------------------------------------------------------------

VERR (verify read) {286-} {pm}

Syntax:	VERR source

Flags:	zero (ZF)

If source segment is reachable from current privilege level and is

readable, set the zero flag (ZF); Otherwise, clear the zero flag (ZF).

Instruction forms:		byte		word		long

VERR from register				AX

VERR from memory				[mem]

--------------------------------------------------------------------------------

VERW (verify write) {286-} {pm}

Syntax:	VERW source

Flags:	zero (ZF)

If source segment is reachable from current privilege level and is

writeable, set the zero flag (ZF); Otherwise, clear the zero flag (ZF).

Instruction forms:		byte		word		long

VERW from register				AX

VERW from memory				[mem]

--------------------------------------------------------------------------------

WAIT (event wait)

Syntax:	WAIT

Flags:	Unaffected

The CPU enters wait state until the coprocessor signals it has finished

its operation. This is used to prevent the CPU from accessing memory

that may be temporarily needed by the coprocessor. WAIT and FWAIT are

identical.

--------------------------------------------------------------------------------

WBINVD (write back and invalidate cache) {486-}

Syntax:	WBINVD

Flags:	Unaffected

Flushes CPU internal cache, signals the external cache to write	back

current data, followed by a signal to flush the external cache.

--------------------------------------------------------------------------------

XADD (exchange and add) {486-}

Syntax:	XADD destination,source		Reverse: XADD source,destination

Flags:	?????

Exchange and ADD, but I am unsure of the precise syntax.

Instruction forms:		byte		word		long

register to register		CL,BL		CX,BX		eCX,eBX

register to memory		CL,[mem]	CX,[mem]	eCX,[mem]

--------------------------------------------------------------------------------

xBCHG (bit change) {!!!}

Syntax:	xBCHG destination,#bit		Reverse: xBCHG #bit,destination

Flags:	overflow (OF) = 0, carry (CF) = 0;

sign (SF), zero (ZF), parity (PF), auxiliary carry (AF) are undefined.

Change #bit of the destination. This is a pseudo operation.

Instruction forms:		byte		word		long

change register bit		#bit,AL		#bit,AX		#bit,eAX

=	XOR #xxx,AL	XOR #xxx,AX	XOR #xxx,eAX

change memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	XOR #xxx,[mem]	XOR #xxx,[mem]	XOR #xxx,[mem]

#xxx is a bit mask, created from #bit.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Also:	BTC, BCHG

--------------------------------------------------------------------------------

xBCLR (bit clear) {!!!}

Syntax:	xBCLR destination,#bit		Reverse: xBCLR #bit,destination

Flags:	overflow (OF) = 0, carry (CF) = 0;

sign (SF), zero (ZF), parity (PF), auxiliary carry (AF) are undefined.

Clear #bit of the destination. This is a pseudo operation.

Instruction forms:		byte		word		long

clear register bit		#bit,AL		#bit,AX		#bit,eAX

=	AND #xxx,AL	AND #xxx,AX	AND #xxx,eAX

clear memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	AND #xxx,[mem]	AND #xxx,[mem]	AND #xxx,[mem]

#xxx is a bit mask, created from #bit.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Also:	BTR, BCLR

--------------------------------------------------------------------------------

xBSET (bit set) {!!!}

Syntax:	xBSET destination,#bit		Reverse: xBSET #bit,destination

Flags:	overflow (OF) = 0, carry (CF) = 0;

sign (SF), zero (ZF), parity (PF), auxiliary carry (AF) are undefined.

Set #bit of the destination. This is a pseudo operation.

Instruction forms:		byte		word		long

set register bit		#bit,AL		#bit,AX		#bit,eAX

=	OR #xxx,AL	OR #xxx,AX	OR #xxx,eAX

set memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	OR #xxx,[mem]	OR #xxx,[mem]	OR #xxx,[mem]

#xxx is a bit mask, created from #bit.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Also:	BTS, BSET

--------------------------------------------------------------------------------

xBTST (bit test) {!!!}

Syntax:	xBTST destination,#bit		Reverse: xBTST #bit,destination

Flags:	zero (ZF); overflow (OF) = 0, carry (CF) = 0;

sign (SF), parity (PF), auxiliary carry (AF) are undefined.

Test #bit of the destination. The destination is left unchanged. This

operation is the same as BTST. This is a pseudo operation.

Instruction forms:		byte		word		long

test register bit		#bit,AL		#bit,AX		#bit,eAX

=	TEST #xxx,AL	TEST #xxx,AX	TEST #xxx,eAX

test memory bit {1}		#bit,[mem]	#bit,[mem]	#bit,[mem]

=	TEST #xxx,[mem]	TEST #xxx,[mem]	TEST #xxx,[mem]

#xxx is a bit mask, created from #bit.

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

Also:	BT, BTST

--------------------------------------------------------------------------------

XCHG (exchange)

EXG (exchange) {680x0}

Syntax:	XCHG destination1,destination2

Flags:	Unaffected

Exchange values in destination1, destination2.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory {1}		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register {1}		[mem],AL	[mem],AX	[mem],eAX

{1}	Both forms are considered the same.

--------------------------------------------------------------------------------

XLAT (translate)

XLATW XLAT.W (word translate)

XLATD XLAT.D XLATL XLAT.L (long translate) {386-}

Syntax:	Inherent

Flags:	Unaffected

Copy byte from DS:[(e)BX+AL] to AL. The data segment can be overridden.

For XLAT, BX is used for a 16-bit segment, eBX for a 32-bit segment.

Word (W) version uses BX. Long (L/D) version uses eBX {386-}.

Example: to override the data segment :-

ES:

XLAT	loads AL from ES:[(e)BX+AL]

--------------------------------------------------------------------------------

XOR (exclusive OR)

EOR (exclusive OR) {680x0}

Syntax:	XOR destination,source		Reverse: XOR source,destination

Flags:	sign (SF), zero (ZF), parity (PF); overflow (OF) = 0, carry (CF) = 0;

auxiliary carry (AF) is undefined.

Logically XOR source with destination.

Instruction forms:		byte		word		long

register to register		AL,BL		AX,BX		eAX,eBX

register to memory		AL,[mem]	AX,[mem]	eAX,[mem]

memory to register		[mem],AL	[mem],AX	[mem],eAX

immediate to register		#imm,AL		#imm,AX		#imm,eAX

immediate to memory {1}		#imm,[mem]	#imm,[mem]	#imm,[mem]

signed immediate byte to register		#imm.b,AX	#imm.b,eAX

signed immediate byte to memory {1}		#imm.b,[mem]	#imm.b,[mem]

{1}	Size (.B, .W or .L/.D) is needed. The default is byte (.B).

������������������������������������������������������������������������������ͻ

�                            Conditional Branches                              �

������������������������������������������������������������������������������ͼ

Syntax:	branch label

Flags:	Unaffected

Branches can be Short (.S or .B) or Long (.W, .L or .D). A short branch is an

8 bit signed displacement. A long branch is a 16-bit signed displacement for a

16-bit segment, and 32-bit signed displacement for a 32-bit segment. For

simplicity, .W .L and .D are all considered long branches.

These branches are long by default. Long branches on a 80286 or lesser

processor are pseudo operations, for example:-

JC label		� JC.S	*+3

� JMP.W	label		16-bit segment

JC label		� JC.S	*+5

� JMP.L	label		32-bit segment

JA (jump if above, unsigned)

JNBE (jump if not below or equal, unsigned)

BHI (branch if higher, unsigned) {680x0}

Branch if carry flag (CF) = 0, and zero flag (ZF) = 0.

JAE (jump if above or equal, unsigned)

JNB (jump if not below, unsigned)

JNC (jump if not carry)

BCC (branch if carry is clear) {680x0}

BHS (branch is higher or the same, unsigned) {680x0}

Branch if carry flag (CF) = 0.

JB (jump if below, unsigned)

JC (jump if carry)

JNAE (jump if not above or equal, unsigned)

BCS (branch if carry is set) {680x0}

BLO (branch if lower, unsigned) {680x0}

Branch if carry flag (CF) = 1.

JBE (jump if below or equal, unsigned)

JNA (jump if not above, unsigned)

BLS (branch if lower or the same, unsigned) {680x0}

Branch if carry flag (CF) = 1, or zero flag (ZF) = 1.

JE (jump if equal)

JZ (jump if zero)

BEQ (branch if equal) {680x0}

Branch if zero flag (ZF) = 1.

JG (jump if greater, signed)

JNLE (jump if not less or equal, signed)

BGT (branch if greater than, signed) {680x0}

Branch if zero flag (ZF) = 0, and sign flag (SF) = overflow flag (OF).

JGE (jump if greater or equal, signed)

JNL (jump if not less, signed)

BGE (branch if greater or equal, signed) {680x0}

Branch if sign flag (SF) = overflow flag (OF).

JL (jump if less, signed)

JNGE (jump if not greater or equal, signed)

BLT (branch if less than, signed) {680x0}

Branch if sign flag (SF) <> overflow flag (OF).

JLE (jump if less or equal, signed)

JNG (jump if not greater, signed)

BLE (branch if less than or equal, signed) {680x0}

Branch if zero flag (ZF) = 1, or if sign flag (SF) <> overflow flag (OF).

JNE (jump if not equal)

JNZ (jump if not zero)

BNE (branch if not equal) {680x0}

Branch if zero flag (ZF) = 0.

JNO (jump if not overflow)

BVC (branch if overflow is clear) {680x0}

Branch if overflow flag (OF) = 0.

JNP (jump if not parity)

JPO (jump if parity odd)

Branch if parity flag (PF) = 0.

JNS (jump if not sign)

BPL (branch if plus) {680x0}

Branch if sign flag (SF) = 0.

JO (jump if overflow)

BVS (branch if overflow is set) {680x0}

Branch if overflow flag (OF) = 1.

JP (jump if parity)

JPE (jump if parity even)

Branch if parity flag (PF) = 1.

JS (jump if sign)

BMI (branch if minus) {680x0}

Branch if sign flag (SF) = 1.

--------------------------------------------------------------------------------

These branches are short by default. Long branches are pseudo operations, for

example:-

JCX0.w label		� JCX0	*+2

� JMP.S	*+3

� JMP.W	label		16-bit segment

JeCX0.l label		� JeCX0	*+2

� JMP.S	*+5

� JMP.L	label		32-bit segment

JCX0 (jump if CX = 0) {!!!}

JCXE (jump if CX = 0) {!!!}

JCXZ (jump if CX = 0)

Branch if CX = 0.

JeCX0 (jump if eCX = 0) {!!!} {386-}

JeCXE (jump if eCX = 0) {!!!} {386-}

JeCXZ (jump if eCX = 0) {386-}

Branch if eCX = 0.

LOOPw (CX)

LOOPd, LOOPl (eCX) {386-}

LOOP (use CX for a 16-bit segment, eCX for a 32-bit segment)

loop: decrement (e)CX, branch if (e)CX <> 0.

LOOPEw, LOOPZw (CX)

LOOPEd, LOOPZd, LOOPEl, LOOPZl (eCX) {386-}

LOOPE, LOOPZ (use CX for a 16-bit segment, eCX for a 32-bit segment)

loop if equal: decrement (e)CX, branch if (e)CX <> 0 and zero flag (ZF) = 1.

LOOPNEw, LOOPNZw (CX)

LOOPNEd, LOOPNZd, LOOPNEl, LOOPNZl (eCX) {386-}

LOOPNE, LOOPNZ (use CX for a 16-bit segment, eCX for a 32-bit segment)

loop if not equal: decrement (e)CX, branch if (e)CX <> 0 and zero flag (ZF) = 0.

--------------------------------------------------------------------------------

BRA {680x0}

Branch always to label. This branch is long by default. See also JMP.

BSR {680x0} {stack}

Branch to subroutine. This branch is always long. See also CALL, RET, RTS.

������������������������������������������������������������������������������ͻ

�                           Set Byte Conditionally                             �

������������������������������������������������������������������������������ͼ

Syntax:	SETcc destination

Flags:	Unaffected

If the condition is true, MOVE #1 to the destination. If the condition

is false, MOVE #0 to the destination. The destination can either be a

register or memory address.

SETA (set if above, unsigned) {386-}

SETNBE (set if not below or equal, unsigned) {386-}

SHI (set if higher, unsigned) {680x0} {386-}

Set if carry flag (CF) = 0, and zero flag (ZF) = 0.

SETAE (set if above or equal, unsigned) {386-}

SETNB (set if not below, unsigned) {386-}

SETNC (set if not carry) {386-}

SCC (set if carry is clear) {680x0} {386-}

SHS  (set is higher or the same, unsigned) {680x0} {386-}

Set if carry flag (CF) = 0.

SETB (set if below, unsigned) {386-}

SETC (set if carry) {386-}

SETNAE (set if not above or equal, unsigned) {386-}

SCS (set if carry is set) {680x0} {386-}

SLO (set if lower, unsigned) {680x0} {386-}

Set if carry flag (CF) = 1.

SETBE (set if below or equal, unsigned) {386-}

SETNA (set if not above, unsigned) {386-}

SLS (set if lower or the same, unsigned) {680x0} {386-}

Set if carry flag (CF) = 1, or zero flag (ZF) = 1.

SETE (set if equal) {386-}

SETZ (set if zero) {386-}

SEQ (set if equal) {680x0} {386-}

Set if zero flag (ZF) = 1.

SETG (set if greater, signed) {386-}

SETNLE (set if not less or equal, signed) {386-}

SGT (set if greater than, signed) {680x0} {386-}

Set if zero flag (ZF) = 0, and sign flag (SF) = overflow flag (OF).

SETGE (set if greater or equal, signed) {386-}

SETNL (set if not less, signed) {386-}

SGE (set if greater or equal, signed) {680x0} {386-}

Set if sign flag (SF) = overflow flag (OF).

SETL (set if less, signed) {386-}

SETNGE (set if not greater or equal, signed) {386-}

SLT (set if less than, signed) {680x0} {386-}

Set if sign flag (SF) <> overflow flag (OF).

SETLE (set if less or equal, signed) {386-}

SETNG (set if not greater, signed) {386-}

SLE (set if less than or equal, signed) {680x0} {386-}

Set if zero flag (ZF) = 1, or if sign flag (SF) <> overflow flag (OF).

SETNE (set if not equal) {386-}

SETNZ (set if not zero) {386-}

SNE (set if not equal) {680x0} {386-}

Set if zero flag (ZF) = 0.

SETNO (set if not overflow) {386-}

SVC (set if overflow is clear) {680x0} {386-}

Set if overflow flag (OF) = 0.

SETNP (set if not parity) {386-}

SETPO (set if parity odd) {386-}

Set if parity flag (PF) = 0.

SETNS (set if not sign) {386-}

SPL (set if plus) {680x0} {386-}

Set if sign flag (SF) = 0.

SETO (set if overflow) {386-}

SVS (set if overflow is set) {680x0} {386-}

Set if overflow flag (OF) = 1.

SETP (set if parity) {386-}

SETPE (set if parity even) {386-}

Set if parity flag (PF) = 1.

SETS (set if sign) {386-}

SMI (set if minus) {680x0} {386-}

Set if sign flag (SF) = 1.

ST (set always) {680x0}

Same as MOVE.B #1,destination, included because of the 680x0 instruction.

SF (set never) {680x0}

Same as MOVE.B #0,destination, included because of the 680x0 instruction.

680x0:	The value stored at the destinations is 1 if the condition is true, not

-1 as it is on the 680x0. NEG the destination afterwards if you need -1

when the condition is true.

������������������������������������������������������������������������������ͻ

�                                   Appendix                                   �

������������������������������������������������������������������������������ͼ

Protected Mode Instructions {pm}

ARPL (adjust requested privilege level)

LAR (load access rights)

LGDT (load global descriptor table)

LIDT (load interrupt descriptor table)

LLDT (load local descriptor table)

LMSW (load machine status word)

@LSL (load segment limit)

LTR (load task register)

SGDT (store global descriptor table)

SIDT (store interrupt descriptor table)

SLDT (store local descriptor table)

SMSW (store machine status word)

STR (store task register)

VERR (verify read)

VERW (verify write)

Renamed 80x86 Instructions

@BSR (bit scan reverse)

@LSL (load segment limit)

Instruction Prefixes

ADDR: (address mode switch)

CS: (override segment)

DS: (override segment)

ES: (override segment)

FS: (override segment)

GS: (override segment)

LOCK (lock bus)

OPSIZE: (operation size switch)

REP (repeat string operation)

REPE/REPZ (repeat string operation if equal/zero)

REPNE/REPNZ (repeat string operation if not equal/not zero)

SS: (override segment)

680x0 Equivalent Branches

BCC (branch if carry clear) = JNC (jump if not carry)

BCS (branch if carry set) = JC (jump if carry)

BEQ (branch if equal) = JE (jump if equal)

BGE (branch if greater or equal, signed)= JGE (jump if greater or equal)

BGT (branch if greater than, signed) = JG (jump if greater)

BHI (branch if higher, unsigned) = JA (jump if above)

BHS (branch if higher or same, unsigned) = JAE (jump if above or equal)

BLE (branch if less or equal, signed) = JLE (jump if less or equal)

BLO (branch if lower, unsigned) = JB (jump if below)

BLS (branch if lower or same, unsigned) = JBE (jump if below or equal)

BLT (branch if less than, signed) = JL (jump if lower)

BMI (branch if minus) = JS (jump if sign)

BNE (branch if not equal) = JNE (jump if not equal)

BPL (branch if plus) = JNS (jump if not sign)

BVC (branch if overflow clear) = JNO (jump if not overflow)

BVS (branch if overflow set) = JO (jump if overflow)

BRA (branch always) = JMP (jump unconditionally)

BSR (branch to subroutine = CALL (call procedure)

680x0 Set Byte Conditionally

SCC (set if carry clear) = SETNC (set if not carry)

SCS (set if carry set) = SETC (set if carry)

SEQ (set if equal) = SETE (set if equal)

SGE (set if greater or equal, signed)= SETGE (set if greater or equal)

SGT (set if greater than, signed) = SETG (set if greater)

SHI (set if higher, unsigned) = SETA (set if above)

SHS (set if higher or same, unsigned) = SETAE (set if above or equal)

SLE (set if less or equal, signed) = SETLE (set if less or equal)

SLO (set if lower, unsigned) = SETB (set if below)

SLS (set if lower or same, unsigned) = SETBE (set if below or equal)

SLT (set if less than, signed) = SETL (set if lower)

SMI (set if minus) = SETS (set if sign)

SNE (set if not equal) = SETNE (set if not equal)

SPL (set if plus) = SETNS (set if not sign)

SVC (set if overflow clear) = SETNO (set if not overflow)

SVS (set if overflow set) = SETO (set if overflow)

ST  (set true, always) = MOVE.B #1,[dest]

SF  (set false, never) = MOVE.B #0,[dest]

680x0 Equivalent Instructions

ADDX (add extended) = ADC (add with carry)

ASL (arithmetic shift left) = SAL (shift arithmetic left)

ASR (arithmetic shift right) = SAR (shift arithmetic right)

BCHG (bit test and change)

BCLR (bit test and clear)

BSET (bit test and set)

BTCHG (bit test with change) = BTC (bit test with complement)

BTCLR (bit test with clear) = BTR (bit test with reset)

BTSET (bit test with set) = BTS (bit test with set)

BTSF (bit scan forward) = BSF (bit scan forward)

BTSR (bit scan reverse) = @BSR (bit scan reverse)

BTST (bit test)

BTTST (bit test) = BT (bit test)

CLR (clear operand)

DIVS (division, signed) = IDIV (signed division)

DIVU (division, unsigned) = DIV (unsigned division)

EOR (exclusive OR) = XOR (exclusive OR)

EXG (exchange) = XCHG (exchange)

JSR (jump to subroutine) = CALL (call procedure)

LSL (logical shift left) = SHL (shift left)

LSR (logical shift right) = SHR (shift right)

MOVE (move) = MOV (move)

MULS (multiplication, signed) = IMUL (signed multiplication)

MULU (multiplication, unsigned) = MUL (unsigned multiplication)

ROXL (rotate with extend left) = RCL (rotate through carry left)

ROXR (rotate with extend right) = RCR (rotate through carry right)

RTS (return from subroutine) = RET (return, near)

SUBX (subtract extended) = SBB (subtract with borrow)

SWAP (swap register halves)

TST (test operand)

680x0/80x86 Hybrid Instructions

CALLF (call FAR procedure)

INT3 (interrupt #03)

JMPF (jump FAR)

JSRF (jump to FAR subroutine)

LSLD (logical shift left double) = SHLD (shift left double)

LSRD (logical shift right double) = SHRD (shift right double)

POPM (pop multiple registers from stack)

PUSHM (push multiple registers onto stack)

REV (reverse byte order)

REVERSE (reverse byte order)

SBC (subtract with carry) = SBB (subtract with borrow)

xBCHG (bit change)

xBCLR (bit clear)

xBSET (bit set)

xBTST (bit test)

������������������������������������������������������������������������������Ŀ

�                      How Instructions Affect The Flags                       �

��������������������������������������������������������������������������������

OF	SF	ZF	AF	PF	CF	DF	IF

AAA		u       u	u	.	u	.

AAD		u       .	.	u	.	u

AAM		u       .	.	u	.	u

AAS		u       u	u	.	u	.

ADC		.       .	.	.	.	.

ADD		.       .	.	.	.	.

AND		0       .	.	u	.	0

ARPL		_       _	.	_	_	_

BOUND		_       _	_	_	_	_

BSF		_       _	.	_	_	_

@BSR		_       _	.	_	_	_

BSWAP		_       _	_	_	_	_

BT		_       _	_	_	_	.

BTC		_       _	_	_	_	.

BTR		_       _	_	_	_	.

BTS		_       _	_	_	_	.

CALL		_       _	_	_	_	_

CBW		_       _	_	_	_	_

CDQ		_       _	_	_	_	_

CLC		_       _	_	_	_	0

CLD		_	_	_	_	_	_	0	_

CLI		_	_	_	_	_	_	_	0

CLTS		_       _	_	_	_	_

CMC		_       _	_	_	_	.

CMP		.       .	.	.	.	.

CMPS		.       .	.	.	.	.

CMPXCHG         .       .	.	.	.	.

CWD		_       _	_	_	_	_

CWDE		_       _	_	_	_	_

DAA		u       .	.	.	.	.

DAS		u       .	.	.	.	.

DEC		.       .	.	.	.	_

DIV		u       u	u	u	u	u

ENTER		_       _	_	_	_	_

ESC		_       _	_	_	_	_

FWAIT		_       _	_	_	_	_

HLT		_       _	_	_	_	_

IDIV		u       u	u	u	u	u

IMUL		.       u	u	u	u	.

IN		_       _	_	_	_	_

INC		.       .	.	.	.	_

INS		_       _	_	_	_	_

INT		_       _	_	_	_	_

INTO		_       _	_	_	_	_

INVD		_       _	_	_	_	_

INVLPG		_       _	_	_	_	_

IRET		.       .	.	.	.	.

Jc		_       _	_	_	_	_

JCX0		_       _	_	_	_	_

JeCX0		_       _	_	_	_	_

JMP		_       _	_	_	_	_

LAHF		_       _	_	_	_	_

LAR		_       _	.	_	_	_

LDS		_       _	_	_	_	_

LEA		_       _	_	_	_	_

LEAVE		_       _	_	_	_	_

LES		_       _	_	_	_	_

LFS		_       _	_	_	_	_

LGDT		_       _	_	_	_	_

LGS		_       _	_	_	_	_

LIDT		_       _	_	_	_	_

LLDT		_       _	_	_	_	_

LMSW		_       _	_	_	_	_

LOCK		_       _	_	_	_	_

LODS		_       _	_	_	_	_

LOOP		_       _	_	_	_	_

LOOPE, LOOPZ	_       _	_	_	_	_

LOOPNE, LOOPNZ	_       _	_	_	_	_

@LSL		_       _	.	_	_	_

LSS		_       _	_	_	_	_

LTR		_       _	_	_	_	_

MOV		_       _	_	_	_	_

MOVS		_       _	_	_	_	_

MOVSX		_       _	_	_	_	_

MOVZX		_       _	_	_	_	_

MUL		.       u	u	u	u	.

NEG		.       .	.	.	.	.

NOP		_       _	_	_	_	_

NOT		_       _	_	_	_	_

OR		0       .	.	u	.	0

OUT		_       _	_	_	_	_

OUTS		_       _	_	_	_	_

POP		_       _	_	_	_	_

POPA		_       _	_	_	_	_

POPF		.	.	.	.	.	.	.	.

PUSH		_       _	_	_	_	_

PUSHA		_       _	_	_	_	_

PUSHF		_       _	_	_	_	_

RCL		.       _	_	_	_	.

RCR		.       _	_	_	_	.

REP		_       _	_	_	_	_

REPE, REPZ	_       _	_	_	_	_

REPNE, REPNZ	_       _	_	_	_	_

RET		_       _	_	_	_	_

RETF		_       _	_	_	_	_

ROL		.       _	_	_	_	.

ROR		.       _	_	_	_	.

SAHF		_       .	.	.	.	.

SAL		.       .	.	u	.	.

SAR		.       .	.	u	.	.

SBB		.       .	.	.	.	.

SCAS		.       .	.	.	.	.

SETcc		_       _	_	_	_	_

SGDT		_       _	_	_	_	_

SHL		.       .	.	u	.	.

SHLD		u       .	.	u	.	.

SHR		.       .	.	u	.	.

SHRD		u       .	.	u	.	.

SIDT		_       _	_	_	_	_

SLDT		_       _	_	_	_	_

SMSW		_       _	_	_	_	_

STC		_       _	_	_	_	1

STD		_	_	_	_	_	_	1	_

STI		_	_	_	_	_	_	_	1

STOS		_       _	_	_	_	_

STR		_       _	_	_	_	_

SUB		.       .	.	.	.	.

TEST		0       .	.	u	.	0

VERR		_       _	.	_	_	_

VERW		_       _	.	_	_	_

WAIT		_       _	_	_	_	_

WBINVD		_       _	_	_	_	_

XADD		?	?	?	?	?	?

XCHG		_       _	_	_	_	_

XLAT		_       _	_	_	_	_

XOR		0       .	.	u	.	0

ADDX (=ADC)	.       .	.	.	.	.

ASL (=SAL)	.       .	.	u	.	.

ASR (=SAR)	.       .	.	u	.	.

Bcc (=Jc)	_       _	_	_	_	_

BCHG		0       u	.	u	u	0

BCLR		0       u	.	u	u	0

BSET		0       u	.	u	u	0

BTCHG (=BTC)	_       _	_	_	_	.

BTCLR (=BTR)	_       _	_	_	_	.

BTSET (=BTS)	_       _	_	_	_	.

BTSF (=BSF)	_       _	.	_	_	_

BTSR (=@BSR)	_       _	.	_	_	_

BTST		0       u	.	u	u	0

BTTST (=BT)	_       _	_	_	_	.

CALLF		_       _	_	_	_	_

CLR		0       0	1	u	.	0

DIVS (=IDIV)	u       u	u	u	u	u

DIVU (=DIV)	u       u	u	u	u	u

EOR (=XOR)	0       .	.	u	.	0

EXG (=XCHG)	_       _	_	_	_	_

INT3 (=INT)	_       _	_	_	_	_

JMPF		_       _	_	_	_	_

JSR (=CALL)	_       _	_	_	_	_

JSRF (=CALLF)	_       _	_	_	_	_

LSL (=SHL)	.       .	.	u	.	.

LSLD (=SHLD)	u       .	.	u	.	.

LSR (=SHR)	.       .	.	u	.	.

LSRD (=SHRD)	u       .	.	u	.	.

MOVE (=MOV)	_       _	_	_	_	_

MULS (=IMUL)	.       u	u	u	u	.

MULU (=MUL)	.       u	u	u	u	.

POPM		_       _	_	_	_	_

PUSHM		_       _	_	_	_	_

ROXL (=RCL)	.       _	_	_	_	.

ROXR (=RCR)	.       _	_	_	_	.

REV, REVERSE	u       _	_	_	_	u

RTS		_       _	_	_	_	_

SBC (=SBB)	.       .	.	.	.	.

Scc (=SETcc)	_       _	_	_	_	_

SUBX (=SBB)	.       .	.	.	.	.

SWAP		u       _	_	_	_	u

TST		0       .	.	u	.	0

xBCHG		0       u	u	u	u	0

xBCLR		0       u	u	u	u	0

xBSET		0       u	u	u	u	0

xBTST		0       u	.	u	u	0

ADDR:		_       _	_	_	_	_

CS:		_       _	_	_	_	_

DS:		_       _	_	_	_	_

ES:		_       _	_	_	_	_

FS:		_       _	_	_	_	_

GS:		_       _	_	_	_	_

OPSIZE:		_       _	_	_	_	_

SS:		_       _	_	_	_	_

_	unaffected

.	set by instruction

u	undefined

0	cleared

1	set

?	unknown

������������������������������������������������������������������������������ͻ

�                               Debug Registers                                �

������������������������������������������������������������������������������ͼ

The debug registers can only be read / written at privilege level 0. Attempts to

access them at any other level with generate an invalid opcode exception. Even

at level 0, they registers can be further protected by the BD and GD bits in DR6

and DR7.

DR0-DR3	contain the linear address associated with each of the four

breakpoint conditions. Since the linear address is used, the

breakpoint facilities operate the same whether paging is enabled

or not.

DR4

DR5	reserved by Intel for future use by Intel.

DR6	debug status register. When a debug exception occurs, the

processor sets DR6 to indicate the exception type and/or

breakpoints that raised the exception.

DR7	debug control register. The bits in this register allow each

breakpoint register to be enabled, as well as the type of

breakpoint (instruction, data write, data read) to be selected.

The protection of all breakpoint registers is specified in this

register.

Upon entry to the debug handler, DR6 indicates what exception was detected.

Note that the bits of DR6 do not indicate whether the breakpoint condition was

enabled (which is indicated by DR7), but simply indicate that the condition was

detected. You therefore need to make sure that debug exceptions were enabled for

it to be able to cause the exception.

Debug exceptions for two different instructions can be reported simultaneously.

A fault on one exception may be reported at the same time as a trap on the

previous instruction. For example, an instruction breakpoint detected in one

instruction may be reported at the same time as a data breakpoint on the

previous instruction.

31  29  27  25  23  21  19  17  15  13  11   9   7   5   3   1

30  28  26  24  22  20  18  16  14  12  10   8   6   4   2   0

DR6	 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 B B B 0 0 0 0 0 0 0 0 0 B B B B

T S D                   3 2 1 0

DR7	 LEN:RWE LEN:RWE LEN:RWE LEN:RWE 0 0 G 0 0 0 G L G L G L G L G L

3   3   2   2   1   1   0   0     D       E E 3 3 2 2 1 1 0 0

DR6 bits

B0 B1 B2 B3	indicate that the breakpoint condition specified by the

corresponding breakpoint linear address register was detected.

Note that these bits are set even if the breakpoint condition

has not been enable in DR7.

BD		the BD bit is set at an instruction boundary if the next

instruction will read or write one of the eight debug registers

(debug register protection). The condition need not be enabled

by the GD bit of DR7.

BS		the BS bit is set if a single-step exception occurs. The single

step condition is enabled by the TF bit in the (e)FLAGS

register. This bit is set if the program enters the debug

handler because of a single step condition. Unlike the other

bits of DR6, the BS bit is set only if a single-step trap

actually occurs, not if a single step condition (enabled or

disabled) was detected.

BT		the BT bit indicates that the cause for the debug exception was a

task switch into a task where the debug trap bit in the TSS is

enabled. There is not an enable bit for this condition in DR7.

Note that the bits of DR6 are never cleared by the processor; the debug handler,

before returning, should clear DR6 to avoid false interpretation of DR6 the next

time a debug exception occurs.

DR7 bits

LEN		the four 2-bit LEN fields indicate the length of the breakpoint

for each of the breakpoint registers. The encoding is :-

00	one-byte length

01	two-byte length

10	reserved

11	four-byte length

RWE		the four 2-bit RWE fields indicate the type of access that will

cause the breakpoint exception to be raised. The encoding is :-

00	instruction execution only

01	data writes only

10	reserved

10	data reads and writes

GE/LE		the GE/LE bits indicated exact data breakpoints (global and

local). If GE or LE is set, the processor will slow execution

such that data breakpoints are reported on exactly the

instruction that causes them. If these bits are not set, the

processor may get slightly ahead of the reporting of the

breakpoint conditions on instructions that perform data writes

near the end of their execution. It is recommended that LE or GE

be enabled when data breakpoints are disabled. Slowing processor

execution in this way should not cause any problems besides a

slight performance loss.

L0-L3 / G0-G3	these are the local and global enable signals for the four debug

breakpoint registers. If they are set, the breakpoint specified

by the corresponding breakpoint register is enabled.

GD		this enables the debug register protection condition that is

flagged by DR6. Note that GD is cleared at entry to the debug

exception handler by the processor. This allows the handler free

access to the debug registers.

������������������������������������������������������������������������������ͻ

�                         Acknowledgments, Disclaimer                          �

������������������������������������������������������������������������������ͼ

Reference material used :-

80386 CPU enhancements, by Daniel A. Norton.

Assembler Programming Topics, Copyright 1991 David Jurgens.

The PMODE DOS extender is by TRAN, aka Thomas Pytel.

TASM is the copyright of Borland International.

MASM is the copyright of Microsoft Corporation.

All other copyrights are acknowledged.

This file is part of the CrossFire package. Feel free to modify this file for

your own use, but if you distribute it, please distribute the whole unmodified

demonstration package (do NOT distribute the registered version). Add files if

you like, but try to keep them in the 'EXTRA' directory.

Disclaimer

To the best of my knowledge the information contained in this file is accurate.

However, the user takes full liability for any damage caused by the use or

misuse of software, documentation and information in the CrossFire package.