April 27, 1995

Users manual for 6811 monitor programs version 3.0

HARDWARE

You will need the following equipment to use this software:

1.  PC computer (8088 or better) with a 9600 baud

serial port COM1.

2.  A 68HC811E2 microcontroller with RS232

level shifters on the SCI pins

or

Any 68HC11 microcontroller running in the

expanded mode with RAM at BF00 - BFFF and

FFC0 - FFFF and RS232 level shifters on the

SCI pins.

3.  An RS232 connection. Pin 2 to RxD,

pin 3 to TxD, pin 4 to pin 5, pin 6

to pin 20.  NOTE: RS232 level shifters are

needed between 68HC11 and RS232 cable!

4.  An appropriate power supply.

A schematic for an appropriate circuit is included in the

file 6811emul.dxf.  The memory circuitry is not absolutely

needed if you are using a 68HC811E2, and the 68HC24 port

replacement unit is only needed if ports B and C are to be

used in the expanded mode.

OVERVIEW

The files included in this package provide a complete

debugging system for 68HC11 based microcontroller systems.

The programs permit the user to:

- download 68HC11 programs from a PC into 68HC11 memory

- run and step through those programs while observing

memory and internal registers contents

- change data in any memory location or control register

- program EEPROM or PROM on chip memory.

This package does not include a 68HC11 cross assembler or

a simulation tool.  Several versions of 68HC11 cross assemblers

are in the public domain and availible on the internet.  Try the

simtel archive under the msdos directory crossasm.

When  using this system, the PC program 6811.exe communicates

with a small (179 byte) monitor program running on the 68HC11

through an RS-232 link at 9600 baud.  The monitor program

requires memory on the 68HC11 from BF00 to BFFF or from FF00

to FFFF.  This monitor program can be loaded into the 68HC11

from the Pc program 6811.exe and no other software is needed.

The program is quite easy to use and is intended to be self

explanitory, but it will be necessary for the user to have some

familiarity with the 68HC11 assembly language and 68HC11

architecture.

MODES

The 68HC11 operates in one of four modes: single chip,

expanded, bootstrap, or special test.  The 6811.exe PC

monitor allows you to use any of the four.

Bootstrap:

The bootstrap mode is used for three things:

-booting up the system the first time

-booting up the system when there is no monitor

program in rom (prom or eeprom)

-programming eeprom or eprom memory locations

The PC monitor is capable of changing the 6811

mode from bootstrap to any other mode without

resetting the 68HC11.

Test:

The test mode is used for debugging situations in which

system paramters which are usually unchangeable (config,

hpiro, etc.) need to be changed.  The debugging environment

is most versitile in this mode.  However, there are more

ways for programs to crash in the test mode than in the

single chip or expanded.

With this software, the test mode can only be entered

from the bootstrap mode, and will not operate without

ram from BF00 to BFFF.

Single chip:

Single chip is the mode in which most end applications

will be run.  Programs can be debugged from the single

chip mode, but ONLY if the 68HC11 monitor program is in

the internal memory of a 68HC11 from FF00 to FFBF.

Expanded mode:

The expanded mode is used for normal debugging, and some

end applications where the address and data bus are needed.

When using this software, the system can be reset in

expanded mode only if the monitor program is in the internal

memory of a 68HC11 from FF00 to FFBF.  Somewhat more

versitility is availible if there is RAM from BF00 - BFBF

and from FFC0 - FFFF.  To debug in expanded mode this way,

the monitor program will prompt a reset in bootstrap mode.

NOTE:

If the expanded mode is entered from the bootstrap

or special test mode, the EEPROM of an E2 controller

will be remapped to addresses 7800 - 7FFF.

The 68HC811E2 version of the 68HC11 has 2k of EEPROM at

addresses F800 - FFFF.  The monitor program 6811rom.s19

can be loaded into the EEPROM from the bootstrap mode,

and run from either the expanded or single chip mode.

With any other verson of the controller, there is no RAM

or EEPROM in the internal memory map from FFC0 to FFFF

(vector table), and so the user must provide this memory

using external RAM.  This requires that the debugging be

done in either expanded or special test mode.

68HC11 limitations set by this software:

68HC11 resources that are used by the monitor program are:

1.  Timer OC5 is used by the single step function.

2.  The SCI is used for communication.

3.  Memory from BF00 - BFBF or from FF00 - FFBF

are used by the monitor program, depending on

mode of operation.

System parameters that are set up by the monitor program are:

1.  COP timer enabled with 262mS timeout.

2.  Main timer period 500nS.

3.  SCI baud rate 9600, 8 bits, one stop bit,

SCI interrupt enabled.

3.  System registers are remapped to addresses

0000 - 003f by default.  These defaults can

be changed if the in a system with appropriate

ram.

4.  CONFIG = FB or 7B for 68HC811E2, or 0B for others

5.  Parameters are passed between the program 6811.exe

and the monitor program on the 68HC11 user stack.

You will need to allocate 9 extra bytes of length

when you allocate stack space.

TO GET STARTED:

minimum system using 68HC811E2 connected to a PC with an

RS 232 cable:

1.      Run the monitor 6811.exe. The first time you do this

you will be prompted for a mode.  Choose the bootstrap

mode.

2.      Load the program 6811prom.s19.  The bootstrap monitor

will need to be configured to program eeprom.

3.      Change modes to either single chip or expanded mode.

4.      On all future resets, you may reset in single chip or

expanded mode.

system containing ram from BF00 - BF00 or FF00 to FFFF connected

to a PC with an RS 232 cable:

1.      Run the monitor 6811.exe.  Chose any mode desired.

RESOURCES

The following resources might be helpful in settin up a 68HC11

debugging system.  I do not make any claims for the businesses

represented:

CGN company produces a small circuit board that includes

a plcc socket for a 52 pin 68HC11, as well as the minimum

circuitry necessary to run it.  This circuitry includes

crystal, reset circuitry, mode jumpers, and rs232 level

shifters (MAX232).  The reason that this board is handy is

because the board takes each of the 68HC11 pins out to a

wire wrap pin on .1 inch centers.  This allows ease of use

in wirewrap applications on vectorboard.

CGN supplies several versions of this board, each including

or not including the rs232 shifters and address and data latches

for expanded mode operation.  The version that I uses and

was happy with was CGN 1001 - 232, which contains level shifters,

but no bus expansion circuitry. CGN also sells the 68HC811E2

version of the controller at a reasonable price.

CGN also sells a Bootloader program for programming EEPROM on the

6811, but from my experience I do not recommend it.

CGN's address:

CGN

1000 Chula Vista Terrace

Sunnyvale, CA 94086

(408) 720 - 1848

============

Motorola provides a university support service.  They are

willing to send databooks to schools at no charge.

Motorola

University Support

Mail Code 56-106

PO Box 52073

Phoenix, AZ 85072

Phone: 800-545-8731

============

Newark electronics is a Motorola distributor, and stocks

68HC11 products.  You can get a catalog from Newark if you

call (800) 298 - 3133  ext 48.

============

The 68HC11 is usually supplied in a PLCC package.  I have seen

PLCC to wire wrap board adapters advertised by JDR Microdevices

in their components catalog.

(800) 538 - 5000  (sales rep)

(408) 494 - 1430  BBS