Tandy DAC package

DAC Divider Values - Experimental Results

Contents

--------

1.  Introduction

2.  Recording Divider vs. Playback Divider

3.  Recording Divider vs. Sampling Rate

4.  Sampling Rate vs. Recording Divider

5.  Playback Divider vs. Sampling Rate

Introduction

------------

While working with Recwav, I found that Playwav would not play back at the

same rate as the music was supposedly recorded.  Part of the problem is due

to the fact that there are not many recording dividers to choose from at

high sampling rates, but the problem also happened at lower rates as well.

I needed to be certain of the sampling rates to record music samples

accurately for mixing (i.e., to make chord samples).  Hoping to get to the

root of the matter, I performed an investigation, and these are the

results.

Recording Divider vs. Playback Divider

--------------------------------------

The following table was obtained using Testdiv (included with this pack-

age).  The procedure was to record a clarinet sample (the closest thing to

a pure tone on my keyboard) with each recording divider, then play back

with various playback dividers until the playback pitch matched the

keyboard pitch as nearly as possible.

A least-squares line was found, where the independent variable was the

recording divider and the dependent variable was the playback divider.

This line was used to find the computed values below:

playback_divider = (10.00302391 * recording_divider) + 12.477531646

Recording    Playback     Playback div-          Residual

divider     divider     ider (computed)    (computed - actual)

---------    --------    ---------------    -------------------

1           21            22.481               1.481

2           31            32.484               1.484

3           42            42.487               0.487

4           57            52.490              -4.510

5           60            62.493               2.493

6           71            72.496               1.496

7           85            82.499              -2.501

8           92            92.502               0.502

9          102           102.505               0.505

10          114           112.508              -1.492

11          121           122.511               1.511

12          133           132.514              -0.486

13          143           142.517              -0.483

14          152           152.520               0.520

15          167           162.523              -4.477

16          171           172.526               1.526

17          181           182.529               1.529

18          190           192.532               2.532

19          200           202.535               2.535

20          211           212.538               1.538

21          228           222.541              -5.459

22          231           232.544               1.544

23          242           242.547               0.547

24          253           252.550              -0.450

25          262           262.553               0.553

26          273           272.556              -0.444

27          285           282.559              -2.441

28          291           292.562               1.562

29          303           302.565              -0.435

30          313           312.568              -0.432

31          323           322.571              -0.429

32          336           332.574              -3.426

33          342           342.577               0.577

34          352           352.580               0.580

35          361           362.583               1.583

36          371           372.586               1.586

37          380           382.589               2.589

38          399           392.592              -6.408

39          400           402.595               2.595

40          412           412.598               0.598

41          422           422.602               0.602

42          432           432.605               0.605

43          442           442.608               0.608

44          456           452.611              -3.389

45          461           462.614               1.614

46          471           472.617               1.617

47          485           482.620              -2.380

48          493           492.623              -0.377

49          504           502.626              -1.374

50          513           512.629              -0.371

51          522           522.632               0.632

52          532           532.635               0.635

53          542           542.638               0.638

54          551           552.641               1.641

55          570           562.644              -7.356

56          570           572.647               2.647

57          582           582.650               0.650

58          591           592.653               1.653

59          601           602.656               1.656

60          612           612.659               0.659

61          627           622.662              -4.338

62          631           632.665               1.665

63          641           642.668               1.668

64          655           652.671              -2.329

65          662           662.674               0.674

66          672           672.677               0.677

67          684           682.680              -1.320

68          691           692.683               1.683

69          703           702.686              -0.314

70          713           712.689              -0.311

71          722           722.692               0.692

72          738           732.695              -5.305

73          741           742.698               1.698

74          752           752.701               0.701

75          760           762.704               2.704

76          771           772.707               1.707

77          781           782.710               1.710

78          798           792.713              -5.287

79          801           802.716               1.716

80          812           812.719               0.719

Recording Divider vs. Sampling Rate

-----------------------------------

The actual sampling rate for each recording divider was measured with a

special program, Testrec.  Testrec filled a 65535-byte buffer with sound

twice and measured the elapsed time using the PC's programmable interval

timer.

A least-squares line was found, where the independent variable was the

recording divider and the dependent variable was the reciprocal of the

sampling rate.  This line was used to find the computed values below:

1

------------- = (2.793150E-06 * recording_divider) + 3.575443E-06

sampling_rate

Recording    Sampling rate,    Sampling rate,         Residual

divider      Hz (actual)      Hz (computed)     (computed - actual)

---------    --------------    --------------    -------------------

1          156052.044        157020.522            968.478

2          115784.421        109149.516          -6634.905

3           83690.555         83647.743            -42.812

4           62776.794         67805.594           5028.801

5           58902.894         57008.655          -1894.239

6           50223.874         49177.877          -1045.997

7           41855.727         43238.575           1382.848

8           38666.527         38579.283            -87.244

9           34934.468         34826.462           -108.006

10           31394.232         31739.031            344.799

11           29507.844         29154.435           -353.409

12           26909.865         26959.083             49.219

13           25116.180         25071.202            -44.977

14           23546.478         23430.425           -116.053

15           21571.257         21991.216            419.959

16           20930.782         20718.582           -212.200

17           19696.155         19585.185           -110.970

18           18779.187         18569.360           -209.827

19           17856.326         17653.715           -202.612

20           16907.821         16824.126            -83.696

21           15698.547         16069.006            370.459

22           15504.791         15378.759           -126.033

23           14775.102         14745.369            -29.734

24           14142.330         14162.088             19.759

25           13651.017         13623.198            -27.819

26           13137.240         13123.815            -13.425

27           12559.070         12659.749            100.679

28           12289.002         12227.382            -61.620

29           11801.303         11823.572             22.269

30           11417.396         11445.582             28.186

31           11081.540         11091.011              9.471

32           10647.172         10757.748            110.576

33           10465.832         10443.929            -21.903

34           10184.170         10147.900            -36.270

35            9915.147          9868.190            -46.957

36            9660.952          9603.486            -57.467

37            9404.320          9352.611            -51.709

38            8970.912          9114.511            143.598

39            8944.488          8888.232            -56.256

40            8683.834          8672.917            -10.917

41            8471.381          8467.787             -3.594

42            8291.352          8272.136            -19.216

43            8098.821          8085.322            -13.499

44            7849.594          7906.759             57.166

45            7762.950          7735.914            -27.036

46            7583.827          7572.295            -11.533

47            7387.860          7415.454             27.594

48            7257.915          7264.978              7.063

49            7098.064          7120.488             22.424

50            6977.443          6981.633              4.190

51            6860.313          6848.090            -12.223

52            6728.250          6719.560             -8.690

53            6610.217          6595.766            -14.451

54            6496.240          6476.450            -19.790

55            6279.715          6361.375             81.660

56            6279.698          6250.318            -29.381

57            6153.434          6143.071            -10.362

58            6054.257          6039.444            -14.814

59            5955.558          5939.254            -16.305

60            5841.633          5842.334              0.701

61            5708.848          5748.527             39.678

62            5675.196          5657.684            -17.512

63            5581.973          5569.668            -12.305

64            5460.662          5484.348             23.686

65            5406.101          5401.603             -4.498

66            5323.577          5321.318             -2.259

67            5233.126          5243.384             10.258

68            5177.712          5167.700            -10.012

69            5091.690          5094.170              2.480

70            5023.805          5022.703             -1.102

71            4957.695          4953.214             -4.481

72            4852.686          4885.621             32.935

73            4830.587          4819.848            -10.739

74            4759.720          4755.822             -3.898

75            4709.760          4693.476            -16.285

76            4644.917          4632.742            -12.175

77            4581.730          4573.561             -8.170

78            4485.554          4515.872             30.318

79            4470.783          4459.621            -11.162

80            4406.844          4404.754             -2.090

Sampling Rate vs. Recording Divider

-----------------------------------

This uses the same data as for recording divider vs. sampling rate.  In

this case, the reciprocal of the sampling rate is taken as the independent

variable.  The least-squares line is:

3.579865E+05

recording_divider := ------------- - 1.276329

sampling_rate

Recording    Sampling    Recording     Residual

divider       rate       divider      (computed

(actual)       (Hz)      (computed)    - actual)

---------    --------    ----------    ---------

1       156052.044     1.017691     0.017691

2       115784.421     1.815507    -0.184493

3        83690.555     3.001173     0.001173

4        62776.794     4.426200     0.426200

5        58902.894     4.801242    -0.198758

6        50223.874     5.851487    -0.148513

7        41855.727     7.276540     0.276540

8        38666.527     7.981977    -0.018023

9        34934.468     8.971044    -0.028956

10        31394.232    10.126611     0.126611

11        29507.844    10.855582    -0.144418

12        26909.865    12.026842     0.026842

13        25116.180    12.976895    -0.023105

14        23546.478    13.927072    -0.072928

15        21571.257    15.319206     0.319206

16        20930.782    15.827024    -0.172976

17        19696.155    16.899125    -0.100875

18        18779.187    17.786613    -0.213387

19        17856.326    18.771835    -0.228165

20        16907.821    19.896509    -0.103491

21        15698.547    21.527473     0.527473

22        15504.791    21.812441    -0.187559

23        14775.102    22.952712    -0.047288

24        14142.330    24.036795     0.036795

25        13651.017    24.947838    -0.052162

26        13137.240    25.973425    -0.026575

27        12559.070    27.227896     0.227896

28        12289.002    27.854316    -0.145684

29        11801.303    29.058165     0.058165

30        11417.396    30.078155     0.078155

31        11081.540    31.028437     0.028437

32        10647.172    32.346361     0.346361

33        10465.832    32.928936    -0.071064

34        10184.170    33.874946    -0.125054

35         9915.147    34.828689    -0.171311

36         9660.952    35.778669    -0.221331

37         9404.320    36.789853    -0.210147

38         8970.912    38.628929     0.628929

39         8944.488    38.746818    -0.253182

40         8683.834    39.948153    -0.051847

41         8471.381    40.982018    -0.017982

42         8291.352    41.899568    -0.100432

43         8098.821    42.925977    -0.074023

44         7849.594    44.329413     0.329413

45         7762.950    44.838429    -0.161571

46         7583.827    45.927617    -0.072383

47         7387.860    47.179727     0.179727

48         7257.915    48.047280     0.047280

49         7098.064    49.158066     0.158066

50         6977.443    50.029939     0.029939

51         6860.313    50.905920    -0.094080

52         6728.250    51.930160    -0.069840

53         6610.217    52.880223    -0.119777

54         6496.240    53.830404    -0.169596

55         6279.715    55.730488     0.730488

56         6279.698    55.730642    -0.269358

57         6153.434    56.900384    -0.099616

58         6054.257    57.853398    -0.146602

59         5955.558    58.833330    -0.166670

60         5841.633    60.005604     0.005604

61         5708.848    61.430992     0.430992

62         5675.196    61.802825    -0.197175

63         5581.973    62.856293    -0.143707

64         5460.662    64.281027     0.281027

65         5406.101    64.942664    -0.057336

66         5323.577    65.969164    -0.030836

67         5233.126    67.131457     0.131457

68         5177.712    67.863585    -0.136415

69         5091.690    69.031675     0.031675

70         5023.805    69.981724    -0.018276

71         4957.695    70.931938    -0.068062

72         4852.686    72.494478     0.494478

73         4830.587    72.831965    -0.168035

74         4759.720    73.935356    -0.064644

75         4709.760    74.733184    -0.266816

76         4644.917    75.794276    -0.205724

77         4581.730    76.857163    -0.142837

78         4485.554    78.532444     0.532444

79         4470.783    78.796124    -0.203876

80         4406.844    79.957897    -0.042103

Playback Divider vs. Sampling Rate

----------------------------------

The actual sampling rate for each playback divider was measured with a

special program, Testplay.  Testplay played a 65535-byte buffer (at zero

volume) twice and measured the elapsed time using the PC's programmable

interval timer.

The computed values below are based on the formula:

3579545

sampling_rate = ----------------

playback_divider

Playback    Sampling rate,    Sampling rate,         Residual

divider      Hz (actual)      Hz (computed)     (computed - actual)

--------    --------------    --------------    -------------------

20         166411.322        178977.250           12565.928

30         119231.326        119318.167              86.841

40          89441.514         89488.625              47.111

50          71560.389         71590.900              30.511

60          59638.192         59659.083              20.891

70          51121.478         51136.357              14.879

80          44732.986         44744.312              11.327

90          39763.854         39772.722               8.868

100          35788.358         35795.450               7.092

110          32535.343         32541.318               5.975

120          29825.073         29829.542               4.469

130          27530.975         27534.962               3.986

140          25564.775         25568.179               3.403

150          23860.687         23863.633               2.946

160          22369.593         22372.156               2.563

170          21053.735         21056.147               2.412

180          19884.625         19886.361               1.737

190          18838.154         18839.711               1.556

200          17896.251         17897.725               1.474

210          17044.185         17045.452               1.268

220          16269.480         16270.659               1.179

230          15562.153         15563.239               1.086

240          14913.816         14914.771               0.954

250          14317.222         14318.180               0.958

260          13766.662         13767.481               0.819

270          13256.820         13257.574               0.754

280          12783.452         12784.089               0.637

290          12342.661         12343.259               0.598

300          11931.182         11931.817               0.635

310          11546.377         11546.919               0.542

320          11185.496         11186.078               0.582

330          10846.618         10847.106               0.488

340          10527.609         10528.074               0.465

350          10226.849         10227.271               0.422

360           9942.834          9943.181               0.347

370           9674.123          9674.446               0.323

380           9419.519          9419.855               0.337

390           9178.013          9178.321               0.307

400           8948.522          8948.863               0.341

410           8730.307          8730.598               0.291

420           8522.435          8522.726               0.291

430           8324.267          8324.523               0.256

440           8135.031          8135.330               0.299

450           7954.316          7954.544               0.228

460           7781.375          7781.620               0.244

470           7615.824          7616.053               0.229

480           7457.141          7457.385               0.244

490           7304.956          7305.194               0.238

500           7158.888          7159.090               0.202

510           7018.520          7018.716               0.196

520           6883.533          6883.740               0.207

530           6753.706          6753.858               0.152

540           6628.643          6628.787               0.144

550           6508.103          6508.264               0.160

560           6391.910          6392.045               0.135

570           6279.754          6279.904               0.149

580           6171.495          6171.629               0.134

590           6066.915          6067.025               0.110

600           5965.805          5965.908               0.103

610           5868.001          5868.107               0.105

620           5773.360          5773.460               0.100

630           5681.693          5681.817               0.124

640           5592.918          5593.039               0.121

650           5506.903          5506.992               0.089

660           5423.472          5423.553               0.081

670           5342.525          5342.604               0.080

680           5263.932          5264.037               0.105

690           5187.645          5187.746               0.101

700           5113.561          5113.636               0.074

710           5041.546          5041.613               0.067

720           4971.526          4971.590               0.065

730           4903.407          4903.486               0.079

740           4837.143          4837.223               0.080

750           4772.662          4772.727               0.065

760           4709.845          4709.928               0.082

770           4648.698          4648.760               0.062

780           4589.080          4589.160               0.080

790           4531.004          4531.070               0.066

800           4474.374          4474.431               0.057

810           4419.125          4419.191               0.066

820           4365.227          4365.299               0.072

830           4312.635          4312.705               0.070

840           4261.316          4261.363               0.048

850           4211.168          4211.229               0.062

860           4162.186          4162.262               0.076

870           4114.361          4114.420               0.059

880           4067.606          4067.665               0.059

890           4021.915          4021.961               0.046

900           3977.222          3977.272               0.050