FLOATPT.DOC -- Floating Point Conversion Procedures

===================================================

From `BLUEBOOK of ASSEMBLY ROUTINES for the IBM PC & XT'

by Christopher L. Morgan

Copyright (C) 1984 by The Waite Group, Inc.

Purpose: These procedures perform input, conversion, and output on

floating point numbers.

Contents:

---------

BIN802DEC	--  Convert from 80-bit binary to decimal digits

DECDOUBLE	--  Double a temporary decimal floating point number

DECHALF	--  Halve a temporary decimal floating point number

DECNORM	--  Normalize temporary decimal floating point

DFP2SFP	--  Convert from double to single precision floating point

FIX		--  Convert from floating point to 16-bit integer

FLOAT	--  Convert from 16-bit integer to floating point

FPIN		--  Convert from external to internal floating point

FPINDIGIT	--  Convert from decimal digit to temporary floating point

FPOUT	--  Convert from internal to external floating point

FPTDIV	--  Divide temporary floating point by 10

FPTMUL	--  Multiply temporary floating point by 10

FPTNORM	--  Normalize temporary floating point

SFP2DFP	--  Convert from single to double precision floating point

SFP2TFP	--  Convert from single precision to temporary floating point

SGNDEC16IN	--  Convert from ASCII signed decimal to binary

TDECSHOW	--  Display floating point

TFP2SFP	--  Convert from temporary floating point to single precision

_____________________________________________________________________________

Overview

__________________________ FLOATING POINT ROUTINES __________________________

-----------------------------------------------------------------------------

Conversion from 80-bit binary to decimal digits

BIN802DEC

Function: This routine converts an 80-bit binary integer to a decimal string.

Input: Upon entry, SI points to an 80-bit binary integer to be used as input

and DI points to a 25-digit decimal string to be used as output.

Output: Upon exit, a 25-digit decimal string is where DI pointed upon entry.

DI still points there upon exit.

Registers used: AX, BX, CX, DX, and SI are modified; DI points to output.

Segments referenced: The data segment contains storage for the 80-bit binary

number (10 bytes) and the 25-digit decimal number (25 bytes).

Note: This is a near procedure needed by FPOUT.

-----------------------------------------------------------------------------

Double a temporary decimal floating point number

DECDOUBLE

Function: This routine multiplies a temporary decimal floating point

number by two.

Input: Upon entry, DS:DI points to a temporary decimal floating point number.

Output: Upon exit, the number has been doubled.

Registers used: AX,CX, & DX are modified.

Segments referenced: The data segment contains storage for the temporary

decimal floating point number.

Note: This is a near procedure needed by FPOUT.

-----------------------------------------------------------------------------

Half a temporary decimal floating point number

DECHALF

Function: This routine divides a temporary decimal floating number by two.

Input: Upon entry, DI points to a temporary decimal floating point number.

Output: Upon exit, the number has been divided by two. The result is not

normalized.

Registers used: AX, CX, and DI are modified.

Segments referenced: The data segment contains a temporary decimal floating

point number.

Note: This is a near procedure needed by FPOUT.

-----------------------------------------------------------------------------

Normalize a temporary decimal floating point number

DECNORM

Function: This routine normalizes a temporary decimal floating number.

Input: Upon entry, DI points to a temporary decimal floating point number.

Output: Upon exit, the number is normalized.

Registers used: AX, CX, and DI are modified.

Segments referenced: The data segment contains a temporary decimal floating

point number.

Note: Equates are used to shorten address fields. This is a near procedure

needed by FPOUT.

-----------------------------------------------------------------------------

Convert from double to single precision floating point

DFP2SFP

Function: This routine converts an internal double precision binary floating

point number to an internal single precision floating point number.

Input: Upon entry, a double precision binary floating point number is in

DFPBUFF. The double precision floating point number has a 40-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Output: Upon exit, a single precision binary floating point number is in

SFPBUFF. The single precision floating point number has a 24-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Registers used: Unmodified; AX is saved and restored.

Segments referenced: The data segment contains storage for the variables,

SFPBUFF and DFPBUFF.

Note: Equates are used to shorten address fields.

-----------------------------------------------------------------------------

Convert from floating point to 16-bit integer

FIX

Function: This routine converts from internal single precision binary

floating point to internal 16-bit signed two's complement integer.

Input: Upon entry, a single precision binary floating point is in SFPBUFF.

The single precision floating point number has a 24-bit binary mantissa,

a sign bit, and an 8-bit exponent biased by 128.

Output: Upon exit, a 16-bit signed two's complement binary number is in DX.

Registers used: DX is used for output; CX,and AX are saved and then restored.

Segments referenced: The data segment contains storage for SFPBUFF.

Note: Equates are used to shorten address fields.

-----------------------------------------------------------------------------

Convert from 16-bit integer to floating point

FLOAT

Function: This routine converts an unsigned 16-bit binary number to a

single precision binary floating point number.

Input: Upon entry, DX contains an unsigned 16-bit binary number.

Output: Upon exit, SFPBUFF contains a single precision floating point

number. The single precision floating point number has a 24-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Registers used: Unmodified; DX is used for input; DX, CX, & AX are saved

and then restored.

Segments referenced: The data segment contains storage for the variable,

SFPBUFF and the message, INTERNAL.

Routines called: STDMSG_OUT,HEX16OUT,STD_SPACE (all for debugging).

Note: Equates are used to shorten address fields.

-----------------------------------------------------------------------------

Convert from external to internal floating point

FPIN

Function: This routine accepts an ASCII decimal floating point number from

a standard input device and converts it to internal binary floating point.

Input: The characters of the floating point number are received in ASCII

through a call to a standard input routine. The decimal floating point

number has an optional sign, followed by decimal digits of the mantissa

with one embedded decimal point. Following the mantissa is an optional

exponent starting with the letter, 'E', then an optional sign, then a

decimal number. It is possible to get erroneous results if the number is

too large or small to be stored as a single precision binary floating

point number.

Output: Upon exit a single precision binary floating point number is in

SFPBUFF. The single precision floating point number has a 24-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Registers used: Unmodified; DI, SI, DX, CX, & AX are saved and restored.

Segments referenced: The data segment contains the variables, FPTEMP1,

FPTEMP2, and SFPBUFF.

Routines called: STD_IN, FPINDIGIT, FPMUL, FPTDIV, and FPTNORM.

Note: Equates are used to shorten address fields

-----------------------------------------------------------------------------

Convert an input single decimal digit to temporary floating point

FPINDIGIT

Function: A single decimal digit is placed in temporary binary floating

point format.

Input: Upon entry, AL contains the value of the digit.

Output: Upon entry and exit, the address of the temporary binary floating

point result is in DI.

Registers used: AS, CX, and DI are modified.

Segments referenced: Upon entry, the data segment contains storage for the

temporary binary floating point number.

Note: This is a near procedure needed by FPIN.

-----------------------------------------------------------------------------

Convert from internal to external floating point

FPOUT

Function: This routine displays a single precision floating point number

on a standard output device as a decimal floating point number.

Input: Upon entry, a single precision binary floating point number is in

SFPBUFF. The single precision floating point number has a 24-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Output: The individual characters of a decimal floating number are sent out

through the standard output device. The decimal floating point number has

a sign character (blank or minus), followed by decimal digits of the

mantissa with one embedded decimal point to the right of the first

significant digit. Following the mantissa is an exponent that starts with

the letter, 'E', then a sign, then a decimal number.

Registers used: AX,CX,BX,DX,SI,& DI are modified.

Segments referenced: The data segment contains storage for the variables,

DECBUFF, DECSIGN, DECEXP, FPTEMP1, and SFPBUFF.

Routines called: STDOUT, SFP2TFP, DECHALF, DECDOUBLE, DECNORM, & TDECSHOW.

Note: Equates are used to shorten address fields.

-----------------------------------------------------------------------------

Divide temporary floating point by 10

FPTDIV

Function: This routine divides a temp binary floating point number by 10.

Input: Upon entry, DS:DI points to a temporary binary floating point number.

Output: Upon exit, DS:DI points to a temporary binary floating point number.

Registers used: AX, CX, DX, and DI are modified. DI points to input & output.

Segments referenced: The data segment contains a temporary binary floating

point number.

Note: Equates are used to shorten address fields. This routine is needed

by FPIN.

-----------------------------------------------------------------------------

Multiply temporary floating point by 10

FPTMUL

Function: This routine multiplies a temporary binary floating point number

by 10. The result is not normalized.

Input: Upon entry, DS:DI points to a temporary binary floating point number.

Output: Upon exit, DS:DI points to a temporary binary floating point number.

This number is not normalized.

Registers used: AX, CX, DX, and DI are modified. DI points to input & output.

Segments referenced: The data segment contains a temporary binary floating

point number.

Note: This near routine is needed by FPIN.

-----------------------------------------------------------------------------

Normalize temporary floating point

FPTNORM

Function: This routine normalizes temporary binary floating point numbers

that have too large a mantissa.

Input: Upon entry, DS:DI points to a temporary binary floating point number

whose mantissa is too large.

Output: Upon exit, DS:DI points to a normalized temporary binary floating

point number.

Registers used: Unmodified; DI must point to the number.

Segments referenced: The data segment must contain the temporary floating

number.

Note: Equates are used to shorten address fields. This is a near procedure

needed by FPIN.

-----------------------------------------------------------------------------

Convert from single to double precision floating point

SFP2DFP

Function: This routine converts an internal single precision binary floating

point number into an internal double precision floating point number.

Input: Upon entry, a single precision floating point number is stored in

SFPBUFF. The single precision floating point number has a 24-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Output: Upon exit, a double precision floating point number is stored in

DFPBUFF. The double precision floating point number has a 40-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Registers used: Unmodified; AX is saved and restored.

Segments referenced: The data segment contains storage for the variables,

SFPBUFF and DFPBUFF.

Note: Equates are used to shorten address fields.

-----------------------------------------------------------------------------

Convert from single precision to temporary floating point

SFP2TFP

Function: This routine converts a single precision binary floating point

number into a temporary binary floating point number.

Input: Upon entry, a single precision floating point number is stored in

SFPBUFF. The single precision floating point number has a 24-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Output: Upon exit, a temporary binary floating point number is stored in

FPTEMP1. The temporary binary floating point number has a 72-bit binary

mantissa with 8 bits to the left of these for internal use, a sign byte,

and a 16-bit two's complement binary exponent.

Registers used: AX is modified.

Segments referenced: The data segment contains the variables, SFPBUFF and

FPTEMP1.

Note: Equates are used to shorten address fields. This is a near procedure

needed by FPOUT.

-----------------------------------------------------------------------------

Convert from ASCII signed decimal to binary

SGNDEC16IN

Function: This routine accepts a signed decimal number from the standard

input device and converts it to internal signed two's complement

16-bit binary form.

Input: Individual digits of the signed decimal number are received in

ASCII through a call to a standard input routine. The sign (+ or -) is

optional and the valid digits are 0 through 9. An ASCII code other than

that for the sign and the valid digits will terminate the routine.

Output: A signed two's complement 16-bit binary number is returned in DX.

Registers used: AX & CX are modified; DX contains the output.

Routines called: STD_IN

-----------------------------------------------------------------------------

Display floating point

TDECSHOW

Function: This routine displays a floating point number on the standard

output device.

Input: Upon input, a number is stored in temporary decimal floating point

form. The temporary format has a string of 25 decimal digits, a sign byte,

and a base ten exponent that is stored in two's complement 16-bit binary

format.

Output: The individual characters of a floating point number are sent out

the standard output device. The form of the output is: a sign character

that is blank or minus, followed by decimal digits of the mantissa with

one embedded decimal point to the right of the first significant digit.

Following the mantissa is an exponent which starts with the letter E,

then a sign, then a decimal number.

Registers used: AL, CX, DX, & SI are modified.

Segments referenced: The data segment contains the variables, DECBUFF (25

bytes), DECSIGN (1 byte), and DECEXP (2 bytes).

Routines called: STD_OUT

Note: This is a near procedure needed by FPOUT.

-----------------------------------------------------------------------------

Convert from temporary floating point to single precision

TFP2SFP

Function: This routine converts from temporary binary floating point to

single precision floating point.

Input: Upon entry, a number is stored in temporary binary floating point

form in FPTEMP1. The temporary binary floating point number has a 72-bit

binary mantissa with 8 bits to the left of these for internal use, a

sign byte, and a 16-bit two's complement binary exponent.

Temporary binary floating point

_________________________________________________________________

/    /    /    /    /    /    /    /    /    /    /    /    /    /

Bytes   /  0 /  1 /  2 /  3 /  4 /  5 /  6 /  7 /  8 /  9 / 10 / 11 / 12 /

/____/____/____/____/____/____/____/____/____/____/____/____/____/

|                                                 |    |         |

\__________________ Mantissa ___________________/  /   \Exponent/

Sign /    (2's comp)

Single precision binary floating point

________________________

/     /     /     /     /

/  0  /  1  /  2  /  3  /

/_____/_____/_____/_____/

|               ||     |

\_  Mantissa _// \   /

Sign /   \ /

\ Exponent biased by 128

Output: Upon exit, a single precision floating point number is stored in

SFPBUFF. The single precision floating point number has a 24-bit binary

mantissa, a sign bit, and an 8-bit exponent biased by 128.

Registers used: AX and DX are modified.

Segments referenced: Upon entry, the data segment contains the messages:

INTERNAL, UNDERFLOW, & OVERFLOW, and storage for the temporary binary

floating point number, FPTEMP1, and the single precision floating point

number, SFPBUFF.

Routines called: STD_SPACE, EMSG_OUT, & HEX16OUT

Note: Equates are used to shorten fields.  This is a near procedure needed

by FPIN.

-----------------------------------------------------------------------------

______________________________________________________________________________

>>>>> Physical EOF FLOATPT.DOC <<<<<

##################################################################################################################