---------------------------------------------------------------------------

--------------------- CAMVIEW 1.50 DOCUMENTATION FILE ---------------------

---------------------------------------------------------------------------

THE AUTHOR MAKES NO GUARANTEES OR WARRANTIES WITH THIS PROGRAM

AND CLAIMS NO RESPONSIBILITY FOR ANY DAMAGE OR LOSS OF TIME OR

DATA CAUSED BY THIS PROGRAM. THIS PROGRAM IS PROTECTED UNDER

THE COPYRIGHT LAWS OF THE UNITED STATES OF AMERICA.

ALL RIGHTS RESERVED.

--------------------------

- SHAREWARE RELEASE 1.50 -

--------------------------

CAMVIEW is a project which has developed from a need for a quick and easy

way to verify the programs written for the CNC milling machines where I

work. After looking for 6 months for a commercial program which would

accomplish this task and finding nothing available for under $4000.00,

I dug out the programming manuals and went to work. 6 months and 30 or

so revisions later, CAMVIEW 1.01 emerged. This version was written and

tested on a Bridgeport V2XT under the BOSS 10 programming subset. It

assumes the following...

N = Line Numbers

G0 = Rapid Mode

G1 = Linear Interpolation (feed)

G2 = Clockwise Arc

G3 = Counter Clockwise Arc

These commands are assumed to be MODAL (G1 will remain in effect until

it is cancelled by another G code).  I think most controllers at least

have these commands in common so CAMVIEW should work fine under most

conditions. If your controller has different conventions (and you are

a registered user) mail that information to me and I'll customize a copy

for you for a modest fee. Currently, the G2 and G3 commands produce only

STRAIGHT LINES but they are represented by different colors. If you find

any Bugs or have any suggestions, PLEASE drop me a line so that i can

incorporate them in the next release.

---------------------------------------------------------------------------

STARTUP:

Note that the default setup in CAMVIEW reads only files with a .NCC

suffix. There must be at least 1 of these files in the startup directory

or the program will abort. CAMVIEW was written on and for a VGA system.

It has had very little testing on an EGA system but the code is there

to detect that hardware and it should work. If the program hangs under EGA,

you can use CAMVIEW /EGA on the command line to force EGA mode.

To start Camview type

CAMVIEW

and issue a carrige return.

You will then be presented with an opening screen. The input screen will

follow shortly along with a directory listing of all the files in the

directory from which CAMVIEW was started which share the specified suffix.

entering S will take you to the setup configuration screen,

entering Q will take you to the DOS prompt,

Just enter the filename of the desired file and press return....

The desired file will be read and the graphics screen will appear.

The tool paths will be automatically scaled and displayed on the screen.

Here is where the fun part comes in....

F1 will ZOOM IN

F2 will ZOOM OUT

F3 will PAN LEFT

F4 will PAN RIGHT

F5 will PAN UP

F6 will PAN DOWN

S will goto SETUP

Q will quit to DOS prompt.

SPACE will ReRun CAMVIEW

You can isolate any part of the screen and ZOOM in to see exactly

whats going on. If the Z REP options is switched on, a series of

arrows on the left and right side of the screen let you know whats

happening on the Z axis while the program is running.

----------------------------------------------------------------------------

Revision history

9-2-91  R1.01 - Initial shareware release

9-9-92  R1.20 - Fixed bug which cause recursive loop with some files.

Tweaked arrays to allow longer input files.

1-5-94  R1.50   Ported program to new compiler which allows optimization

for the 80386 instruction set.

Provided support for up to 5000 line CNC files.

CAMVIEW now runs around 3x faster under most conditions.

NOTE:

CamView 3D is now in beta testing.  If you would like to participate

in the program, send us a check for $10.00 to cover shipping and

material costs.  Place ATTN: BETA TEST on outside of envelope.

This program is open to registered users only.

---------------------------

- Registration fee $30.00 -

---------------------------

Please support the ShareWare concept and help assure the continued

development of CamView and other fine programs.

Forward all Questions, Comments, and Registrations to below address.

Make all checks payable to Robert Pemberton.

Robert Pemberton

Matrix Softworks

2524 Ashbrooke Dr.

Lexington Ky. 40513

Internet        rapemb00@mik.uky.edu

America Online  RPEMBERTON