This is a brief explanation to help clarify the correct settings for high

speed modem (14.4 or MNP support) modem's.  With these modems you need to

set the modems baud speed HIGHER than the modems highest connection rate.

ie 14.4 modem would be set to 19.2 or higher.  You can set this as high as

you would like, however, there will be a point where you will start to

loose screen characters.  At this point you would need to set the speed

lower.

In configuration are E (Alt-C;E) you should have these settings:

���������������������������� CONFIGURATION AREA E

��������������������������������������������������

[C] Use CTS/RTS flow control:         YES

[F] Auto-detect connect speed:        NO

You baud speed setting should be 19.2 or higher, as well as your dialing

directories.  You can change the dialing entries speed with the "B"

command in the dialing directory.