NAME

E100PKT 0.2, packet driver for DOS

SYNOPSIS

E100PKT <option>

DESCRIPTION

E100PKT is a basic packet driver for Intel's 8255x family of

Ethernet adapters. It should work with any PCI device that

Linux's e100 kernel module (in e100.c) does.

Options are case-insensitive and must begin with either a / or -.

All the program's input and output is in hex without a leading

"0x" (e.g. "60" is 96 decimal).

SYSTEM REQUIREMENTS

* An IBM PC or compatible

* An Intel 80386 or compatible processor

* DOS 2 or higher

* 2560 bytes of free conventional memory

* An Intel E100 Ethernet adapter. That is, a PCI device whose

vendor id is 8086 and device id is one of the following:

1029    103c    1057    1091    27dc

1030    103d    1059    1092

1031    103e    1064    1093

1032    1050    1065    1094

1033    1051    1066    1095

1034    1052    1067    1209

1038    1053    1068    1229

1039    1054    1069    2449

103a    1055    106a    2459

103b    1056    106b    245d

OPTIONS

/h

/?

Show usage.

/v

Show version.

/u

Unload the packet driver from memory.

/i <int_no>

Load the packet driver into memory at the specified software

interrupt. Valid interrupt numbers are: 60-66, 68-6f, or

7b-7e (inclusive).

/d

Dump PCI configuration space to E100.PCI

Dump EEPROM to E100.EEP

Dump CSR to E100.CSR

BUGS

* E100PKT cannot be loaded high.

* E100PKT uses too much memory.

* E100PKT cannot be unloaded if its hardware interrupt vector

has been hooked since loading it.

* Please send any feedback and bug reports to <sethsimon@sdf.org>.

HOME PAGE

gopher://sdf.org/1/users/sethsimon/e100pkt/ has the latest

version and links to technical documentation.

COPYRIGHT

Copyright (C) 2018, Seth Simon <sethsimon@sdf.org>.

E100PKT comes with ABSOLUTELY NO WARRANTY. This is free software

covered by GNU GENERAL PUBLIC LICENSE 3, and you are welcome to

redistribute it under certain conditions. See http://www.gnu.org

or COPYING file for details.