# C104H/PCI Document

## Resource Requirement:

- IRQ * 1

- I/O : 
	UART register :		32 bytes ( 8 bytes / port  *  4port )
	IRQ Vector register : 	1 byte ( 1 bit/port  *  port = 4 bits )

## PCI Configuration

1.MOXA Vendor ID = 0x1393

2.C168H/PCI Device ID = 0x1040

3.Hardware resource on Device Configuration register of PCI configuration space:

- IRQ 	0x3C, BYTE

- Interrupt Vector	0x1C, DWORD 
Bit 0 : 	Always 1
Bit 1 : 	Reserved
Bits 31: 2 : 	Interrupt Vector

- I/O Base Address	0x18, DWORD 
Bit 0 : 	Always 1
Bit 1 : 	Reserved
Bits 31: 2 : 	I/O Base Address

## UART register structure:

(For detailed UART register description, please refer to the TL16C550C data book.)

For Example, If C104H/PCI base address is 0x180:

1st port’s UART register I/O address is 0x180 .. 0x187,
		1st register I/O address is 0x180,
		2nd register I/O address is 0x181, …..

2nd port’s UART register I/O address is 0x188.. 0x18f,

1st register I/O register is 0x188,
		2nd register I/O register is 0x189, …..

……

## IRQ Vecter register structure:

(C104H/PCI only uses 4 bits.)

| Bit Value | Status |
| --- | --- |
| 0 | Interrupt pending. Please read the UART register to get the detail interrupt information *. |
| 1 | No interrupt pending. |