# MICR Font Calibration Tool

Calibration is a two-step process that ensures the MICR is printed at the correct width and intensity. Print this document on the printer that is to be used to print checks. The MICR toner and check stock used to print the checks should also be loaded in the printer for accurate results.

## Step 1; verify the width:

Measure the distance between the middle of the “1” characters. The distance should be as close to 4 inches (10.16 CM) as possible. Using this calibration method, the allowable tolerance is +/- 1/8 of an inch (.32 CM); which is between 3 7/8" and 4 1/8" (9.84 CM and 10.48 CM).

IDAutomationSMICR N2 (Narrow by 2 pixels at 600dpi; 1 pixel at 300 dpi)

1-000222333444--667777888999000-1

IDAutomationSMICR N1 (Narrow by 1 pixel at 600dpi)

1-000222333444—-667777888999000-1

IDAutomationSMICR  (Normal width for printers in alignment)

1-000222333444—-667777888999000-1

IDAutomationSMICR W1 (Wide by 1 pixel at 600dpi)

1-000222333444--667777888999000-1

IDAutomationSMICR W2 (Wide by 2 pixels at 600dpi; 1 pixel at 300 dpi)

1=000222333444--667777888999000=1

## Step 2; verify the intensity:

Select the font from the intensity column that is closest to the intensity of a pre-printed bank check printed by a banking institution. Some banks also provide a printing and layout gauge that should be used if it is available. Use the font name that matches the appropriate row of intensity and column of width.

| Intensity | Narrow 2 | Narrow 1 | Standard Width | Wide 1 | Wide 2 |
| --- | --- | --- | --- | --- | --- |
| Extra Light 02346780 | IDAutomationSMICR N2XL | IDAutomationSMICR N1XL | IDAutomationSMICR XL | IDAutomationSMICR W1XL | IDAutomationSMICR W2XL |
| Light 02346780 | IDAutomationSMICR N2L | IDAutomationSMICR N1L | IDAutomationSMICR L | IDAutomationSMICR W1L | IDAutomationSMICR W2L |
| Normal 02346780 | IDAutomationSMICR N2 | IDAutomationSMICR N1 | IDAutomationSMICR | IDAutomationSMICR W1 | IDAutomationSMICR W2 |
| Bold 02346780 | IDAutomationSMICR N2B | IDAutomationSMICR N1B | IDAutomationSMICR B | IDAutomationSMICR W1B | IDAutomationSMICR W2B |
| Extra Bold 02346780 | IDAutomationSMICR N2XB | IDAutomationSMICR N1XB | IDAutomationSMICR XB | IDAutomationSMICR W1XB | IDAutomationSMICR W2XB |

Copyright © 2006 IDAutomation.com, Inc.