\title{PLIP: The Parallel Line Internet Protocol Device}

\author{ Donald Becker (becker@super.org)}

\affiliation{I.D.A. Supercomputing Research Center, Bowie MD 20715}

%% At some point T. Thorn will probably contribute text,

%% \author{ Tommy Thorn (tthorn@daimi.aau.dk)}

\section{PLIP Introduction}

This document describes the parallel port packet pusher for Net/LGX.

This device interface allows a point-to-point connection between two

parallel ports to appear as a IP network interface.

\chapter{PLIP hardware interconnection}

PLIP uses several different data transfer methods.  The first (and the

only one implemented in the early version of the code) uses a standard

printer "null" cable to transfers data four bits at a time using

data bit outputs connected to status bit inputs.

The second data transfer method relies on both machines having

bi-directional parallel ports, rather than output-only ``printer''

ports.  This allows byte-wide transfers and avoids reconstructing

nibbles into bytes, leading to much faster transfers.

\section{Parallel Transfer Mode 0 Cable}

The cable for the first transfer mode is a standard

printer "null" cable which transfers data four bits at a time using

data bit outputs of the first port (machine T) connected to the

status bit inputs of the second port (machine R).  There are five

status inputs, and they are used as four data inputs and a clock (data

strobe) input, arranged so that the data input bits appear as contiguous

bits with standard status register implementation.

A cable that implements this protocol is available commercially as a

"Null Printer" or "Turbo Laplink" cable.  It can be constructed with

two DB-25 male connectors symmetrically connected as follows:

STROBE output	1*

D0->ERROR	2 - 15		15 - 2

D1->SLCT	3 - 13		13 - 3

D2->PAPOUT	4 - 12		12 - 4

D3->ACK	5 - 10		10 - 5

D4->BUSY	6 - 11		11 - 6

D5,D6,D7 are   7*, 8*, 9*

AUTOFD output 14*

INIT   output 16*

SLCTIN	17 - 17

extra grounds are 18*,19*,20*,21*,22*,23*,24*

GROUND	25 - 25

* Do not connect these pins on either end

If the cable you are using has a metallic shield it should be

connected to the metallic DB-25 shell at one end only.

\section{Parallel Transfer Mode 1}

The second data transfer method relies on both machines having

bi-directional parallel ports, rather than output-only ``printer''

ports.  This allows byte-wide transfers, and avoids reconstructing

nibbles into bytes.  This cable should not be used on unidirectional

``printer'' (as opposed to ``parallel'') ports or when the machine

isn't configured for PLIP, as it will result in output driver

conflicts and the (unlikely) possibility of damage.

The cable for this tranfer mode should be constructed as follows:

STROBE->BUSY 1 - 11

D0->D0	2 - 2

D1->D1	3 - 3

D2->D2	4 - 4

D3->D3	5 - 5

D4->D4	6 - 6

D5->D5	7 - 7

D6->D6	8 - 8

D7->D7	9 - 9

INIT -> ACK  16 - 10

AUTOFD->PAPOUT 14 - 12

SLCT->SLCTIN 13 - 17

GND->ERROR	18 - 15

extra grounds are 19*,20*,21*,22*,23*,24*

GROUND	25 - 25

* Do not connect these pins on either end

Once again, if the cable you are using has a metallic shield it should

be connected to the metallic DB-25 shell at one end only.

\section{PLIP Mode 0 tranfer protocol}

The PLIP driver is compatible with the "Crynwr" parallel port transfer

standard in Mode 0.  That standard specifies the following protocol:

send header nibble '8'

count-low octet

count-high octet

... data octets

checksum octet

Each octet is sent as

<wait for rx. '1'>	<send 0x10+(octet&0x0F)>

<wait for rx. '0'>	<send 0x00+((octet>>4)&0x0F)>

To start a transfer the transmitting machine outputs a nibble 0x08.

The raises the ACK line, triggering an interrupt in the receiving

machine.  The receiving machine disables

Restated:

(OUT is bit 0-4, OUT.j is bit j from OUT. IN likewise)

Send_Byte:

OUT := low nibble, OUT.4 := 1

WAIT FOR IN.4 = 1

OUT := high nibble, OUT.4 := 0

WAIT FOR IN.4 = 0