PRESS [arrow keys] to scroll through this document:

Zeiss ETH2: (per manual received 08-01-94)

Keys are @ theodolite unless otherwise indicated;

Displays refer to instrument display window(s) unless otherwise indicated.

The ETH2 is equiped with a non-volatile memory which should be good for

at least 10 years from the initial installation of the internal lithium

battery.  The following items are stored in this memory:

_compensator adj. status

_index correction

_collimation correction

_angle measurement unit

_vertical reference system

_main operating position

The following should be executed at initial set-up of instrument(s):

1a. Battery life:

a. One fully-charged battery pack on instrument will last ~8 hours

without illumination.

1b. Rough-Plumb instrument using visual vials on instrument.

2. Turn ON/OFF instrument:

a. ON-->  press [ENTER]

b. OFF--> press [MODE+]+[MODE-]

3. Select Horizontal Circle direction:

a. press [MODE+] until {U- 1:} displayed.

b. press [ENTER]

c. see displayed: {-, +} in alternation.

d. press [ENTER] @ {+}

4. Select Vertical Circle orientation & direction:

a. press [MODE+] until  {U- 2:} displayed.

b. press [ENTER]

c. see displayed: {ZENITH, VERTIC, +-ELEV., SLOPE%} in alternation.

d. press [ENTER] @ {ZENITH}

e. see displayed: {POS.1 RIGHT, POS.1 LEFT} in alternation.

f. press [ENTER] @ {RIGHT}

g. see displayed: {ZENITH  DISPL. R}

5. Select Circle Angle Units:

a. press [MODE+] until {U- 3:} displayed.

b. press [ENTER]

c. see displayed: {D.M.S., GON, MIL} in alternation.

d. press [ENTER] @ {GON}

e. see displayed: {UNITS.A.V. GON}

6. Select Instrument combination:

a. press [MODE+] until {U- 8} displayed.

b. press [ENTER]

c. see displayed: {ETH2, ETH2 ELDI 4, ETH2 ELDI 10} in alternation.

d. press [ENTER] @ {ETH2}

e. see displayed: {ETH2}

7. Initialize Circles (start routine):

a. this routine should start automatically after pressing [ENTER]

per 6.e. above.

b. see displayed briefly: {GON ZENITH}

c. plunge telescope down @ {ZERO 1} then plunge up @ {ZERO 2}.

d. rotate instrument CW around vertical spindle @ {A ZERO}.

8. Turn Compensator ON:

a. press [MODE+] until {A- 5} displayed.

b. press [ENTER]

c. see displayed: {A- 5:, A  5:} in alternation.

d. press [ENTER] @ {A- 5:} to switch Compensator ON.

e. see displayed: {COMPENS. ON}

9. Precision Plumbing using instrument tilt angles:

a. press [MODE+] until {A- 6} displayed.

b. press [ENTER]

c. see {CLAMP AND PRESS E}

d. lock h tangent screw; press [ENTER]

e. see {TURN 0 200 CLAMP E}

f. rotate inst. to 0 and lock h tangent screw; press [ENTER]

g. see {A # V #}

h. adjust leveling screws to make A # and V # = 0

i. set V Circle @ 100 GON and rotate 200 and 100 GON azimuths and

assure that V = 100, else problem with compensator.

10. Calibrate V Circle Indexing:

a. place OT bifilar Target @ ~ 16 feet from inst. & @ ~100 GON on V Circ.

b. press [MODE+] until {A- 1} displayed.

c. press [ENTER]

d. see {POS. 1}; sight target w/r H Wire Direct; press [ENTER]

e. see {POS. 2}; sight target w/r H Wire In-Direct; press [ENTER]

f. see {V INDEX  I #}; press [ENTER] to accept or [MODE+] to reject.

11. Calibrate Horizontal Collimation:

a. using target per 10.a. above.

b. press [MODE+] until {A- 2} displayed.

c. press [ENTER]

d. see {POS. 1}; sight target w/r V Wire Direct; press [ENTER]

e. see {POS. 2}; sight target w/r V Wire In-Direct; press [ENTER]

f. see {A COLL.  c #}; press [ENTER] to accept or [MODE+] to reject.

12. 0 Set of H Circle (when collimated in ET):

a. press [MODE+] until {M- 5} displayed.

b. press [ENTER]

c. see {A 000.0000}

d. press [ENTER] to accept.

13. Place instrument in Theodolite mode (H & V angle measurement):

a. press [MODE+] until {M- 2} displayed.

b. see {THEOD. TH: A V}

c. press [ENTER]

14. Communication settings required for ET:

a. 1200 baud, 1 Start bit, 2 Stop bits,

word length = 7 bits + parity bit, Parity = Odd

TO EXIT:

If using MS-DOS Editor--> PRESS [Alt],[ENTER],[X]

For Other Editors--> Follow specific editors menus.